| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O1177-v2 |
| --- | --- |

| Title: | Frame_field_info text combining JVET-O0041, JVET-O0041, and JVET-O0043 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg | Tel: Email: | chadfogg@gmail.com |
| Source: | Movie Labs |  |  |

_____________________________

Abstract

This text combines the changes necessary to combine JVET-O0041 and JVET-O0042 as per the decisions made in HLS BoG (JVET-O1076). Version 2 of this document reflects decisions made in the JVET track Gary session on Thursday 11 July 2019: in particular, that the official Annex C output process does not output repeated pictures, but is relegated to an external process outside the scope of VVC. However, in real decoder systems, the repetition and picture display order information provided from frame_field_info would be necessary to convey pictures from the decoder output process to a common (often system mandatory) display connection, such as HDMI.

# Introduction

Considerations made in this document:

- delta_to_divisor_minus1 cannot be used to express a no-output picture scenario, which in the context of an SEI message, would be redundant with the slice header yes/no output picture flag. (pic_output_flag). During the HLS BoG, it made sense to combine DeltaToDivisor = 0 with pic_output_flag = 0 if DeltaToDivisor were to be signalled in the slice header instead of an SEI message that this document describes.

- frame display of field sequences would be indicated by: source_scan_type=(1) progressive & field_seq_flag=1

- there is no mechanism to repeat field pictures. AVC and HEVC also did not have the ability to express repeated field pictures. Repeated field pictures only make sense in the context of progressive frames, which were intended to be encoded with pic_struct 3,4,5,6 in PAFF (Picture Adaptive Field/Frame) AVC bitstream sequences.

- general_progressive_source_flag and general_interlace_source_flag restrictions on source_scan_type that VVC draft inherented from HEVC should be removed. These violate the intended purpose of source_scan_type (which is meant to have the same puroose as ct_type in AVC).

- pic_struct is mapped to the new elements (derived as sub-elements from pic_struct) for informative purposes in a table, helping readers who are familiar with pic_struct from AVC and HEVC.

- unpaired field pictures (pic_struct 1 & 2) are not supported in this syntax, though could be accompdated with a simple change of the field_paired_with_next_opposite_display_field_flag from 1-bit *_flag to multi-bit *_idc, but at the cost of at least 1 extra bit.

[ note to VVC editors: the length of these new bitstream elements is temporarily long in order to help editors understand the intended purpose. It is expected that the VVC text editors will shorten the variables appropriately.  Yellow highlights point out potential topic for consideration ]

# Draft text of VVC changes

## Frame field info SEI message

[ note to VVC editors: alter frame-field information SEI message syntax according to JVET-O0042 and JVET-O1076  ]

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| delta_to_divisor_minus1 | ue(v) |
| ffi_field_seq_flag | u(1) |
| if( ffi_field_seq_flag == 1 ) { |  |
| bottom_pic_flag | u(1) |
| field_is_paired_with_next_opposite_display_field_flag | u(1) |
| } else { |  |
| display_fields_from_frame_flag | u(1) |
| if( dislay_fields_from_frame_flag == 1 ) |  |
| display_top_field_first_flag | u(1) |
| } |  |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

[ Frame-field information SEI message semantics ]

The frame-field information SEI message may be used to indicate how the associated picture should be displayed, the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

When field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS.

delta_to_divisor_minus1 indicates the number of picture periods, that the current coded picture occupies during the output process. When not present, delta_to_divisor_minus1 is assumed to be equal to 0. When frame_field_info  is present for the current picture, the value DeltaToDivisor is equal to delta_to_divisor_minus1 + 1.

ffi_field_seq_flag shall have the same value and interpretation as field_seq_flag in the seq_parameter_set_rbsp.

[ Note to editor: ffi_field_seq_flag is included in frame_field_info so that elements in this SEI message can be parsed independently of coding headers].

bottom_pic_flag equal to 1 indicates that the current field picture has bottom display field parity. bottom_pic_flaq equal to 0 indicates the current field picture has top display field parity.

field_paired_with_next_opposite_display_field_flag equal to 1 indicates the current coded field picture s paired with the next field picture in display order that has opposite parity of the current coded picture picture indicated by bottom_pic_flag. field_paired_with_next_opposite_display_field_flag equal to 0 indicates that the current coded field picture is paired with the previous field in display order that has opposite parity to of the current coded picture indicated by bottom_pic_flag.

display_fields_from_frame_flag equal to 1 indicates that the current coded frame sequence picture is intended to be displayed as two field pictures.  field_output_from_flag_flag equal to 0 indicates that the coded frame sequence picture should be displayed as a frame picture.

display_top_field_first_flag equal to 1 indicates the top field of the current coded frame sequence picture should be displayed first, and the bottom field should be displayed second. display_top_field_first_flag equal to 0 indicates that the bottom field of the current coded frame sequence picture should be displayed first, and the top field should be displayed second.

When fixed_pic_rate_within_cvs_flag is equal to 1, frame doubling is indicated by the picture structure scenario 7 (combination of DeltaToDivisor equal to 2, field_seq_flag equal to 0, and display_fields_from_frame_flag equal to 0),  the frame should be displayed two times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65, and frame tripling is indicated by the picture structure scenario 8 (combination of DeltaToDivisor equal to 3, field_seq_flag equal to 0, and output_fields_from_frame_flag equal to 0), the frame should be displayed three times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65.

NOTE 2 – Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2 and 4:4:4 chroma formats are shown in Figure D.1, Figure D.2 and Figure D.3, respectively.

The field_is_paired_with_next_opposite_display_field_flag is an association indicator for coded fields that provides hints to associate fields of complementary parity together as frames. The parity of a field can be top or bottom, and the parity of two fields is considered complementary when the parity of one field is top and the parity of the other field is bottom.

NOTE 3 – When frame_field_info is not present for the current picture, then in many cases default values may be inferred or indicated by other means. In the absence of other indications of the intended display type of a picture, the decoder should infer DeltaToDivisor equal to 1 when frame_field_info is not present.

source_scan_type equal to 1 indicates that the source scan type of the associated picture should be interpreted as progressive. source_scan_type equal to 0 indicates that the source scan type of the associated picture should be interpreted as interlaced. source_scan_type equal to 2 indicates that the source scan type of the associated picture is unknown or unspecified. source_scan_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall interpret the value 3 for source_scan_type as equivalent to the value 2.

The following applies to the semantics of source_scan_type:

–	If general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the value of source_scan_type shall be equal to 0 when present, and should be inferred to be equal to 0 when not present.

–	Otherwise, if general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 1 when present and should be inferred to be equal to 1 when not present.

–	Otherwise, when general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 2 when present and should be inferred to be equal to 2 when not present.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate of a previous picture in output order.

NOTE 4 – The duplicate_flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other such duplication and picture rate interpolation methods. This flag would commonly be used when a video feed is encoded as a field sequence in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 5 – When field_seq_flag is equal to 1 and duplicate_flag is equal to 1, this should be interpreted as an indication that the access unit contains a duplicated field of the previous field in output order with the same parity (indicated by bottom_pic_flag) as the current field.

Table D.2 – Interpretation of picture structure scenarios in frame_field_info

| picture structure (informative) |  | Indicated Display of Picture |  | field_seq_flag |  | DeltaToDivisor (delta_to_divisor_minus1 + 1) |  | bottom_pic_flag |  | field_paired_with_next_opposite_display_field_flag |  | output_fields_from_frame_flag |  | output_top_field_first_flag |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (progressive) Frame | 0 | 1 | - | - | 0 | - |
| 1 | Top field | 1 | 1 | 0 | 0 or 1 | - | - |
| 2 | Bottom field | 1 | 1 | 1 | 0 or 1 | - | - |
| 3 | Top field, bottom field, in that order | 0 | 2 | - | - | 1 | 1 |
| 4 | Bottom field, top field, in that order | 0 | 2 | - | - | 1 | 0 |
| 5 | Top field, bottom field, top field repeated, in that order | 0 | 3 | - | - | 1 | 1 |
| 6 | Bottom field, top field, bottom field repeated, in that order | 0 | 3 | - | - | 1 | 0 |
| 7 | Frame doubling | 0 | 2 | - | - | 0 | - |
| 8 | Frame tripling | 0 | 3 | - | - | 0 | - |
| 9 | Top field paired with previous bottom field in output order | 1 | 1 | 0 | 0 | - | - |
| 10 | Bottom field paired with previous top field in output order | 1 | 1 | 1 | 0 | - | - |
| 11 | Top field paired with next bottom field in output order | 1 | 1 | 0 | 1 | - | - |
| 12 | Bottom field paired with next top field in output order | 1 | 1 | 1 | 1 | - | - |
|  | repeat more than 2 pictures |  | > 3 |  |  |  |  |

[ note to VVC editors: like HEVC, this construct does not indicate repeated field sequence pictures. One of the purposes of duplicate_flag was to handle such scenarios by allowing the encoder to code repeated field pictures in field sequences (where the coded repeated field picture consists of all skipped CTU's) and then marking that coded picture with duplicate_flag = 1. We are also deliberately not including additional indicators to display frame pictures from field coded pictures in a field sequence: the functionality of pic_struct 9,10,11,12 in the table above can be used for that purpose instead. ]

When present, it is a requirement of bitstream conformance that the values of pic_struct frame_field_info elements shall be constrained such that exactly one of the following conditions is true:

–	The value of pic_struct is equal to frame_field_info elements conforms to picture structure scenarios 0, 7 or 8 listed in the above table for all pictures in the CVS.

–	The value of pic_struct is equal to frame_field_info elements conforms to picture structure scenarios 1, 2, 9, 10, 11 or 12 listed in the above table for all pictures in the CVS.

–	The value of pic_struct is equal to frame_field_info elements conforms to picture structure scenarios 3, 4, 5 or 6 listed in the above table for all pictures in the CVS.

[ note to VVC editors: Update clause C.3.3 in the VVC workding draft (JVET-N1001) according to the red highlihted text below in order to better distinguish between pictures that are repeated and coded pictures  ]

## Changes to Annex C

C.3.3 Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of access unit n, CpbRemovalTime[ n ].

When picture n has PictureOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows:

DpbOutputTime[ n ] = CpbRemovalTime[ n ] + ClockTick * picDpbOutputDelay –	(C-12)
				picDpbOutputDelta[ OpTid ]

where picDpbOutputDelay is the value of pic_dpb_output_delay and picDpbOutputDelta is the value of picDpbOutputDelta[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ], and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message associated with access unit n, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message associated with access unit n.

The output of the current picture is specified as follows:

–	If PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to CpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PictureOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause C.3.4.

–	Otherwise (PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than CpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes DpbOutputTime[ n ].

When output, the picture is cropped, using the conformance cropping window specified in the active SPS for the picture.

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextCodedPicInOutputOrder ] − DpbOutputTime[ n ]	(C-13)

where nextCodedPicInOutputOrder is the coded picture that follows coded picture n in output order and has PictureOutputFlag equal to 1.

## Changes to Annex E

[ Editor: Update clause E.3.2 in the VVC workding draft (JVET-N1001) according to the red highlihted text below in order to better distinguish between pictures that are repeated and coded pictures  : ]

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when HighestTid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

For each picture n that is output and not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  DeltaToDivisor	(E-65)

where DpbOutputInterval[ n ] is specified in Equation C-13 and DeltaToDivisor is specified in Table E-5 based on the value of frame_field_info for the CVS containing picture n. Entries marked "-" in Table E-5 indicate a lack of dependence of DeltaToDivisor on the corresponding syntax element.

When HighestTid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C-1 (using the value of ClockTick for the CVS containing picture n) towhen one of the following conditions is true for the following picture in output order nextCodedPicInOutputOrder that is specified for use in Equation C-13:

–	picture nextCodedPicInOutputOrder is in the same CVS as picture n.

–	picture nextCodedPicInOutputOrder is in a different CVS and fixed_pic_rate_general_flag[ i ] is equal to 1 in the CVS containing picture nextCodedPicInOutputOrder, the value of ClockTick is the same for both CVSs, and the value of elemental_duration_in_tc_minus1[ i ] is the same for both CVSs.

When HighestTid is equal to i and fixed_pic_rate_within_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C-1 (using the value of ClockTick for the CVS containing picture n) when the following picture in output order nextCodedPicInOutputOrder that is specified for use in Equation C-13 is in the same CVS as picture n.

Table E-5 – Divisor for computation of DpbOutputElementalInterval[ n ]

| frame_field_info is present for the current coded picture | DeltaToDivisor |
| --- | --- |
| No | 1 |
| Yes | delta_to_divisor_minus1 + 1 |

[ Note to VVC editors: it doesn't seem useful to have a table for just two entries. Should the purpose served by the table above be a few sentences instead ?]

# VTM source code change suggests

[ Note to VVC VTM software coordinators: to support the output of fields and frames according to frame_field_info() SEI message, the following changes to the VTM reference decoder are suggested ]

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/blob/master/source/Lib/DecoderLib/DecLib.cpp

Change the determination of isField such that the equivalent frame_field_info of pic_struct 1,2,3,4,5,6,9,10,11,12 output field pictures to file, while pic_struct 0,7,8 output frames to file.

Change from:

[..]

if(!m_SEIs.empty())

{

// Check if any new Picture Timing SEI has arrived

SEIMessages pictureTimingSEIs = getSeisByType(m_SEIs, SEI::PICTURE_TIMING);

if (pictureTimingSEIs.size()>0)

{

SEIPictureTiming* pictureTiming = (SEIPictureTiming*) *(pictureTimingSEIs.begin());

isField    = (pictureTiming->m_picStruct == 1) || (pictureTiming->m_picStruct == 2) || (pictureTiming->m_picStruct == 9) || (pictureTiming->m_picStruct == 10) || (pictureTiming->m_picStruct == 11) || (pictureTiming->m_picStruct == 12);

isTopField = (pictureTiming->m_picStruct == 1) || (pictureTiming->m_picStruct == 9) || (pictureTiming->m_picStruct == 11);

}

}

//Set Field/Frame coding mode

m_pcPic->fieldPic = isField;

m_pcPic->topField = isTopField;

[..]

Change to: [ based on JVET 11 July 2019 track discussion, the VTM reference decoder should just output field pictures when field_seq_flag=1, and frame pictures when frame_seq_flag=0]

- field picture output to file would be field for the equivlanent of pic_struct 1,2,3,4,5,6,9,10,11,12

- determination of top field first would be for pic_struct 3,5.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/blob/master/source/App/DecoderApp/DecApp.cpp

Change from:

Picture* pcPic = *(iterPic);

if( numPicsNotYetDisplayed>2 && pcPic->fieldPic ) //Field Decoding

{

PicList::iterator endPic   = pcListPic->end();

endPic--;

iterPic   = pcListPic->begin();

while (iterPic != endPic)

{

Picture* pcPicTop = *(iterPic);

iterPic++;

Picture* pcPicBottom = *(iterPic);

if ( pcPicTop->neededForOutput && pcPicBottom->neededForOutput &&

(numPicsNotYetDisplayed >  numReorderPicsHighestTid || dpbFullness > maxDecPicBufferingHighestTid) &&

(!(pcPicTop->getPOC()%2) && pcPicBottom->getPOC() == pcPicTop->getPOC()+1) &&

(pcPicTop->getPOC() == m_iPOCLastDisplay+1 || m_iPOCLastDisplay < 0))

{

// write to file

numPicsNotYetDisplayed = numPicsNotYetDisplayed-2;

if ( !m_reconFileName.empty() )

{

const Window &conf = pcPicTop->cs->sps->getConformanceWindow();

#if !JVET_N0063_VUI

const Window  defDisp = (m_respectDefDispWindow && pcPicTop->cs->sps->getVuiParametersPresentFlag()) ? pcPicTop->cs->sps->getVuiParameters()->getDefaultDisplayWindow() : Window();

#endif

const bool isTff = pcPicTop->topField;

bool display = true;

if( m_decodedNoDisplaySEIEnabled )

{

SEIMessages noDisplay = getSeisByType( pcPic->SEIs, SEI::NO_DISPLAY );

const SEINoDisplay *nd = ( noDisplay.size() > 0 ) ? (SEINoDisplay*) *(noDisplay.begin()) : NULL;

if( (nd != NULL) && nd->m_noDisplay )

{

display = false;

}

}

if (display)

{

m_cVideoIOYuvReconFile.write( pcPicTop->getRecoBuf(), pcPicBottom->getRecoBuf(),

m_outputColourSpaceConvert,

false, // TODO: m_packedYUVMode,

#if JVET_N0063_VUI

conf.getWindowLeftOffset(),

conf.getWindowRightOffset(),

conf.getWindowTopOffset(),

conf.getWindowBottomOffset(),

#else

conf.getWindowLeftOffset()   + defDisp.getWindowLeftOffset(),

conf.getWindowRightOffset()  + defDisp.getWindowRightOffset(),

conf.getWindowTopOffset()    + defDisp.getWindowTopOffset(),

conf.getWindowBottomOffset() + defDisp.getWindowBottomOffset(),

#endif

NUM_CHROMA_FORMAT, isTff );

}

}

// update POC of display order

m_iPOCLastDisplay = pcPicBottom->getPOC();

// erase non-referenced picture in the reference picture list after display

if ( ! pcPicTop->referenced && pcPicTop->reconstructed )

{

pcPicTop->reconstructed = false;

}

if ( ! pcPicBottom->referenced && pcPicBottom->reconstructed )

{

pcPicBottom->reconstructed = false;

}

pcPicTop->neededForOutput = false;

pcPicBottom->neededForOutput = false;

}

}

}

Change to: [ based on JVET 11 July 2019 track discussion, the VTM reference decoder should just output field pictures when field_seq_flag=1, and frame pictures when frame_seq_flag=0]

- For equivalent frame_field_info to pic_struct 3,4,5,6: output fields from frame coded picture, including repeats for pic_struct cases 5,6.

- repeat frames for pic_struct 7,8, or any case of num_output_pictures_in_coded_picture_interval

# References

JVET-N1001:  Versatile Video Coding ([working] Draft 5).

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.