| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2021-v1 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Daniel Luo Vadim Seregin Wade Wan | Tel: Email: | jiancong.luo@alibaba-inc.com vseregin@qti.qualcomm.com wade.wan@broadcom.com |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 1 (CE1) on reference picture resampling. The experiments are conducted after the 15th JVET meeting and the results, including cross-checks, will be reported to the 16th JVET meeting.

# Introduction

In the CE, the following tests are defined:

| Test # | Description | Source | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| CE1-1.a | Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com |  |
| CE1-1.b | Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com |  |
| CE1-2 | Use 9-tap downsampling and extended SHVC filters for upsampling | JVET-O0319 | Pankaj Topiwala pankajtva@gmail.com |  |
| CE1-3 | Use SHM filters for downsampling | JVET-O0641 | Mohammed Golam Sarwer m.sarwer@alibaba-inc.com |  |

# Detailed description of tests

### CE1-1 Use integer versions of 8-tap Lanczos filters when reference picture is larger than current picture (JVET-O0240)

The contribution JVET-O0240 proposes two alternative sets of filter coefficients to be used for motion compensation interpolation filtering when the reference picture is of higher resolution than the current picture. The contribution suggests that the scaling factor between the current picture and the reference picture is used to determine which filter to use. The Lanczos2 filter has filtering characteristics adjusted for 2x scaling factor. The Lanczos1 filter has filtering characteristics adjusted for 4x scaling factor. In the Core Experiment the two proposed filters are tested individually for all scaling factors.

### CE1-1.a Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | −5 | 4 | 37 | 56 | 37 | 4 | −5 | 0 |
| 1 | −5 | 3 | 34 | 57 | 39 | 6 | −5 | −1 |
| 2 | −5 | 2 | 32 | 57 | 41 | 7 | −5 | −1 |
| 3 | −4 | 1 | 30 | 56 | 42 | 9 | −5 | −1 |
| 4 | −4 | 0 | 28 | 55 | 45 | 11 | −5 | −2 |
| 5 | −4 | −1 | 25 | 55 | 47 | 13 | −5 | −2 |
| 6 | −3 | −2 | 23 | 53 | 48 | 15 | −4 | −2 |
| 7 | −3 | −3 | 21 | 53 | 50 | 17 | −4 | −3 |
| 8 | −3 | −4 | 19 | 52 | 52 | 19 | −4 | −3 |
| 9 | −3 | −4 | 17 | 50 | 53 | 21 | −3 | −3 |
| 10 | −2 | −4 | 15 | 48 | 53 | 23 | −2 | −3 |
| 11 | −2 | −5 | 13 | 47 | 55 | 25 | −1 | −4 |
| 12 | −2 | −5 | 11 | 45 | 55 | 28 | 0 | −4 |
| 13 | −1 | −5 | 9 | 42 | 56 | 30 | 1 | −4 |
| 14 | −1 | −5 | 7 | 41 | 57 | 32 | 2 | −5 |
| 15 | −1 | −5 | 6 | 39 | 57 | 34 | 3 | −5 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 32 | 64 | 32 | 0 |
| 1 | 30 | 64 | 33 | 1 |
| 2 | 28 | 64 | 35 | 1 |
| 3 | 27 | 63 | 36 | 2 |
| 4 | 25 | 64 | 37 | 2 |
| 5 | 24 | 63 | 39 | 2 |
| 6 | 22 | 63 | 40 | 3 |
| 7 | 21 | 62 | 42 | 3 |
| 8 | 20 | 61 | 43 | 4 |
| 9 | 18 | 60 | 45 | 5 |
| 10 | 17 | 60 | 46 | 5 |
| 11 | 16 | 59 | 47 | 6 |
| 12 | 15 | 58 | 48 | 7 |
| 13 | 13 | 57 | 50 | 8 |
| 14 | 12 | 56 | 52 | 8 |
| 15 | 11 | 55 | 53 | 9 |
| 16 | 10 | 54 | 54 | 10 |
| 17 | 9 | 53 | 55 | 11 |
| 18 | 8 | 52 | 56 | 12 |
| 19 | 8 | 50 | 57 | 13 |
| 20 | 7 | 48 | 58 | 15 |
| 21 | 6 | 47 | 59 | 16 |
| 22 | 5 | 46 | 60 | 17 |
| 23 | 5 | 45 | 60 | 18 |
| 24 | 4 | 43 | 61 | 20 |
| 25 | 3 | 42 | 62 | 21 |
| 26 | 3 | 40 | 63 | 22 |
| 27 | 2 | 39 | 63 | 24 |
| 28 | 2 | 37 | 64 | 25 |
| 29 | 2 | 36 | 63 | 27 |
| 30 | 1 | 35 | 64 | 28 |
| 31 | 1 | 33 | 64 | 30 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-1.b Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | 7 | 17 | 26 | 28 | 26 | 17 | 7 | 0 |
| 1 | 7 | 16 | 25 | 28 | 25 | 17 | 8 | 2 |
| 2 | 6 | 15 | 24 | 29 | 26 | 18 | 8 | 2 |
| 3 | 6 | 15 | 24 | 28 | 26 | 18 | 9 | 2 |
| 4 | 5 | 14 | 23 | 28 | 27 | 19 | 9 | 3 |
| 5 | 5 | 13 | 23 | 28 | 27 | 19 | 10 | 3 |
| 6 | 5 | 13 | 22 | 28 | 27 | 20 | 10 | 3 |
| 7 | 4 | 12 | 22 | 28 | 27 | 20 | 11 | 4 |
| 8 | 4 | 12 | 21 | 27 | 27 | 21 | 12 | 4 |
| 9 | 4 | 11 | 20 | 27 | 28 | 22 | 12 | 4 |
| 10 | 3 | 10 | 20 | 27 | 28 | 22 | 13 | 5 |
| 11 | 3 | 10 | 19 | 27 | 28 | 23 | 13 | 5 |
| 12 | 3 | 9 | 19 | 27 | 28 | 23 | 14 | 5 |
| 13 | 2 | 9 | 18 | 26 | 28 | 24 | 15 | 6 |
| 14 | 2 | 8 | 18 | 26 | 29 | 24 | 15 | 6 |
| 15 | 2 | 8 | 17 | 25 | 28 | 25 | 16 | 7 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 38 | 52 | 38 | 0 |
| 1 | 34 | 49 | 36 | 9 |
| 2 | 33 | 49 | 36 | 10 |
| 3 | 32 | 48 | 37 | 11 |
| 4 | 31 | 48 | 38 | 11 |
| 5 | 30 | 48 | 38 | 12 |
| 6 | 29 | 48 | 38 | 13 |
| 7 | 28 | 47 | 39 | 14 |
| 8 | 27 | 47 | 40 | 14 |
| 9 | 27 | 46 | 40 | 15 |
| 10 | 26 | 46 | 40 | 16 |
| 11 | 25 | 46 | 41 | 16 |
| 12 | 24 | 45 | 42 | 17 |
| 13 | 23 | 45 | 42 | 18 |
| 14 | 22 | 44 | 43 | 19 |
| 15 | 21 | 44 | 43 | 20 |
| 16 | 20 | 44 | 44 | 20 |
| 17 | 20 | 43 | 44 | 21 |
| 18 | 19 | 43 | 44 | 22 |
| 19 | 18 | 42 | 45 | 23 |
| 20 | 17 | 42 | 45 | 24 |
| 21 | 16 | 41 | 46 | 25 |
| 22 | 16 | 40 | 46 | 26 |
| 23 | 15 | 40 | 46 | 27 |
| 24 | 14 | 40 | 47 | 27 |
| 25 | 14 | 39 | 47 | 28 |
| 26 | 13 | 38 | 48 | 29 |
| 27 | 12 | 38 | 48 | 30 |
| 28 | 11 | 38 | 48 | 31 |
| 29 | 11 | 37 | 48 | 32 |
| 30 | 10 | 36 | 49 | 33 |
| 31 | 9 | 36 | 49 | 34 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-2 Use 9-tap downsampling and extended SHVC filters for upsampling (JVET-O0319)

- Downsampling: [1 0 -8 16 46 16 -8 0 1] / 64 (complexity of 7-tap)

- Upsampling: (from HEVC/H.265, Table H.2, extended to 32 phases)

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p [ 0 ] | p [ 1 ] | p [ 2 ] | p [ 3 ] |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | -2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

### CE1-3 Use SHM filters for downsampling (JVET-O0641)

13-tap SHM downsampling filters are used for reference picture downsampling and motion compensation interpolation filters for both luma and chroma components, while existed VTM interpolation filters (8-tap for luma, 4 –tap for chroma) are used for upsampling the reference pictures. The detail filter coefficients are given below.

const int shmDownSampleFilter[8][13] =

{

{0,  0, 5, -6, -10, 37, 76, 37, -10,  -6, 5, 0,  0},

{0, -1, 5, -3, -12, 29, 75, 45,  -7,  -8, 5, 0,  0},

{0, -1, 4, -1, -13, 22, 73, 52,  -3, -10, 4, 1,  0},

{0, -1, 4,  1, -13, 14, 70, 59,   2, -12, 3, 2, -1},

{0, -1, 3,  2, -13,  8, 65, 65,   8, -13, 2, 3, -1},

{0, -1, 2,  3, -12,  2, 59, 70,  14, -13, 1, 4, -1},

{0,  0, 1,  4, -10, -3, 52, 73,  22, -13,-1, 4, -1},

{0,  0, 0,  5,  -8, -7, 45, 75,  29, -12,-3, 5, -1}

};

Const int upSampleFilterLuma[16][8] =

{

{  0, 0,   0, 64,  0,   0,  0,  0 },

{  0, 1,  -3, 63,  4,  -2,  1,  0 },

{ -1, 2,  -5, 62,  8,  -3,  1,  0 },

{ -1, 3,  -8, 60, 13,  -4,  1,  0 },

{ -1, 4, -10, 58, 17,  -5,  1,  0 },

{ -1, 4, -11, 52, 26,  -8,  3, -1 },

{ -1, 3,  -9, 47, 31, -10,  4, -1 },

{ -1, 4, -11, 45, 34, -10,  4, -1 },

{ -1, 4, -11, 40, 40, -11,  4, -1 },

{ -1, 4, -10, 34, 45, -11,  4, -1 },

{ -1, 4, -10, 31, 47,  -9,  3,  1  },

{ -1, 3,  -8, 26, 52, -11,  4,  -1 },

{  0, 1,  -5, 17, 58, -10,  4,  -1 },

{  0, 1,  -4, 13, 60,  -8,   3,  -1  },

{  0, 1,  -3,  8, 62,  -5,   2,  -1  },

{  0, 1,  -2,  4, 63,  -3,   1,   0  }

};

Const int upSampleFilterChroma[32][4] =

{

{  0, 64,  0,  0 },

{ -1, 63,  2,  0 },

{ -2, 62,  4,  0 },

{ -2, 60,  7, -1 },

{ -2, 58, 10, -2 },

{ -3, 57, 12, -2 },

{ -4, 56, 14, -2 },

{ -4, 55, 15, -2 },

{ -4, 54, 16, -2 },

{ -5, 53, 18, -2 },

{ -6, 52, 20, -2 },

{ -6, 49, 24, -3 },

{ -6, 46, 28, -4 },

{ -5, 44, 29, -4 },

{ -4, 42, 30, -4 },

{ -4, 39, 33, -4 },

{ -4, 36, 36, -4 },

{ -4, 33, 39, -4 },

{ -4, 30, 42, -4 },

{ -4, 29, 44, -5 },

{ -4, 28, 46, -6 },

{ -3, 24, 49, -6 },

{ -2, 20, 52, -6 },

{ -2, 18, 53, -5 },

{ -2, 16, 54, -4 },

{ -2, 15, 55, -4 },

{ -2, 14, 56, -4 },

{ -2, 12, 57, -3 },

{ -2, 10, 58, -2 },

{ -1,  7, 60, -2 },

{  0,  4, 62, -2 },

{  0,  2, 63, -1 },

};

# Test conditions and guidelines

Low delay B (LB) test configuration is used to evaluate tests in the CE:

- only 5 second length

- 0.5 second switch points

- 2:1 and 1.5:1 as primary; 4:1 optional, with both dimensions changing together

SHM filters should be used to downsample the source and the downsampled picture is encoded when the resolution is reduced.

For test comparison purposes, the fixed input source downsampling (SHM filters) and fixed upsampling of decoded pictures (VTM interpolation filters) are used in all tests as depicted on Fig. 1.

Figure 1. Fixed source downsampling and fixed decoded picture upsampling

Two PSNRs are requested to be provided:

- PSNR1: it is measured on the decoded picture size

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed VTM interpolation filters are used for upsampling.

Figure 2. Source downsampling and PSNR calculation

The existing interpolation filters for upsampling/downsampling in the reference picture resampling are used as the anchor for test results comparison.

# Timeline

| T1 | 3 weeks after the JVET meeting | To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. |
| --- | --- | --- |
| T2 | Test model software release + 2 weeks | Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector. - Initial study by cross-checkers can begin. - Proponents may continue to modify the software in this branch until T3 - 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes |
| T3 | 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later | Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time. |
| T4 | Regular document deadline – 1 week | CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting). |

# Participants

| Name | Company | Email | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Jonatan Samuelsson Sachin Deshpande Andrew Segall | Sharp | samuelssonj@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com | X |  |
| Pankaj Topiwala | FastVDO | pankajtva@gmail.com | X |  |
| Mohammed Golam Sarwer Jiancong (Daniel) Luo | Alibaba | m.sarwer@alibaba-inc.com jiancong.luo@alibaba-inc.com | X |  |