| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2021-v3 |
| --- | --- |

| Title: | Description of Core Experiment 1 (CE1): Reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Daniel Luo Vadim Seregin Wade Wan | Tel: Email: | jiancong.luo@alibaba-inc.com vseregin@qti.qualcomm.com wade.wan@broadcom.com |
| Source: | CE1 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 1 (CE1) on reference picture resampling. The experiments are conducted after the 15th JVET meeting and the results, including cross-checks, will be reported to the 16th JVET meeting.

# Introduction

In the CE, the following tests are defined:

| Test # | Description | Source | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| CE1-1.1 | Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com |  |
| CE1-1.2 | Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com |  |
| CE1-1.3 | Use integer version of 8-tap Lanczos2.67 filter when reference picture is larger than current picture | JVET-O0240 | Jonatan Samuelsson samuelssonj@sharplabs.com |  |
| CE1-2.1 | Use 9-tap downsampling when the reference picture is larger than the current picture, and extended SHVC filters for upsampling when it is smaller. Resampling and motion compensation are done in one stage as in VTM. | JVET-O0319 | Pankaj Topiwala pankajtva@gmail.com |  |
| CE1-2.2 | Use 9-tap downsampling when the reference picture is larger than the current picture, and extended SHVC filters for upsampling when it is smaller. Resampling is done separately from motion compensation. | JVET-O0319 | Pankaj Topiwala pankajtva@gmail.com |  |
| CE1-3 | Use SHM filters for downsampling | JVET-O0641 | Mohammed Golam Sarwer m.sarwer@alibaba-inc.com | Vadim Seregin vseregin@qti.qualcomm.com |

# Detailed description of tests

### CE1-1 Use integer versions of 8-tap Lanczos filters when reference picture is larger than current picture (JVET-O0240)

The contribution JVET-O0240 proposes two alternative sets of filter coefficients to be used for motion compensation interpolation filtering when the reference picture is of higher resolution than the current picture. The contribution suggests that the scaling factor between the current picture and the reference picture is used to determine which filter to use. The Lanczos2 filter has filtering characteristics adjusted for 2x scaling factor. The Lanczos1 filter has filtering characteristics adjusted for 4x scaling factor. In the Core Experiment the two proposed filters are tested individually for 2x scaling case and 4x scaling case. The Lanczos2 filter is also tested for 1.5x scaling case. The precision of the filter coefficients have been adjusted in the CE compared to the original proposal in order to align the filters with the existing design in the VVC draft. Additionally, a Laczoss2.67 filter (adjusted for 1.5x scaling factor) is tested for the 1.5x scaling case and the 2x scaling case. The filters used for downsampling in CE1-1 are applied for conventional inter prediction only, and not for the affine mode. The affine mode uses the existing 6-tap filters in the VVC draft also for the case when reference picture resampling is applied.

### CE1-1.1 Use integer version of 8-tap Lanczos2 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | −2 | 2 | 18 | 28 | 18 | 2 | −2 | 0 |
| 1 | −2 | 1 | 17 | 28 | 19 | 3 | −2 | 0 |
| 2 | −2 | 1 | 16 | 28 | 20 | 4 | −2 | −1 |
| 3 | −2 | 0 | 15 | 28 | 21 | 5 | −2 | −1 |
| 4 | −2 | 0 | 14 | 28 | 22 | 5 | −2 | −1 |
| 5 | −2 | −1 | 13 | 28 | 23 | 6 | −2 | −1 |
| 6 | −2 | −1 | 12 | 27 | 24 | 7 | −2 | −1 |
| 7 | −2 | −1 | 11 | 26 | 25 | 8 | −2 | −1 |
| 8 | −1 | −2 | 9 | 26 | 26 | 9 | −2 | −1 |
| 9 | −1 | −2 | 8 | 25 | 26 | 11 | −1 | −2 |
| 10 | −1 | −2 | 7 | 24 | 27 | 12 | −1 | −2 |
| 11 | −1 | −2 | 6 | 23 | 28 | 13 | −1 | −2 |
| 12 | −1 | −2 | 5 | 22 | 28 | 14 | 0 | −2 |
| 13 | −1 | −2 | 5 | 21 | 28 | 15 | 0 | −2 |
| 14 | −1 | −2 | 4 | 20 | 28 | 16 | 1 | −2 |
| 15 | 0 | −2 | 3 | 19 | 28 | 17 | 1 | −2 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 16 | 32 | 16 | 0 |
| 1 | 15 | 32 | 16 | 1 |
| 2 | 14 | 32 | 17 | 1 |
| 3 | 13 | 32 | 18 | 1 |
| 4 | 13 | 31 | 19 | 1 |
| 5 | 12 | 32 | 19 | 1 |
| 6 | 11 | 32 | 20 | 1 |
| 7 | 10 | 31 | 21 | 2 |
| 8 | 10 | 30 | 22 | 2 |
| 9 | 9 | 30 | 23 | 2 |
| 10 | 8 | 30 | 23 | 3 |
| 11 | 8 | 29 | 24 | 3 |
| 12 | 7 | 29 | 25 | 3 |
| 13 | 7 | 28 | 25 | 4 |
| 14 | 6 | 28 | 26 | 4 |
| 15 | 6 | 27 | 26 | 5 |
| 16 | 5 | 27 | 27 | 5 |
| 17 | 5 | 26 | 27 | 6 |
| 18 | 4 | 26 | 28 | 6 |
| 19 | 4 | 25 | 28 | 7 |
| 20 | 3 | 25 | 29 | 7 |
| 21 | 3 | 24 | 29 | 8 |
| 22 | 3 | 23 | 30 | 8 |
| 23 | 2 | 23 | 30 | 9 |
| 24 | 2 | 22 | 30 | 10 |
| 25 | 2 | 21 | 31 | 10 |
| 26 | 1 | 20 | 32 | 11 |
| 27 | 1 | 19 | 32 | 12 |
| 28 | 1 | 19 | 31 | 13 |
| 29 | 1 | 18 | 32 | 13 |
| 30 | 1 | 17 | 32 | 14 |
| 31 | 1 | 16 | 32 | 15 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-1.2 Use integer version of 8-tap Lanczos1 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | 4 | 8 | 13 | 14 | 13 | 8 | 4 | 0 |
| 1 | 3 | 8 | 12 | 14 | 13 | 9 | 4 | 1 |
| 2 | 3 | 8 | 12 | 14 | 13 | 9 | 4 | 1 |
| 3 | 3 | 7 | 12 | 15 | 13 | 9 | 4 | 1 |
| 4 | 3 | 7 | 12 | 14 | 13 | 9 | 5 | 1 |
| 5 | 3 | 7 | 11 | 14 | 13 | 10 | 5 | 1 |
| 6 | 2 | 6 | 11 | 14 | 14 | 10 | 5 | 2 |
| 7 | 2 | 6 | 11 | 14 | 14 | 10 | 5 | 2 |
| 8 | 2 | 6 | 10 | 14 | 14 | 10 | 6 | 2 |
| 9 | 2 | 5 | 10 | 14 | 14 | 11 | 6 | 2 |
| 10 | 2 | 5 | 10 | 14 | 14 | 11 | 6 | 2 |
| 11 | 1 | 5 | 10 | 13 | 14 | 11 | 7 | 3 |
| 12 | 1 | 5 | 9 | 13 | 14 | 12 | 7 | 3 |
| 13 | 1 | 4 | 9 | 13 | 15 | 12 | 7 | 3 |
| 14 | 1 | 4 | 9 | 13 | 14 | 12 | 8 | 3 |
| 15 | 1 | 4 | 9 | 13 | 14 | 12 | 8 | 3 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 19 | 26 | 19 | 0 |
| 1 | 17 | 24 | 18 | 5 |
| 2 | 17 | 24 | 18 | 5 |
| 3 | 16 | 24 | 19 | 5 |
| 4 | 15 | 24 | 19 | 6 |
| 5 | 15 | 24 | 19 | 6 |
| 6 | 15 | 24 | 19 | 6 |
| 7 | 14 | 24 | 19 | 7 |
| 8 | 14 | 23 | 20 | 7 |
| 9 | 13 | 23 | 20 | 8 |
| 10 | 13 | 23 | 20 | 8 |
| 11 | 12 | 23 | 21 | 8 |
| 12 | 12 | 22 | 21 | 9 |
| 13 | 12 | 22 | 21 | 9 |
| 14 | 11 | 22 | 21 | 10 |
| 15 | 11 | 22 | 21 | 10 |
| 16 | 10 | 22 | 22 | 10 |
| 17 | 10 | 21 | 22 | 11 |
| 18 | 10 | 21 | 22 | 11 |
| 19 | 9 | 21 | 22 | 12 |
| 20 | 9 | 21 | 22 | 12 |
| 21 | 8 | 21 | 23 | 12 |
| 22 | 8 | 20 | 23 | 13 |
| 23 | 8 | 20 | 23 | 13 |
| 24 | 7 | 20 | 23 | 14 |
| 25 | 7 | 19 | 24 | 14 |
| 26 | 6 | 19 | 24 | 15 |
| 27 | 6 | 19 | 24 | 15 |
| 28 | 6 | 19 | 24 | 15 |
| 29 | 5 | 19 | 24 | 16 |
| 30 | 5 | 18 | 24 | 17 |
| 31 | 5 | 18 | 24 | 17 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-1.3 Use integer version of 8-tap Lanczos2.67 filter when reference picture is larger than current picture

For luma, the following filter is used when the reference picture is larger than current picture:

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 0 | -1 | -5 | 17 | 42 | 17 | -5 | -1 | 0 |
| 1 | 0 | -5 | 15 | 41 | 19 | -5 | -1 | 0 |
| 2 | 0 | -5 | 13 | 40 | 21 | -4 | -1 | 0 |
| 3 | 0 | -5 | 11 | 39 | 24 | -4 | -2 | 1 |
| 4 | 0 | -5 | 9 | 38 | 26 | -3 | -2 | 1 |
| 5 | 0 | -5 | 7 | 38 | 28 | -2 | -3 | 1 |
| 6 | 1 | -5 | 5 | 36 | 30 | -1 | -3 | 1 |
| 7 | 1 | -4 | 3 | 35 | 32 | 0 | -4 | 1 |
| 8 | 1 | -4 | 2 | 33 | 33 | 2 | -4 | 1 |
| 9 | 1 | -4 | 0 | 32 | 35 | 3 | -4 | 1 |
| 10 | 1 | -3 | -1 | 30 | 36 | 5 | -5 | 1 |
| 11 | 1 | -3 | -2 | 28 | 38 | 7 | -5 | 0 |
| 12 | 1 | -2 | -3 | 26 | 38 | 9 | -5 | 0 |
| 13 | 1 | -2 | -4 | 24 | 39 | 11 | -5 | 0 |
| 14 | 0 | -1 | -4 | 21 | 40 | 13 | -5 | 0 |
| 15 | 0 | -1 | -5 | 19 | 41 | 15 | -5 | 0 |

The regular VVC 8-tap filter is used when the reference picture is smaller than current picture.

For chroma, the following filter is used when the reference picture is larger than current picture.

| Fractional sample position p | interpolation filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | fC[ p ][ 0 ] | fC[ p ][ 1 ] | fC[ p ][ 2 ] | fC[ p ][ 3 ] |
| 0 | 12 | 40 | 12 | 0 |
| 1 | 11 | 40 | 13 | 0 |
| 2 | 10 | 40 | 15 | -1 |
| 3 | 9 | 40 | 16 | -1 |
| 4 | 8 | 40 | 17 | -1 |
| 5 | 8 | 39 | 18 | -1 |
| 6 | 7 | 39 | 19 | -1 |
| 7 | 6 | 38 | 21 | -1 |
| 8 | 5 | 38 | 22 | -1 |
| 9 | 4 | 38 | 23 | -1 |
| 10 | 4 | 37 | 24 | -1 |
| 11 | 3 | 36 | 25 | 0 |
| 12 | 3 | 35 | 26 | 0 |
| 13 | 2 | 34 | 28 | 0 |
| 14 | 2 | 33 | 29 | 0 |
| 15 | 1 | 33 | 30 | 0 |
| 16 | 1 | 31 | 31 | 1 |
| 17 | 0 | 30 | 33 | 1 |
| 18 | 0 | 29 | 33 | 2 |
| 19 | 0 | 28 | 34 | 2 |
| 20 | 0 | 26 | 35 | 3 |
| 21 | 0 | 25 | 36 | 3 |
| 22 | -1 | 24 | 37 | 4 |
| 23 | -1 | 23 | 38 | 4 |
| 24 | -1 | 22 | 38 | 5 |
| 25 | -1 | 21 | 38 | 6 |
| 26 | -1 | 19 | 39 | 7 |
| 27 | -1 | 18 | 39 | 8 |
| 28 | -1 | 17 | 40 | 8 |
| 29 | -1 | 16 | 40 | 9 |
| 30 | -1 | 15 | 40 | 10 |
| 31 | 0 | 13 | 40 | 11 |

The regular VVC 4-tap filer is used when the reference picture is smaller than current picture.

### CE1-2 Use 9-tap downsampling and extended SHVC filters for upsampling (JVET-O0319)

Our vision for ARC (or reference picture resampling) functionality is to define resampling of both reference pictures and source pictures using a single set of co-designed resampling filters, to allow for high quality prediction. This is initially intended to be outside the MEC process. The ARC functionality can be embedded within the interpolation process. For the purpose of testing both approaches, we offer phased versions of both our downsampler and an existing upsampler (the upsampler is from table 8-9 and 8-13 in the Draft VVC specification, JVET-2001).

- Downsampling: [1 0 -8 16 46 16 -8 0 1] / 64 (complexity of 7-tap) is .used for downsampling and motion compensation interpolation for both luma and chroma components when the reference picture is larger than the current picture. Phased version below..

- Upsampling: (from table 8-9 from JVET-O2001). This is used when the reference picture is smaller than the current picture. The detailed filter coefficients are given in tables below.

We present our filters in the traditional [-pi, pi] phase format, but we have [0, 2pi] versions if needed.

| Fractional sample position p | downsampling filter coefficients |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p[ 0 ] | p[ 1 ] | p[ 2 ] | p[ 3 ] | p[ 4 ] | p[ 5 ] | p[ 6 ] | p[ 7 ] | p[ 8 ] |
| 0 | 4 | -6 | 0 | 32 | 32 | 0 | -6 | 4 | 4 |
| 1 | 4 | -6 | 1 | 34 | 30 | -2 | -5 | 4 | 4 |
| 2 | 3 | -7 | 3 | 36 | 28 | -3 | -4 | 4 | 4 |
| 3 | 3 | -7 | 5 | 37 | 26 | -4 | -4 | 4 | 4 |
| 4 | 3 | -7 | 6 | 39 | 24 | -5 | -3 | 4 | 3 |
| 5 | 2 | -8 | 9 | 41 | 22 | -6 | -2 | 3 | 3 |
| 6 | 2 | -8 | 11 | 43 | 20 | -7 | -2 | 3 | 2 |
| 7 | 1 | -8 | 13 | 44 | 18 | -7 | -1 | 2 | 2 |
| 8 | 1 | 0 | -8 | 16 | 46 | 16 | -8 | 0 | 1 |
| 9 | 2 | -1 | -7 | 18 | 45 | 13 | -8 | 1 | 1 |
| 10 | 3 | -2 | -7 | 20 | 44 | 11 | -8 | 1 | 2 |
| 11 | 3 | -2 | -6 | 22 | 42 | 8 | -8 | 2 | 3 |
| 12 | 4 | -3 | -5 | 24 | 39 | 6 | -7 | 3 | 3 |
| 13 | 4 | -4 | -4 | 26 | 37 | 5 | -7 | 3 | 4 |
| 14 | 4 | -4 | -3 | 28 | 36 | 3 | -7 | 3 | 4 |
| 15 | 4 | -5 | -1 | 30 | 33 | 1 | -6 | 4 | 4 |

CE1-2.1.  Test within the interpolation process, similar to CE1-1, but using our filters in place of the existing interpolation filters. (This process does not rescale the reference pictures, nor stores them in the DPB, but rather works with block-based methods to scale and perform MEC as needed.)  This functionality is provided for comparison with the method of CE1-1.

CE1-2.2.  Test outside of the MEC process, where reference pictures are resampled to match the resolution of input source frames, and then MEC is performed using existing MEC process. We will illustrate the application of these filters, for either reference or source picture downsampling, for just the luma samples in two important subcases, 2:1 and 1.5:1.

For 2:1 downsampling, one simply applies the 0-phase downsampling filter (p=8 filter above), and then selects every other sample.

For 3:2 (1.5:1) downsampling, it is more complicated. One needs to replace every 3 points with 2 points. That is, for points at positions [0, 1, 2, 3, 4, 5] etc, one replaces them with points at [0.25, 1.75, 3.25, 4.75] etc.  Since the 0-phase here is the p=8 filter, in a 16-phase family, for the 0.25 position, one uses the p=12 filtter, and for the 1.75 position, one uses the position p=4 filter.

We note again that our upsampler, in the table below, is Table 8-9 from the VVC Draft Specification, JVET-2001.

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | f[ p ][ 0 ] | f[ p ][ 1 ] | f[ p ][ 2 ] | f[ p ][ 3 ] |
| 0 | 0 | 64 | 0 | 0 |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | −2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

| Fractional sample position p | Upsampling filter coefficients |  |  |  |
| --- | --- | --- | --- | --- |
|  | p [ 0 ] | p [ 1 ] | p [ 2 ] | p [ 3 ] |
| 1 | −1 | 63 | 2 | 0 |
| 2 | −2 | 62 | 4 | 0 |
| 3 | -2 | 60 | 7 | −1 |
| 4 | −2 | 58 | 10 | −2 |
| 5 | −3 | 57 | 12 | −2 |
| 6 | −4 | 56 | 14 | −2 |
| 7 | −4 | 55 | 15 | −2 |
| 8 | −4 | 54 | 16 | −2 |
| 9 | −5 | 53 | 18 | −2 |
| 10 | −6 | 52 | 20 | −2 |
| 11 | −6 | 49 | 24 | −3 |
| 12 | −6 | 46 | 28 | −4 |
| 13 | −5 | 44 | 29 | −4 |
| 14 | −4 | 42 | 30 | −4 |
| 15 | −4 | 39 | 33 | −4 |
| 16 | −4 | 36 | 36 | −4 |
| 17 | −4 | 33 | 39 | −4 |
| 18 | −4 | 30 | 42 | −4 |
| 19 | −4 | 29 | 44 | −5 |
| 20 | −4 | 28 | 46 | −6 |
| 21 | −3 | 24 | 49 | −6 |
| 22 | −2 | 20 | 52 | −6 |
| 23 | −2 | 18 | 53 | −5 |
| 24 | −2 | 16 | 54 | −4 |
| 25 | −2 | 15 | 55 | −4 |
| 26 | −2 | 14 | 56 | −4 |
| 27 | −2 | 12 | 57 | −3 |
| 28 | −2 | 10 | 58 | −2 |
| 29 | −1 | 7 | 60 | −2 |
| 30 | 0 | 4 | 62 | −2 |
| 31 | 0 | 2 | 63 | −1 |

### CE1-3 Use SHM filters for downsampling (JVET-O0641)

In the motion compensation for reference picture downsampling, the 12-tap SHM downsampling filters are used. The selection of filters is based on the downsampling ratio as in SHM-12.4. The same filter is used for both luma and chroma components. In the motion compensation for reference picture upsampling, the existed VTM interpolation filters (i.e., the 8-tap filter for luma, the 6-tap for luma 4x4 sub-block, the 6-tap filter for halp-pel AMVR, and the 4–tap for chroma) are used. The detail downsampling filter coefficients are given below.

| Fractional sample position p | interpolation filter coefficients for 2:1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0 |
| 1 | 2 | -3 | -9 | 4 | 38 | 58 | 43 | 7 | -9 | -4 | 1 | 0 |
| 2 | 2 | -2 | -9 | 2 | 35 | 58 | 44 | 9 | -8 | -4 | 1 | 0 |
| 3 | 1 | -2 | -9 | 1 | 34 | 58 | 46 | 11 | -8 | -5 | 1 | 0 |
| 4 | 1 | -1 | -8 | -1 | 31 | 57 | 47 | 13 | -7 | -5 | 1 | 0 |
| 5 | 1 | -1 | -8 | -2 | 29 | 56 | 49 | 15 | -7 | -6 | 1 | 1 |
| 6 | 1 | 0 | -8 | -3 | 26 | 55 | 51 | 17 | -7 | -6 | 1 | 1 |
| 7 | 1 | 0 | -7 | -4 | 24 | 54 | 52 | 19 | -6 | -7 | 1 | 1 |
| 8 | 1 | 0 | -7 | -5 | 22 | 53 | 53 | 22 | -5 | -7 | 0 | 1 |
| 9 | 1 | 1 | -7 | -6 | 19 | 52 | 54 | 24 | -4 | -7 | 0 | 1 |
| 10 | 1 | 1 | -6 | -7 | 17 | 51 | 55 | 26 | -3 | -8 | 0 | 1 |
| 11 | 1 | 1 | -6 | -7 | 15 | 49 | 56 | 29 | -2 | -8 | -1 | 1 |
| 12 | 0 | 1 | -5 | -7 | 13 | 47 | 57 | 31 | -1 | -8 | -1 | 1 |
| 13 | 0 | 1 | -5 | -8 | 11 | 46 | 58 | 34 | 1 | -9 | -2 | 1 |
| 14 | 0 | 1 | -4 | -8 | 9 | 44 | 58 | 35 | 2 | -9 | -2 | 2 |
| 15 | 0 | 1 | -4 | -9 | 7 | 43 | 58 | 38 | 4 | -9 | -3 | 2 |

| Fractional sample position p | interpolation filter coefficients for 1.5 : 1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0 |
| 1 | 0 | 5 | -4 | -11 | 33 | 76 | 40 | -9 | -7 | 5 | 0 | 0 |
| 2 | -1 | 5 | -3 | -12 | 29 | 75 | 45 | -7 | -8 | 5 | 0 | 0 |
| 3 | -1 | 4 | -2 | -13 | 25 | 75 | 48 | -5 | -9 | 5 | 1 | 0 |
| 4 | -1 | 4 | -1 | -13 | 22 | 73 | 52 | -3 | -10 | 4 | 1 | 0 |
| 5 | -1 | 4 | 0 | -13 | 18 | 72 | 55 | -1 | -11 | 4 | 2 | -1 |
| 6 | -1 | 4 | 1 | -13 | 14 | 70 | 59 | 2 | -12 | 3 | 2 | -1 |
| 7 | -1 | 3 | 1 | -13 | 11 | 68 | 62 | 5 | -12 | 3 | 2 | -1 |
| 8 | -1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1 |
| 9 | -1 | 2 | 3 | -12 | 5 | 62 | 68 | 11 | -13 | 1 | 3 | -1 |
| 10 | -1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1 |
| 11 | -1 | 2 | 4 | -11 | -1 | 55 | 72 | 18 | -13 | 0 | 4 | -1 |
| 12 | 0 | 1 | 4 | -10 | -3 | 52 | 73 | 22 | -13 | -1 | 4 | -1 |
| 13 | 0 | 1 | 5 | -9 | -5 | 48 | 75 | 25 | -13 | -2 | 4 | -1 |
| 14 | 0 | 0 | 5 | -8 | -7 | 45 | 75 | 29 | -12 | -3 | 5 | -1 |
| 15 | 0 | 0 | 5 | -7 | -9 | 40 | 76 | 33 | -11 | -4 | 5 | 0 |

# Test conditions and guidelines

VTM-6.1 software with the affine patch shall be used as the anchor and the base software for CE development.

Low delay B (LB) test configuration is used to evaluate tests in the CE:

- only 5 second length (half of the CTC frame count)

- 0.5 second switch points

- 2:1 and 1.5:1 as primary; 4:1 optional, with both dimensions changing together

Configuration files for 1.5x, 2.0x, and 4.0x scaling ratios can be found in the cfg/rpr directory of the VTM software package.

SHM filters are used for both luma and chroma components to downsample the source and the downsampled picture is encoded when the resolution is reduced. In the source downsampling, zero position alignment (type 2, see Fig. 1) between luma and choma components is assumed.

Figure 1. Chroma sampling types.

For test comparison purposes, the fixed input source downsampling (SHM filters) and fixed upsampling of decoded pictures (VTM interpolation filters) are used in all tests as depicted on Fig. 2.

Figure 2. Fixed source downsampling and fixed decoded picture upsampling

Two PSNRs are requested to be provided:

- PSNR1: it is measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE.

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed VTM interpolation filters are used for upsampling.

Figure 3. Source downsampling and PSNR calculation

The existing interpolation filters (luma 8-tap and chroma 4-tap interpolation filters) for upsampling/downsampling in the reference picture resampling are used as the anchor for test results comparison.

# Timeline

Due to late availability of the RPR support in the VTM software, T2 and T3 milestones are merged into one, and T4 is delayed by 2 days.

| T1 | 3 weeks after the JVET meeting | To revise the CE description and refine questions to be answered. Questions should be discussed and agreed on JVET reflector. Any changes of planned tests after this time need to be announced and discussed on the JVET reflector. |
| --- | --- | --- |
| T2 | Test model software release + 2 weeks September 13th, 2019 | Integration of all tools into a separate CE branch of the VTM is completed and announced to JVET reflector. - Initial study by cross-checkers can begin. - Proponents may continue to modify the software in this branch until T3 - 3rd parties are encouraged to study and make contributions to the next meeting with proposed changes |
| T3 | 3 weeks before the next JVET meeting or T2 + 1 week, whichever is later September 13th, 2019 | Any changes to the CE test branches of the software must be frozen, so the cross-checkers can know exactly what they are cross-checking. A software version tag should be created at this time. The name of the cross-checkers and list of specific tests for each tool under study in the CE plan description shall be documented in an updated CE description by this time. |
| T4 | Regular document deadline – 1 week September 19th, 2019 | CE contribution documents including specification text and complete test results shall be uploaded to the JVET document repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting). |

# Participants

| Name | Company | Email | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Jonatan Samuelsson Sachin Deshpande Andrew Segall | Sharp | samuelssonj@sharplabs.com sdeshpande@sharplabs.com asegall@sharplabs.com | X |  |
| Pankaj Topiwala | FastVDO | pankajtva@gmail.com | X |  |
| Mohammed Golam Sarwer Jiancong (Daniel) Luo | Alibaba | m.sarwer@alibaba-inc.com jiancong.luo@alibaba-inc.com | X |  |