| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2024-v1 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang Xiaoyu Xiu | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE4 coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 4 (CE4) on inter prediction is to investigate the following 2 topics:

- Geometric partition related merge modes;

- Simplification on PROF and BDOF.

# CE4-1: Geometric partition related merge modes

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4-1.1 | Geometric (GEO) merge mode as proposed in JVET-O0489 with 32 angles and 5 distance values | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) |  |
| 4-1.2 | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=32 or 24 | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) |  |
| 4-1.3 | Test 4-1.2 + GEO with optimized encoder | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) |  |
| 4-1.4 | GEO + alternative mode flag signaling (that is, TPM is signaled as GEO partitions) | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) |  |
| 4-1.5 | GEO + ¼ and ½ adaptive-size partial transform | JVET-O0489 | A. Filippov (Huawei) |  |
| 4-1.6 | GEO + ½ fixed-size partial transform | JVET-O0489 | A. Filippov (Huawei) |  |
| 4-1.7 | Adaptive blending filtering for TPM | JVET-O0513 | H. Gao (Huawei), M. Bläser (RWTH Aachen) |  |
| 4-1.8 | Test 4-1.7 + GEO | JVET-O0513 | H. Gao (Huawei), M. Bläser (RWTH Aachen) |  |
| 4-1.9 | CIIP with triangular partitions | JVET-O0522 | S. Blasi (BBC) |  |
| 4-1.10 | Test 4-1.9 + GEO | JVET-O0522 JVET-O0489 | S. Blasi (BBC) H. Gao (Huawei) |  |
| 4-1.11 | Test 4-1.9 + Test 4.1.7 | JVET-O0522 JVET-O0513 | S. Blasi (BBC) |  |
| 4-1.12 | Test 4-1.9 + Partial transform as in Test 4.1.5 | JVET-O0489 JVET-O0522 | A. Filippov (Huawei), S. Blasi (BBC) |  |
| 4-1.13 | Test 4-1.12 + Test 4.1.5 (GEO) | JVET-O0489 JVET-O0522 | A. Filippov (Huawei), S. Blasi (BBC) |  |

## Subjective evaluation

Proponents and crosscheckers are requested to check the subjective quality of a test in the video mode and feedback whether there is observable quality improvement over the anchor. The bitstream of the coded samples should be provided to CE4 participants. Subjective assessment by experts is mandatory during the meeting. Testing sequences inside and outside CTC to be used for subjective viewing is required for QP xx and xx.

## Detailed description of tests

### Test 4-1.1 ~ 4-1.6: Geometrical partitioning for inter blocks in JVET-O0489 (Huawei, RWTH Aachen)

The figure below illustrates TPM in VVC-5 and additional shapes proposed for non-rectangular inter blocks.

VVC-5                   Proposal

…

Proposal and TPM design in VVC-5.

Overall number of GEO partitioning is 140. This number found to be optimal from view point flexibility vs signalling. Additional signalling for GEO requires signaling for the angle , and separation line displacement relatively to the center of the block .  represents a quantized angle between 0 and 360 degrees, with 11.25 degrees of separation and  represents a distance with 5 different values. The values  and  pairs are stored in a table of size 140x(3+5)/8=140 bytes.

GEO partitioning signalling

Similar to TPM, the proposed GEO partitioning for inter is allowed for uni-predicted blocks not smaller than 88 in order to have the same memory bandwidth with bi-predicted blocks at decoder side. Motion vector prediction for GEO partitioning is aligned with TPM. As well as in TPM blending between 2 predictions is applied on inner boundary.

- Test 4-1.1: Geometric (GEO) merge mode as proposed in JVET-O0489. Use 32 quantized angle and 5 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.2a: Test 4-1.1 + Use 32 quantized angles and 4 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.2b: Test 4-1.1 + Use 24 quantized angles and 4 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.3: GEO with further encoder-only optimization algorithm on top of Test 4-1.2.

- Test 4-1.4: GEO + alternative mode flag signaling (the TPM is signaled as GEO partitions).

- Test 4-1.5: ¼ and ½ adaptive-size partial transform method is tested.

- Test 4-1.6: ½ fixed-size partial transform method is tested.

### Test 4-1.7 ~ 4-1.8: Adaptive blending filtering for TPM in JVET-O0513 (Huawei, RWTH Aachen)

Proposed solution further extends flexibility of Triangle Partition Mode by introducing a control flag to disable the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination can either be 0 or 8, the weighting matrix example is shown as in following figure.

In Test 4-1.8, the blending off method is further tested on top of geometric merge mode.

### Test CE4-1.9: CIIP with triangular partitions in JVET-O0522 (BBC)

In JVET-O0522, CIIP with triangular partitions are proposed. An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the partitioning scheme as in TPM mode. The intra predictor is used in one of the two triangular partitions, and the inter predictor is used in the other triangular partition. The blending scheme as used in TPM is used to blend the two triangular partitions.

### Test CE4-1.10: CIIP with triangular and GEO partitions (BBC, Huawei, RWTH Aachen)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with the GEO partitioning scheme proposed in JVET-O0489.

### Test CE4-1.11: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.7 Adaptive blending filtering in JVET-O0513 (BBC)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with adaptive blending filtering in JVET-O0513 to control usage of the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination in CIIP with triangular partitions is either 0 or 8.

### Test CE4-1.12: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.5 Partial transform in JVET-O0489 (Huawei, BBC)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with partial transform in JVET-O0489 (with no geometric partitioning itself) to keep residues localized along boundary between partitions.

### Test CE4-1.13: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.5 Geometrical partitioning with partial transform in JVET-O0489 (Huawei, BBC)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with partial transform in JVET-O0489 (with geometric partitioning enabled) to keep residues localized along boundary between partitions.

# CE4-2: Simplification on PROF and BDOF

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4-2.1 | 1/32-pel precision of PROF motion refinement + [-32, 31] for PROF clipping range | JVET-O1142 | X. Xiu (Kwai) H. Huang (Qualcomm) C.-Y. Lai (MediaTek) Y. He (InterDigital) |  |
| 4-2.2 | [-32, 31] for PROF clipping range | JVET-O1142 | X. Xiu (Kwai) H. Huang (Qualcomm) C.-Y. Lai (MediaTek) Y. He (InterDigital) |  |
| 4-2.3 | Restrict the MV refinement in BDOF to be 0 or in a form of +2^N or –2^N (N is an integer) | JVET-O0573 | H. Liu (Bytedance) |  |
| 4-2.4 | Restrict the MV refinement in PROF to be 0 or in a form of +2^N or –2^N (N is an integer) | JVET-O0573 | H. Liu (Bytedance) |  |
| 4-2.5 | Test 4-2.3 + Test 4-2.4 | JVET-O0573 | H. Liu (Bytedance) |  |

## Detailed description of tests

### Test 4-2.1~4-2.2: Harmonization on BDOF and PROF in JVET-O1142 (Kwai, Qualcomm, Mediatek, InterDigital)

In this CE test, two methods will be studied to align the representation bit-depths of the PROF and the BDOF.

In the first method, the precision of the PROF motion refinements is changed from 1/64-pel to 1/32-pel and clipped into the range of [-32, 31].

In the second method, the range of PROF motion refinements is also clipped to [-32, 31] instead of [-64, 63]. In both methods, the clipping that is applied to the derived sample refinements is removed.

### Test 4-2.3 ~ 4-2.5: Restriction on motion vector refinement in BDOF in JVET-O0573 (Bytedance)

This contribution proposes to restrict the motion vector refinement in BDOF and PROF to be 0 or in a form of +2^N or –2^N (N is an integer).

1. Test #1: apply proposed method to BDOF.

2. Test #2: apply proposed method to PROF.

3. Test #3: apply proposed method to both BDOF and PROF.

# Test condition and evaluation criteria

Each testing item is required to be compared against VTM-6.0, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-N1010. Simulation with RA and LDB configurations are mandatory.

# Crosscheck report

Crosscheckers shall fill in the table below, and send it to CE coordinators 3 days before T4.

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |

Explanation of each item in the table

- SW match CE description: Y SW implementation match CE description, N mismatch.

- Text match SW: Y text match SW, N mismatch.

- Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

- EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

# SW Integration guideline

All tests are required to be implemented on top of VTM-6.0.

# Timeline

| T1 | 02 Aug., 2019 | Finalize CE description (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | 26 Aug., 2019 | Deliver software packages and begin cross-checking (2 weeks after test model software release) |
| T3 | 10 Sep., 2019 | Freeze software revision and provide full results for proponents Begin final crosschecking for cross-checkers |
| T4 | 17 Sep., 2019 | Upload contribution documents with specification text and complete test results (regular document deadline – 1 week) |

# Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Aachen Univ. | Johannes Sauer Max Bläser | sauer@ient.rwth-aachen.de blaeser@ient.rwth-aachen.de |
| Alibaba | Yan Ye Yu-Chen Sun Jie Chen Mohammed Golam Sarwer | yan.ye@alibaba-inc.com yuchen.s@alibaba-inc.com jiechen.cj@alibaba-inc.com m.sarwer@alibaba-inc.com |
| Apple | Jaehong Chon | jaehong.chon@gmail.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| DJI | Xiaozhen Zheng | xiaozhen.zheng@dji.com |
| Dolby | Peng Yin | pyin@dolby.com |
| Ericsson | Ruoyang Yu Julien Le Tanou | ruoyang.yu@ericsson.com julien.le.tanou@ericsson.com |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Foxconn | Yu-Ciao Yang Po-Han Lin Tsung-Hua Li | yuciaoyang@fginnov.com PohanLin@fginnov.com thli@fginnov.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Futurewei | Jianle Chen | jianle.chen@futurewei.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |
| Huawei | Huanbang Chen Haitao Yang Sychev Maxim Solovyev Timofey Alexey Filippov Vasily Rufitskiy Semih Esenlik Alexander Karabutov Han Gao Semih Esenlik Xiang Ma | chenhuanbang@huawei.com haitao.yang@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com semih.esenlik@huawei.com karabutov.alexander@huawei.com han.gao@huawei.com semih.esenlik@huawei.com maxiang6@huawei.com |
| Intel | Grace Yu | grace.yu@intel.com |
| InterDigital | Yuwen He Hua Yang Wei Chen Saurav Bandyopadhyay | Yuwen.He@interdigital.com hua.yang@interdigtial.com wei.chen@interdigital.com Saurav.Bandyopadhyay@InterDigital.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Ittiam | Sriram Sethuraman | sriram.sethuraman@ittiam.com |
| KDDI | Kyohei Unno Kei Kawamura Yoshitaka Kidani | ky-unno@kddi-research.jp ki-kawamura@kddi.com yo-kidani@kddi.com |
| Kwai | Yi-Wen Chen Xianglin Wang Xiaoyu Xiu Tsung-Chuan Ma | yiwenchen@kwai.com xianglinwang@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |
| LGE | Seung Hwan Kim Jaehyun Lim Naeri Park Hyeongmun Jang Jie Zhao Seethal Paluri | seunghwan3.kim@lge.com Jaehyun.lim@lge.com naeri.park@lge.com hm.jang@lge.com jie.zhao@lge.com seethal.paluri@lge.com |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Shih-Ta Hsiang Yuling Hsiao Chen-Yen Lai Yu-Chi Su Zhi-Yi Lin Chun-Chia Chen Yucheng Lin | yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com shih-ta.hsiang@mediatek.com Yuling.Hsiao@mediatek.com Jenny.Lai@mediatek.com Yu-Chi.Su@mediatek.com Zhi-Yi.Lin@mediatek.com chunchia.chen@mediatek.com Yucheng.Lin@mediatek.com |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Nokia | Jani Lainema Alireza Aminlou Ramin Ghaznavi-Youvalari | jani.lainema@nokia.com alireza.aminlou@nokia.com ramin.ghaznavi-youvalari@nokia.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Panasonic | Kiyofumi Abe Tadamasa Toma Che-Wei Kuo Chong Soon Lim | abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| PKU | Zhao Wang Suhong Wang Xuewei Meng | zhaowang@pku.edu.cn suhong.wang@pku.edu.cn xwmeng@pku.edu.cn |
| Qualcomm | Wei-Jung Chien Vadim Seregin Chun-Chi Chen Yan Zhang Yu Han Han Huang Chao-Hsiung Hung Luong Pham Van Dmytro Rusanovskyy | wchien@qti.qualcomm.com vseregin@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com hanhuang@qti.qualcomm.com chaohsiu@qti.qualcomm.com lphamvan@qti.qualcomm.com dmytror@qti.qualcomm.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park Yinji Piao | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com yj1003.piao@samsung.com |
| Sharp | Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai Kiran Misra Eiichi Sasaki Tianyang Zhou Andrew Segall | tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp misrak@sharplabs.com eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp asegall@sharplabs.com |
| Sony | Kenji Kondo Masaru Ikeda | kenji.kondo@sony.com Masaru.Ikeda@sony.com |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tenceNnt.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |