| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2024-v3 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Haitao Yang Xiaoyu Xiu | Email: | chunchic@qti.qualcomm.com haitao.yang@huawei.com xiaoyuxiu@kwai.com |
| Source: | CE4 coordinators |  |  |

_____________________________

# Abstract

The goal of Core Experiment 4 (CE4) on inter prediction is to investigate the following 2 topics:

- Geometric partition related merge modes;

- Simplification on PROF and BDOF.

# CE4-1: Geometric partition related merge modes

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4-1.1* | Geometric (GEO) merge mode as proposed in JVET-O0489 with 32 angles and 5 distance values | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) | Adam Wieckowski (Fraunhofer HHI) Kevin Reuzé (Qualcomm) |
| 4-1.2a | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=32 | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) | Adam Wieckowski (Fraunhofer HHI) |
| 4-1.2b | Geometric (GEO) merge mode with reduced number of partitions with N angles and 4 distance values, i.e., N=24 | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) | Adam Wieckowski (Fraunhofer HHI) |
| 4-1.3 | Withdrawn |  |  |  |
| 4-1.4 | Withdrawn |  |  |  |
| 4-1.5 | GEO + ¼ and ½ adaptive-size partial transform | JVET-O0489 | A. Filippov (Huawei) | Jonathan Pfaff (HHI) Kevin Reuzé (Qualcomm) |
| 4-1.6 | GEO + ½ fixed-size partial transform | JVET-O0489 | A. Filippov (Huawei) | Jonathan Pfaff (HHI) |
| 4-1.7 | Adaptive blending filtering for TPM | JVET-O0513 | H. Gao (Huawei), M. Bläser (RWTH Aachen) | ChunChia Chen (MediaTek) |
| 4-1.8 | Test 4-1.7 + GEO | JVET-O0513 | H. Gao (Huawei), M. Bläser (RWTH Aachen) | ChunChia Chen (MediaTek) |
| 4-1.9 | CIIP with triangular partitions | JVET-O0522 | S. Blasi (BBC) | Man-Shu Chiang (MediaTek) |
| 4-1.10 | Test 4-1.9 + GEO | JVET-O0522 JVET-O0489 | S. Blasi (BBC) H. Gao (Huawei) | Man-Shu Chiang (MediaTek) |
| 4-1.11 | Withdrawn |  |  |  |
| 4-1.12 | Test 4-1.9 + Partial transform as in Test 4.1.5 | JVET-O0489 JVET-O0522 | A. Filippov (Huawei), S. Blasi (BBC) | Jonathan Pfaff (HHI) |
| 4-1.13 | Test 4-1.12 + Test 4.1.5 (GEO with partial transform) | JVET-O0489 JVET-O0522 | A. Filippov (Huawei), S. Blasi (BBC) | Jonathan Pfaff (HHI) |
| 4-1.14 | GEO with TPM-like motion storage method vs GEO with the proposal motion storage method in JVET-N0489 | JVET-O0489 | H. Gao (Huawei) M. Bläser (RWTH Aachen) | X. Xiu (Kwai) Kevin Reuzé (Qualcomm) |
| * It is noted that all the GEO-related tests in CE4-1 signal GEO flag at the bottom of the merge tree to switch between TPM and GEO. |  |  |  |  |

## Subjective evaluation

Subjective assessment by experts is mandatory during the meeting. Subjective-testing method as specified in JVET-O1118 will be followed. The purpose of this section is to draft a specification of the subtests to be conducted, and therefore sufficient material can be prepared by proponents before the upcoming meeting.

For the camera captured content related CE test, there are 11 sequences suggested on CE reflector for subjective evaluation so far, as shown in the table below. A tentative list of CE tests from proponents for subjective viewing include CE4-1.1, CE4-1.2, CE4-1.5, CE4-1.6, CE4-1.10, CE4-1.12, CE4-1.13, and CE4-1.14.

For a manageable session of subjective evaluation, the maximum number of subtests needs to be constrained within 60. Therefore, conduct subjective evaluation for all the suggested CE tests using the sequences and configurations as shown in the table below is not possible. Please be noted that the selection of sequences, the list of CE tests to be evaluated, and the configuration of each subtests is not finalized yet, and will be decided before the subjective viewing in the upcoming JVET meeting. The principle is to reduce the number of subtests to be less than or equal to 60.

| # | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Tango2 | 3840x2160 | 60fps | 294 | RA | 32,37 |
| 2 | Catrobot1 | 3840x2160 | 60fps | 300 | RA | 32,37 |
| 3 | Parkrunning3 | 3840x2160 | 50fps | 300 | RA | 32,37 |
| 4 | Cactus | 1920x1080 | 50fps | 500 | RA/LDB | 32, 37 |
| 5 | ParkScene | 1920x1080 | 24fps | 240 | RA/LDB | 32, 37 |
| 6 | Kimono | 1920x1080 | 24fps | 240 | RA/LDB | 32, 37 |
| 7 | BQMall | 832x480 | 60fps | 600 | RA/LDB | 32, 37 |
| 8 | RaceHorsesC | 832x480 | 30fps | 300 | RA/LDB | 32, 37 |
| 9 | BasketballDrill | 832x480 | 50fps | 500 | RA/LDB | 32, 37 |
| 10 | KristenAndSara | 1280x720 | 60fps | 600 | RA/LDB | 32, 37 |
| 11 | Johnny | 1280x720 | 60fps | 600 | RA/LDB | 32, 37 |

Another viewing session is planned for screen-content related CE tests, CE4-1.7 and CE4-1.8. The suggested configuration of all subtest is described in the table below.

| # | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Desktop_420 | 1920x1080 | 60fps | 600 | RA/LDB | 32, 37 |
| 2 | Console_420 | 1920x1080 | 60fps | 600 | RA/LDB | 32, 37 |
| 3 | Desktop_444 | 1920x1080 | 60fps | 600 | RA/LDB | 32, 37 |
| 4 | Console_444 | 1920x1080 | 60fps | 600 | RA/LDB | 32, 37 |

Though the specification of the subtests of the two session as described above is not finalized yet, proponents are requested to provide the bitstreams of all subtests specified in the two tables above, as well as the MD5 checksum of the decoded YUV files. Crosscheckers are requested to decode the bitstreams with the decoder compiled from the software of each CE test and confirm whether the decoded YUV files match those provided from proponent.

## Detailed description of tests

### Test 4-1.1 ~ 4-1.2 and Test 4-1.14: Geometrical partitioning for inter blocks in JVET-O0489 (Huawei, RWTH Aachen)

The figure below illustrates TPM in VVC-5 and additional shapes proposed for non-rectangular inter blocks.

VVC-5                   Proposal

…

Proposal and TPM design in VVC-5.

Overall number of GEO partitioning is 140. The total number can be obtained as 12x5 + 20x4 = 140, because binary tree and triangle splits are excluded and the angles larger than or equal to 180 degrees do not have distance 0. This number found to be optimal from view point flexibility vs signalling. Additional signalling for GEO requires signaling for the angle , and separation line displacement relatively to the center of the block .  represents a quantized angle between 0 and 360 degrees and  represents a distance with 5 different values. The values  and  pairs are stored in a table of size 140x(3+5)/8=140 bytes.

GEO partitioning signalling

Same to TPM, the proposed GEO partitioning for inter is allowed for uni-predicted blocks not smaller than 88 in order to have the same memory bandwidth with bi-predicted blocks at decoder side. Motion vector prediction for GEO partitioning is aligned with TPM. As well as in TPM blending between 2 predictions is applied on inner boundary.

##### Geo blending Mask:

In the VTM implementation of GEO, the blending mask is calculated on the fly, as reported in JVET-O0489. According to the implementation, the first term ((x<<1) + 1)*Dis[displacementX] requires one addition operation per pixel, the second term ((y<<1) + 1))*Dis[displacementY] requires one addition per pixel row. In summary for worst case (4x4 blcok) 1.25 additions are requireds per pixel to obtain the blending mask.

The GEO blending mask is derived with following equations:

distFromLine = ((x<<1) + 1)*Dis[displacementX] + ((y<<1) + 1))*Dis[displacementY] –rho         (1)

distScaled = min((abs(distFromLine) + 4) >> 3, 26)						    (2)

sampleWeightL[ x ][ y ] = distFromLine <= 0 ? GeoFilter[distScaled] : 8 − GeoFilter[distScaled]      (3)

In the Eq. (1), displacementX is equal to the current AngleIdx, displacementY = (displacementX + NumAngles >> 2 ) % NumAngles, where NumAngles is equal to 32 as reported in JVET-O0489. The integer look-up Table of Dis[.] with 32 entries are shown in the following figure:

rho in Eq. (1) are derived as DistanceIdx * (rhoStep[ratio][AngleIdx] ≪log2ShortEdge) + w*Dis[displacementX] + h* Dis[displacementY]. In entries in rhoStep[ratio][AngleIdx] is equal to rhoMax/NumDistance, where rhoMax is calculated once each different aspect ratio with Eq. (4):

ρmaxφ,w,h=cos(φ)h2tanπ2-φ+w2-1.0, 0<φ<π2 		(4)

Where h and w are width and length of a coding block, φ is the quantized angle and log2ShortEdge is the log2 scaled short edge length. Note that since rhpMax are symmetric scalable, only the angles in the first quadrant need to be calculated. Since only 4 possible aspect ratios for GEO, and 9 angleIdx in the first quadrant (including 0 and 90 degree), rhoStep[.][.] lookup table has in total 4*9=36 entries. Note that the value constant value 1.0 in Eq. (4) is designed for 5 distances test, the value might be changed for reduced number of distances.

The chroma blending mask is subsamples from the top-left luma sample weights of each 2x2.

The geo_flag is using 4 context models, first 3 are based on whether above and left neighboring blocks are using GEO, the fourth is based on the ratio of the current block. The CABAC initial values is optimized.

##### Motion storage (detailed description in JVET-O0489):

The sample weight values of the 4 corners of the 4x4 motion storage unit are summed up. If the sum is smaller than th2 and greater than th1, bi-prediction motion information is stored. Otherwise uni-prediction information is stored depending on if sum is smaller than or equal to th1.

th1 and th2 are set as 32 >> thScaler and 32 – th1 respectively, where thScaler is equal to ((log2h+log2w) >> 1) - 1. The derivation process for bi-prediction motion information is same as VVC draft 6.

##### Deblocking:

Similar to TPM, the motion map that is used in spatial and temporal MV prediction is used for deblocking filter boundary strength derivation of CU block boundaries.

- Test 4-1.1: Geometric (GEO) merge mode as proposed in JVET-O0489. Use 32 quantized angle and 5 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.2a: Test 4-1.1 + Use 32 quantized angles and 4 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.2b: Test 4-1.1 + Use 24 quantized angles and 4 quantized distance values (except for partitions achievable by binary split and TPM).

- Test 4-1.3: GEO with further encoder-only optimization algorithm on top of Test 4-1.2.

- Test 4-1.14: in this test, GEO merge mode with the TPM-like motion storage method (based on distFromLine) vs GEO merge mode with the reported motion storage in JVET-O0489 is tested.

### Test 4-1.5: Partial transform that uses ¼ and ½ adaptive-size transform.

Partial transform for GEO predicted inter blocks

Since GEO partitioning provides superior flexibility for inter-prediction even objects with complicated shapes are predicted well, so residual is smaller compared to rectangular or just triangular blocks. It was noticed that non-zero residual for GEO blocks is observed mainly around inner boundary. So, it is suggested to reduce the residual the size or WH blocks to either W(H/n) or (W/n)H keeping non-zero residual only around inner boundary. Essentially with this change residual size to be processed and number coefficient to be signalled become n times smaller. The value of n is signaled within the bitstream, and could be equal to 1 (when partial transform is not applied), 2 or 4. Note that in this proposed partial transform is applied to GEO and TPM blocks.

In this test, the size of the area that is selected from the PU to be transformed is defined adaptively in accordance with:

- GEO partitioning information and the side of the PU that is being split by GEO mode.

- signalled syntax elements that defines the ratio between a side length of the PU and a corresponding side length (either width or height) of the partial transform area specified for the PU.

Signaling of the Partial transform size is performed within the following steps:

- Determine whether cbf is nonzero. If cbf is zero PT is not used and no PT signaling is performed

- Determine GEO parameters from GEO partition index. This is done by applying bitmask operations on partition index. The lower 5 bits correspond to the angle parameter, the higher bits correspond to distance parameters.

- Apply a lookup table to the extracted parameters to derive the sides of the PU that should belong to both resulting GEO partitions.

- Based on the length of the sides, determine the maximum split depth allowable for the PU.

  - If the length of a shorter side is 4 samples, no PT signaling is performed and PT is not applicable.

  - If the length of a shorter side is 8 samples, a single ½ PT signaling is allowed, that is signalled using a binary flag.

  - Otherwise, unary code is used to signal whether PT is not used, a ½ PT (codeword 1 0) or a ¼ PT (codeword 1 1) should be applied.

A PT area (shown as hatched region in the Figure above) is defined by two boundary positions which are intersection points of a GEO splitting line and a boundary of a PU. The width and the height of a transform block derived from the PT area (as shown in the Figure) are a power-of-two integers. Therefore, the starting and ending point positions are adjusted:

- For the shorter side, boundary position is adjusted, so that PT area side is aligned with the PU side and is within the area of the PU;

- In case another boundary position is not on the opposite side, it is quantized to the closest power-of-two value;

- the PT area is specified by the line that is defined by the two modified boundary positions. For each position along this line, the number of samples which is equal to ½ or ¼ of the shorter side are selected column-wise in case the shorter side is the left or right side of the PU, or row-wise in case the shorter side is the top side or bottom side of the PU. The center position of the selected samples is the position of the line that connects the two modified boundary positions.

### Test 4-1.6: Partial transform that uses ½ fixed-size partial transform method is tested.

This test introduces reproduces the test 4-1.5, but introduces constraints on the size of partial transform area. In particular, the size of partial transform is not signaled in a bitstream but is derived from the size of the PU and prediction mode of the PU (e.g., triangular or GEO).

### Test 4-1.7 ~ 4-1.8: Adaptive blending filtering for TPM in JVET-O0513 (Huawei, RWTH Aachen)

Proposed solution further extends flexibility of Triangle Partition Mode by introducing a control flag to disable the blending filtering operation. When the blending filtering is disabled, the sample weight for weighted combination can either be 0 or 8, the weighting matrix example is shown as in following figure.

In Test 4-1.8, the blending off method is further tested on top of geometric merge mode.

### Test CE4-1.9: CIIP with triangular partitions in JVET-O0522 (BBC)

In JVET-O0522, CIIP with triangular partitions are proposed. An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the partitioning scheme as in TPM mode. The intra predictor is used in one of the two triangular partitions, and the inter predictor is used in the other triangular partition. The blending scheme as used in TPM is used to blend the two triangular partitions.

### Test CE4-1.10: CIIP with triangular and GEO partitions (BBC, Huawei, RWTH Aachen)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with the GEO partitioning scheme proposed in JVET-O0489.

### Test CE4-1.11: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.7 Adaptive blending filtering in JVET-O0513 (BBC)

Withdrawn

### Test CE4-1.12: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.5 Partial transform in JVET-O0489 (Huawei, BBC)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with partial transform in JVET-O0489 (with no geometric partitioning itself) to keep residues localized along boundary between partitions.

In JVET-O0522 additional three CIIP modes are introduced. In this CE test, the area of a PU that is selected to be transformed is defined according to the split direction and to the prediction modes that are specified for each of the triangular CIIP block in accordance with the CIIP triangular mode.

### Test CE4-1.13: Combination of Test 4-1.9 in JVET-O0522 and Test 4-1.5 Geometrical partitioning with partial transform in JVET-O0489 (Huawei, BBC)

This test combines the CIIP with triangular partitions proposed in JVET-O0522 with partial transform in JVET-O0489 (with geometric partitioning enabled) to keep residues localized along boundary between partitions.

# CE4-2: Simplification on PROF and BDOF

## Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4-2.1 | 1/32-pel precision of PROF motion refinement + [-32, 31] for PROF clipping range | JVET-O1142 | X. Xiu (Kwai) H. Huang (Qualcomm) C.-Y. Lai (MediaTek) Y. He (InterDigital) | G. Li (Tencent) |
| 4-2.2 | [-32, 31] for PROF clipping range | JVET-O1142 | X. Xiu (Kwai) H. Huang (Qualcomm) C.-Y. Lai (MediaTek) Y. He (InterDigital) | G. Li (Tencent) |
| 4-2.3 | Withdrawn |  |  |  |
| 4-2.4 | Withdrawn |  |  |  |
| 4-2.5 | Withdrawn |  |  |  |

## Detailed description of tests

### Test 4-2.1~4-2.2: Harmonization on BDOF and PROF in JVET-O1142 (Kwai, Qualcomm, Mediatek, InterDigital)

In this CE test, two methods will be studied to align the representation bit-depths of the PROF and the BDOF.

In the first method, the precision of the PROF motion refinements is changed from 1/64-pel to 1/32-pel and clipped into the range of [-32, 31].

In the second method, the range of PROF motion refinements is also clipped to [-32, 31] instead of [-64, 63]. In both methods, the clipping that is applied to the derived sample refinements is removed.

In both CE4-2.1 and 2.2, the range of BDOF motion refinements are changed from [-32, 32] to [-32, 31].

### Test 4-2.3 ~ 4-2.5: Restriction on motion vector refinement in BDOF in JVET-O0573 (Bytedance)

Withdrawn

# Test condition and evaluation criteria

Each testing item is required to be compared against VTM-6.0, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-N1010. Simulation with RA and LDB configurations are mandatory.

# Crosscheck report

Crosscheckers shall fill in the table below, and send it to CE coordinators 3 days before T4.

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |

Explanation of each item in the table

- SW match CE description: Y SW implementation match CE description, N mismatch.

- Text match SW: Y text match SW, N mismatch.

- Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

- EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

# SW Integration guideline

All tests are required to be implemented on top of VTM-6.0.

# Timeline

| T1 | 02 Aug., 2019 | Finalize CE description (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | 02 Sep., 2019 | Deliver software packages and begin cross-checking (2 weeks after test model software release) |
| T3 | 10 Sep., 2019 | Freeze software revision and provide full results for proponents Begin final crosschecking for cross-checkers |
| T4 | 17 Sep., 2019 | Upload contribution documents with specification text and complete test results (regular document deadline – 1 week) |

# Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Aachen Univ. | Johannes Sauer Max Bläser | sauer@ient.rwth-aachen.de blaeser@ient.rwth-aachen.de |
| Alibaba | Yan Ye Yu-Chen Sun Jie Chen Mohammed Golam Sarwer | yan.ye@alibaba-inc.com yuchen.s@alibaba-inc.com jiechen.cj@alibaba-inc.com m.sarwer@alibaba-inc.com |
| Apple | Jaehong Chon | jaehong.chon@gmail.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| DJI | Xiaozhen Zheng | xiaozhen.zheng@dji.com |
| Dolby | Peng Yin | pyin@dolby.com |
| Ericsson | Ruoyang Yu Julien Le Tanou | ruoyang.yu@ericsson.com julien.le.tanou@ericsson.com |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Foxconn | Yu-Ciao Yang Po-Han Lin Tsung-Hua Li | yuciaoyang@fginnov.com PohanLin@fginnov.com thli@fginnov.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Futurewei | Jianle Chen | jianle.chen@futurewei.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |
| Huawei | Huanbang Chen Haitao Yang Sychev Maxim Solovyev Timofey Alexey Filippov Vasily Rufitskiy Semih Esenlik Alexander Karabutov Han Gao Semih Esenlik Xiang Ma | chenhuanbang@huawei.com haitao.yang@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com semih.esenlik@huawei.com karabutov.alexander@huawei.com han.gao@huawei.com semih.esenlik@huawei.com maxiang6@huawei.com |
| Intel | Grace Yu | grace.yu@intel.com |
| InterDigital | Yuwen He Hua Yang Wei Chen Saurav Bandyopadhyay | Yuwen.He@interdigital.com hua.yang@interdigtial.com wei.chen@interdigital.com Saurav.Bandyopadhyay@InterDigital.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Ittiam | Sriram Sethuraman | sriram.sethuraman@ittiam.com |
| KDDI | Kyohei Unno Kei Kawamura Yoshitaka Kidani | ky-unno@kddi-research.jp ki-kawamura@kddi.com yo-kidani@kddi.com |
| Kwai | Yi-Wen Chen Xianglin Wang Xiaoyu Xiu Tsung-Chuan Ma | yiwenchen@kwai.com xianglinwang@kwai.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com |
| LGE | Seung Hwan Kim Jaehyun Lim Naeri Park Hyeongmun Jang Jie Zhao Seethal Paluri | seunghwan3.kim@lge.com Jaehyun.lim@lge.com naeri.park@lge.com hm.jang@lge.com jie.zhao@lge.com seethal.paluri@lge.com |
| MediaTek | Yu-Wen Huang Chih-Wei Hsu Ching-Yeh Chen Tzu-Der Chuang Shih-Ta Hsiang Yuling Hsiao Chen-Yen Lai Yu-Chi Su Zhi-Yi Lin Chun-Chia Chen Yucheng Lin | yuwen.huang@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com shih-ta.hsiang@mediatek.com Yuling.Hsiao@mediatek.com Jenny.Lai@mediatek.com Yu-Chi.Su@mediatek.com Zhi-Yi.Lin@mediatek.com chunchia.chen@mediatek.com Yucheng.Lin@mediatek.com |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Nokia | Jani Lainema Alireza Aminlou Ramin Ghaznavi-Youvalari | jani.lainema@nokia.com alireza.aminlou@nokia.com ramin.ghaznavi-youvalari@nokia.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Panasonic | Kiyofumi Abe Tadamasa Toma Che-Wei Kuo Chong Soon Lim | abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com chewei.kuo@sg.panasonic.com chongsoon.lim@sg.panasonic.com |
| PKU | Zhao Wang Suhong Wang Xuewei Meng | zhaowang@pku.edu.cn suhong.wang@pku.edu.cn xwmeng@pku.edu.cn |
| Qualcomm | Wei-Jung Chien Vadim Seregin Chun-Chi Chen Yan Zhang Yu Han Han Huang Chao-Hsiung Hung Luong Pham Van Dmytro Rusanovskyy | wchien@qti.qualcomm.com vseregin@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com hanhuang@qti.qualcomm.com chaohsiu@qti.qualcomm.com lphamvan@qti.qualcomm.com dmytror@qti.qualcomm.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park Yinji Piao | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com yj1003.piao@samsung.com |
| Sharp | Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai Kiran Misra Eiichi Sasaki Tianyang Zhou Andrew Segall | tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp misrak@sharplabs.com eiichi.sasaki@sharp.co.jp zhou.tianyang@sharp.co.jp asegall@sharplabs.com |
| Sony | Kenji Kondo Masaru Ikeda | kenji.kondo@sony.com Masaru.Ikeda@sony.com |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tenceNnt.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |