| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2025 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Loop filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen Andrey Norkin | Tel: Email: | chingyeh.chen@mediatek.com anorkin@netflix.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance loop filtering techniques in the test model (VTM). Techniques covered by this CE are related to improvements to deblocking filter, adaptive loop filter (ALF), bilateral filter, and the Hadamard filter.

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE5-1, CE5-2, and CE5-3, which address aspects of the deblocking filter, ALF, bilateral and Hadamard filters. Participants in this activity are Bytedance, Ericsson, Huawei, MediaTek, and Sharp.

The software basis for this CE is VTM-6.0 [2]. For the test sequences, configurations and test conditions, in JVET-O2010 [1] for SDR sequences are used. For subjective viewing, results with additional sequences encoded with different QPs may be required.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 2 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |
| 3 | Anand Meher Kotra | Huawei | anand.meher.kotra@huawei.com |
| 5 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |
| 6 | Kiran Misra | Sharp | misrak@sharplabs.com |
| 8 | Jenny Lai | MediaTek | Jenny.lai@mediatak.com |
| 9 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |
| 10 | Jack Enhorn | Ericsson | jack.enhorn@ericsson.com |
| 11 | Jacob Ström | Ericsson | Jacob.strom@ericsson.com |
| 12 | Jizheng Xu | Bytedance | xujizheng@bytedance.com |

# Test conditions and evaluation criteria

The proposals should be encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-6.0 according to the CTC [1].

Anchor-2 is VTM-6.0 with ALF switched off (other conditions are the same as in Anchor 2).

The proposals in the CE shall be implemented based on VTM-6.0. The results for two test conditions shall be provided for CE5-1, CE5-2, and CE5-3: Test Conditions 1 use configuration of Anchor 1, Test Conditions 2 use configuration of Anchor 2. The results for LDP configuration are mandatory for this CE.

Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Deblocking filter

- Adaptive Loop Filter

- Bilateral filter

- Hadamard filter

- Sample adaptive offset

Different technologies will be compared at a subjective viewing at the next meeting.

# CE5-1: Deblocking filtering

## CE5-1.1: Deblocking at DMVR boundaries

Proposal in this sub-CE investigate effect of using DMVR refined motion for deblocking decisions and also the effect of turning off DMVR.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.1.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com #### JVET-O0158 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE5-1.1.2 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-O0158 + restriction | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE5-1.1.3 DMVR off | Kenneth Andersson kenneth.r.andersson@ericsson.com |  |

## Description of technologies in CE5-1.1

#### CE5-1.1.1 Deblocking based on DMVR refined motion JVET-O0158

It has been shown in JVET-O0158 that block artifacts can occur for CU and subblock boundaries of DMVR. The reason is that VVC use unrefined motion for deblocking decisions.

The test investigates the effect of using the DMVR refined motion for deblocking decisions to make deblocking aware of actual difference in motion of DMVR predicted subblocks.

#### CE5-1.1.2 Restriction of CE5-1.1.1 (JVET-O0158)

This test is the same as CE5-1.1.1 with the restriction to not use refined motion across horizontal CTU boundary or CU boundary. To be defined before final software is uploaded.

#### CE5-1.1.3 DMVR off

The test switches off DMVR for comparison with tests CE5-1.1.1 and CE5-1.1.1.

# CE5-1.2: Deblocking simplifications

Proposal in this sub-CE investigates changes to simplify the chroma deblocking filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.2 | Jizheng Xu xujizheng@bytedance.com JVET-O0566 |  |

## Description of technologies in CE5-1.2

#### CE5-1.2 Aligning deblocking chroma decisions JVET-O0566

In the current VVC design, when one side of the chroma blocks are of intra, the boundary strength is set to 2. Otherwise, when one chroma block has non-zero transform coefficients, the boundary is set to 1. Thus, it is possible the bS=1 for Cb and bS=0 for Cr. The tests sets bS=1 for both Cb and Cr when one component has non-zero transform coefficients.

Performance of the proposal on HDR material is also to be tested.

# CE5-1.3 Longer tap deblocking filter at chroma CTB boundaries.

Proposal in this sub-CE investigates changes to longer tap filter application at Chroma CTB boundaries

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.3 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-O0803 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

#### CE5-1.3 Longer tap filter application at Chroma CTB boundaries JVET-O0803

In VTM-5.0, longer tap filter is turned off at the Chroma CTB boundaries, unlike the Luma CTB boundaries where a longer tap asymmetric filter (3+7) is applied.

This test proposes unification of deblocking design by also applying a longer tap asymmetric filter (1+3) at the Chroma CTB boundaries as shown in the figure below.

At the horizontal CTU boundaries the modified longer tap filter decisions are as follows: (p2,0, p2,1  are simply replaced with p1,0  and  p1,1)

dp0 = Abs( p21,0 − 2 * p1,0 + p0,0 )

dp1 = Abs( p21,1 − 2 * p1,1 + p0,1 )

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )

dq1 = Abs( q2,1 − 2 * q1,1 + q0,1 )

At the horizontal CTU boundaries the modified longer tap deblocking equations are as follows:

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + 3 * p1 + 2 * p0 + q0 + q1 + q2  + 4 )  >>  3 )

q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + 2 * p1 + p0 + 2 * q0 + q1 + q2 + q3  + 4 )  >>  3 )

q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3  + 4 )  >>  3 )

q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 )  >>  3 )

# CE5-2: Adaptive loop filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-2.1 | Kiran Misra misrak@sharplabs.com JVET-O0636 |  |
| CE5-2.1 | Kiran Misra misrak@sharplabs.com JVET-O0636 |  |

#### CE5-2.1 Cross-component adaptive loop filter for chroma JVET-O0636

The contribution proposes a tool called the Cross-Component Adaptive Loop Filter (CC-ALF). CC-ALF operates as part of the adaptive loop filter process and makes use of luma sample values to refine each chroma component.

#### CE5-2.2 Cross-component adaptive post-processing filter for chroma JVET-O0636

This is the same proposal as in the CR5-2.1 implementing as post-processing filter.

# CE5-3 Bilateral/Hadamard loop filter tests

This CE would evaluate and compare the bilateral and Hadamard loop filters (CE5-3.1 and CE5-3.2). It would also evaluate SAO version that allows sharpening (CE5-3.3).

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-3.1 | Jacob Strom jacob.strom@ericsson.com JVET-O0548 | Sergey Ikonin sergey.ikonin@huawei.com |
| CE5-3.2 | Sergey Ikonin sergey.ikonin@huawei.com JVET-O1120 |  |
| CE5-3.3 | Jenny Lai jenny.lai@mediatek.com JVET-K0233 |  |

In this sub-CE, we will investigate both the bilateral filter from JVET-O0548 and the Hadamard filter from JVET-O1120 under the same conditions. There will be at least four tests for each filter. Moreover, sample adaptive offset (SAO) without edge offset (EO) sign constraint is also tested.

- Test 1: Filtering in combination with SAO. This will be the base-line implementation similar to test1 in JVET-O0548 and test3 in JVET-O1120 but implemented in VTM-6.0. Hence it will be allowed to apply the filtering on the reconstructed block in the RDO (mode selection part) of the encoder.

- Test 2: Filtering without SAO. In this implementation, SAO is turned off. Otherwise it is similar to test 1.

- Test 3: Filtering in combination with SAO without RDO. In this implementation, it will not be allowed to apply filtering on the reconstructed block in the RDO (mode selection part) of the encoder. Otherwise it is similar to test 1.

- Test 4: Filtering as a post filter. In this implementation, the filtering will be applied outside the coding loop, as if it was used as a post filter. Hence the sample input to the filter should be the same as the output yuv samples of the decoded anchor. Apart from the sample input, the filter may use information available to the decoder, such as QP and/or transform size. Only parameters available in the decoder anchor bit streams are allowed.

## CE5-3.1 Bilateral filter JVET-O0548

This sub-CE will contain following tests based on JVET-O0548.

- CE5-3.1.1 This will follow the test 1 restrictions outlined in the previous section

- CE5-3.1.2 This will follow the test 2 restrictions outlined in the previous section

- CE5-3.1.3 This will follow the test 3 restrictions outlined in the previous section

- CE5-3.1.4 This will follow the test 4 restrictions outlined in the previous section

## CE5-3.2 Hadamard filter JVET-O1120

This sub-CE will contain following tests based on JVET-O1120.

- CE5-3.2.1 This will follow the test 1 restrictions outlined in the previous section

- CE5-3.2.2 This will follow the test 2 restrictions outlined in the previous section

- CE5-3.2.3 This will follow the test 3 restrictions outlined in the previous section

- CE5-3.2.4 This will follow the test 4 restrictions outlined in the previous section

## CE5-3.3 Sample adaptive offset JVET-K0233

In current VVC draft, the signs of SAO EO are inferred in order to allow smooth filtering only. In JVET-K0233, this constraint is removed. That is, the signs of SAO EO are explicitly signalled, so both of smooth filtering and sharp filtering are allowed.

- CE5-3.3.1: Remove the sign constraint of SAO EO

- CE5-3.3.2: Remove SAO

# Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

# Evaluation Criteria

1. Measure impact on Bitrate/PSNR. The BD-rate is calculated for CTC and for CTC with ALF switched off relative to Anchor-1 and Anchor-2 respectively. The results for LDP configurations are mandatory for both CTC and CTC with ALF switched off. For CE5-2, the BD-rate is only calculated for CTC. For CE5-3, the results with SAO switch off are also obtained.

2. Visual impact on output pictures of representative sequences. The subjective test is expected to be held to determine the visual impact for proposals in CE5-1, CE5-2, and CE5-3. The subjective test will use 10 second versions of the sequences. The proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity of the proposals evaluated in the CE shall be compared. In particular, the following aspects of the proposals shall be summarized for the deblocking filters (CE5-1):

- decoding/encoding time vs the anchor. Decoding time shall be measured on the same computer without writing output pictures to the disk (not cluster times).

- number of samples from the block boundary that can be modified by deblocking (may be per block size)

- number of samples from the block boundary that are needed for deblocking decisions (may be per block size)

- number of filtering operations per sample including decisions and filtering (such as additions, multiplications, comparisons)

- number of line buffers

- parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units

- whether proposals increase the worst case complexity and if so, by how much

- data should be provided for both luma and/or chroma if they are modified

4. Decoding time change of the proposals in 5-1 compared to the anchor shall be reported (measured on the same computer).

# Subjective test(s)

Expert viewing at the Geneva meeting will be performed for proposals in CE5-1 and CE5-3.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1].

The subjective test sequences shall be provided by XX-X, 2019 (uploaded to the JVET ftp folder /ce/JVET-O2025_loop_filters), coding configuration with ALF ON and ALF OFF shall be used. The sequences listed in this section shall also be available at the Geneva meeting from the proponents for their corresponding sub-CEs (CE5-1, CE5-2, CE5-3).

The anchors for the subjective viewing of deblocking and SAO proposals will use the ALF ON and ALF OFF configurations. Anchors will be provided by a volunteer.

The following sequences and QP will be used for subjective evaluation in CE5-x category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| 02 | Campfire | 3840x2160p | 30fps | 300 | RA | 37, 32 |
| 03 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 06 | Johnny | 1280x720p | 60fps | 600 | LDB | 32, 37 |

* First 300 frames of the RedKayak sequence shall be used.

The following sequences and QP will be used for subjective evaluation in CE5-2 category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | Redkayak | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| 02 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| 03 | FourPeople | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 04 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 05 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |
| 06 | EBULupoAndCandleLight YCbCr 4:4:4 | 1920x1080p | 50fps | 500 | LDB | 32, 37 |

- First 300 frames of the RedKayak sequence shall be used.

- First 500 frames of the EBULupoAndCandleLight shall be used

The following HDR PQ video material is planned.to be evaluated subjectively for CE5-1.2 to study its potential effect on the HDR material.

The test material at resolution lower than 4K will be expanded to full screen by the player and evaluated together with the 4K test material.

The HDR material will be shown at the HDR-capable display.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off for CE5-1 and CE5-3. Also, test with SAO switched off is planned for CE5-3. In addition, the proponents of these sub-tests need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds. The complexity of the proposals according to Section 7 should be reported by the proponents.

The test conditions are according the CTC on VTM for CE5-2.

Studying effect of the deblocking method in CE5-1.2 on HDR PQ video material is planned.

# Time-line and Responsibilities

T1: 2019-August-2:				   Final CE description uploaded

T2: VTM6.0 release + 2 weeks:	   Cross-checking begins, proponents provide software

T3: 2019-xx-xx: 					Final version of CE software, full test results must be provided at this time. Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-O2010.

- “Algorithm description for Versatile Video Coding and Test Model 5 (VTM5)”, JVET-O2002.