| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2026-v3 |
| --- | --- |

| Title: | Description of Core Experiment 6 (CE6): Transforms and transform signalling |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez Mehdi Salehifar Xin Zhao | Tel: Email: | hegilmez@qti.qualcomm.com mehdi.salehifar@lge.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signalling on top of the Test Model VTM-6.0. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., operation counts, transform selection, will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Introduction

In CE6, a total of 10 tests are defined, which are organized in the following sub-test categories:

- CE6-1: LFNST with one mode (4 tests)

- CE6-2: LFNST with reduced kernels (6 tests)

# Participant list

| Affiliation | Name | E-mail | P/C |
| --- | --- | --- | --- |
| Alibaba | Jie Chen | jiechen.cj@alibaba-inc.com |  |
|  | Ru-Ling Liao | ruling.lrl@alibaba-inc.com |  |
| BBC | Gosala Kulupana | gosala.kulupana@bbc.co.uk |  |
| Broadcom | Minhua Zhou | minhua.zhou@broadcom.com |  |
|  | Peisong Chen | peisong.chen@broadcom.com |  |
|  | Wade Wan | wade.wan@broadcom.com |  |
| Bytedance | Li Zhang | lizhang.idm@bytedance.com |  |
| Cannon | Chris Rosewarne | Chris.Rosewarne@canon.com.au | C |
| Chosun University | Bumshik Lee | bslee@chosun.ac.kr |  |
|  | Sandeep Shrestha | sandeepshrestha407@chosun.kr |  |
| Ericsson | Christopher Hollmann | christopher.hollmann@ericsson.com |  |
| ETRI | Jung Won Kang | jungwon@etri.re.kr |  |
| Fujitsu | Jianqing Zhu | zhujianqing@cn.fujitsu.com |  |
|  | Jie Yao | yaojie@cn.fujitsu.com |  |
| HHI | Mischa Siekmann | mischa.siekmann@hhi.fraunhofer.de | C |
| HiSilicon | Yongbing Lin | linyongbing@hisilicon.com |  |
|  | Jianhua Zheng | zhengjianhua@hisilicon.com |  |
| Huawei | Jianle Chen | jianle.chen@huawei.com | P |
|  | Haitao Yang | haitao.yang@huawei.com |  |
|  | Yin Zhao | yin.zhao@huawei.com |  |
|  | Alexander Karabutov | karabutov.alexander@huawei.com |  |
|  | Semih Esenlik | semih.esenlik@huawei.com |  |
|  | Han Gao | han.gao@huawei.com |  |
| Humax | Yonggil Lee | yglee@humaxdigital.com |  |
| Intel | Samuel Wong | samuel.wong@intel.com |  |
|  | Grace Yu | grace.yu@intel.com |  |
| InterDigital | Saurav Bandyopadhyay | Saurav.Bandyopadhyay@InterDigital.com |  |
|  | Fabrice Le Léannec | Fabrice.Leleannec@InterDigital.com |  |
|  | Fabien Racape | Fabien.Racape@InterDigital.com |  |
|  | Eduard Francois | Edouard.Francois@InterDigital.com |  |
|  | Karam Naser | Karam.NASER@InterDigital.com |  |
| Kwai | Xianglin Wang | xianglinwang@kwai.com |  |
|  | Yi-Wen Chen | yiwenchen@kwai.com |  |
|  | Xiaoyu Xiu | xiaoyuxiu@kwai.com |  |
|  | T.-C. Ma | tsung-chuanma@kwai.com |  |
|  | H.-J. Jhu | jhuhong-jheng@kwai.com |  |
| LGE | Seung-Hwan Kim | seunghwan3.kim@lge.com | C |
|  | Mehdi Salehifar | mehdi.salehifar@lge.com |  |
|  | Moonmo Koo | moonmo.koo@lge.com |  |
|  | Jaehyun Lim | jaehyun.lim@lge.com |  |
|  | Hyeongmun Jang | hm.jang@lge.com |  |
| MediaTek | Olena Chubach | Olena.Chubach@mediatek.com | P |
|  | M.-S. Chiang | man-shu.chiang@mediatek.com |  |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |  |
| Nokia | Jani Lainema | jani.lainema@nokia.com | P |
| Oppo | Fannie.Yu | yuyuanfang@oppo.com |  |
| Orange | Pierrick Philippe | pierrick.philippe@orange.com |  |
| Panasonic | Kiyofumi Abe | abe.kiyo@jp.panasonic.com |  |
|  | Toma Tadamasa | toma.tadamasa@jp.panasonic.com |  |
|  | Yusuke Kato | kato.yusuke003@jp.panasonic.com |  |
| Qualcomm | Amir Said | asaid@qti.qualcomm.com | C |
|  | Chao-Hsiung Hung | chaohsiu@qti.qualcomm.com |  |
|  | Hilmi E. Egilmez | hegilmez@qti.qualcomm.com |  |
|  | Nan Hu | nanh@qti.qualcomm.com |  |
|  | Yung-Hsuan Chao | yunghsua@qti.qualcomm.com |  |
|  | Vadim Seregin | vseregin@qti.qualcomm.com |  |
|  | Ted Hsieh | thsieh@qti.qualcomm.com |  |
| Samsung | Kiho Choi | kiho14.choi@samsung.com |  |
| Sejong Univ. | Dong-jae Won | bigdj2002@sju.ac.kr |  |
| Sharp | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp | P |
|  | Tianyang Zhou | zhou.tianyang@sharp.co.jp |  |
|  | Yukinobu Yasugi | yasugi.yukinobu@sharp.co.jp |  |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com |  |
|  | Takeshi Tsukuba | takeshi.tsukuba@sony.com |  |
|  | Teruhiko Suzuki | teruhiko.s@sony.com |  |
| Tencent | Xin Zhao | xinzzhao@tencent.com | P/C |
|  | Xiang Li | xlxiangli@tencent.com |  |
|  | Liang Zhao | leolzhao@tencent.com |  |
|  | Sehoon Yea | sehoonyea@tencent.com |  |
| Ubilinx | Eric Chai | eric.chai@ublnx.com |  |
|  | Weimin Zeng | weimin.zeng@ublnx.com |  |
| WILUS Inc. | Jason Jung | jason.jung@wilusgroup.com |  |
|  | John Son | john.son@wilusgroup.com |  |
| ZTE | Hong Wang | philcage@163.com |  |

# Description of tools

## CE6-1: LFNST with one mode

CE6-1.1: JVET-O0292 CE6-related: Simplifications for LFNST [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] and JVET-O0350 CE6-related: LFNST with one mode [J. Lainema (Nokia)]

It is proposed to remove the option of lfnst_idx equal to 2 for encoding time reduction. lfnst_idx equal to 0 means LFNST is not used and lfnst_idx equal to 1 means LFNST is used. The LFNST kernel in a transform set is specified according to a pre-defined method in terms of the intra prediction mode of the current CU. Different methods with normative and encoder-only change are tested. The method described in JVET-O0292 is used in test CE6-1.1.a and CE6-1.1.b and the method described in JVET-O0350 is used in test CE6-1.1.c and CE6-1.1.d.

Method described in JVET-O0292:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel A, where A is 1 or 2, is selected by intra prediction mode (INTRA_LT_CCLM, INTRA_L_CCLM, INTRA_T_CCLM, INTRA_PLANAR, INTRA_DC, INTRA_ANGULAR2~INTRA_ANGULAR66).

If intra prediction mode is 0~34, A is set to be “Intra prediction mode % 2 == 0 ? 1 : 2.”

Otherwise if intra prediction mode is 35~83, A is set to be “Intra prediction mode % 2 == 0 ? 2 : 1.”

Method described in JVET-O0350:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6-1.1a | LFNST kernel selection according to the equation in JVET-O0292 with normative change | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1b | LFNST kernel selection according to the equation in JVET-O0292 with encoder-only change | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1c | LFNST kernel selection according to the equation in JVET-O0292 with encoder-only change and the last bit used to signal LFNST index is context coded. | M.-S. Chiang (MediaTek) J. Lainema (Nokia) | M. Koo (LGE) |
| 6-1.1d | LFNST kernel selection according to the equation in JVET-O0350 with normative change | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |
| 6-1.1e | LFNST kernel selection according to the equation in JVET-O0350 with encoder-only change | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |
| 6-1.1f | LFNST kernel selection according to the equation in JVET-O0350 with encoder-only change and the last bit used to signal LFNST index is context coded. | J. Lainema (Nokia) M.-S. Chiang (MediaTek) | M. Koo (LGE) |

## CE6-2: LFNST transform with reduced kernels

CE6-2.1: JVET-O0195 Non-CE6: Simplification on LFNST [Y. Zhao, H. Yang (Huawei)]

It is proposed to use 3 transform sets. Transform set 2 (thus the matrices related to this set) is removed. Accordingly, the mapping table from predModeIntra to lfnstTrSetIdx is changed to Table 1. LFNST transform matrices are not modified. Because of the reduction of transform matrices from 4 sets to 3 sets, 25% LFNST matrix memory (i.e., 2KB ) is saved.

Table 1 Mapping table with three transform sets

| predModeIntra | lfnstTrSetIdx |
| --- | --- |
| predModeIntra < 0 | 1 |
| 0 <= predModeIntra <= 1 | 0 |
| 2 <= predModeIntra <= 18 | 1 |
| 19 <= predModeIntra <= 49 | 3 |
| 50 <= predModeIntra <= 80 | 1 |
| 81 <= predModeIntra <= 83 | 0 |

According to the new mapping table, directional prediction modes with reference samples from both left and top sides uses transform set 3, and those with reference samples from only one side uses transform set 1, as illustrated in Fig. 1.

Fig. 1 Transform set mapping with 4 sets in VTM5 (left) and the proposed method with 3 sets (right)

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-2.1 | Reduce 4 transform sets to 3 transform sets - set 0 for DC / Planar - set 1 for angular modes between 19 and 49 - set 3 for the other angular modes | Y. Zhao (Huawei) | M. Koo (LGE) |

CE6-2.2: JVET-O0539 CE6-related: Unified LFNST using block size independent kernel [X. Zhao, X. Li, S. Liu (Tencent)]

The current LFNST design applies two different sizes of non-separable secondary transform kernel, the one with size 16x16 is applied for 4xN and Nx4 blocks, and the other one with size 16x48 is applied for larger blocks. To unify the LFNST design, it is proposed to use only the LFNST kernels for all block sizes. For 4xN and Nx4 blocks, only the overlapping part between the current block and the output of the LFNST is calculated.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-2.2a | LFNST with the original 16x48 kernels defined in VTM-6.0 and apply them to all block sizes | X. Zhao (Tencent) | M.Siekmann (HHI) M. Koo (LGE) H. E. Egilmez (Qualcomm) |
| CE6-2.2c | Combined test of CE6-2.2a and CE6-2.1 | X. Zhao (Tencent) | M.Siekmann (HHI) M. Koo (LGE) |
| CE6-2.2d | Combination of CE6-2.2a and CE6-2.3a - For larger block sizes, the new kernels in CE6-2.3a are applied - For 4xN and Nx4 blocks, the new kernels in CE6-2.3a are applied, but only pick the overlapping part with current block as output, as proposed in CE6-2.2a | X. Zhao (Tencent) T. Zhou (Sharp) | M.Siekmann (HHI) M. Koo (LGE) H. E. Egilmez (Qualcomm) |

CE6-2.3: JVET-O0620 Non-CE6: Simplification with new LFNST transform basis [T. Zhou, T. Hashimoto, T. Ikai (Sharp)] [miss] [late]

This test studies the reduction of LFNST matrices by replacing 48x16 matrices with 36x16 matrices. The corresponding position of 36 coefficients are decided depending on stIdx.

The training sequences and software will be released at the software finalization deadline, i.e. T3.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE6-2.3 | Simplification of the low frequency separable transform using 36x16 RST matrices | T. Zhou (Sharp) | M.Siekmann (HHI) M. Koo (LGE) X. Zhao (Tencent) C. Rosewarne (Canon) |

CE6-2.3: Use 36x16 matrices

8 6 6 6 4 4 1 1 6 6 6 6 4 4 4 0 ( lfnstIdx == 0) ( lfnstIdx == 1)

Figure 2: 36 coefficients pattern

# Test conditions

The following test conditions shall be used for this CE.

## Software

The Test Model VTM-6.0 shall be used for these experiments, as specified in JVET-O2010 [1].

## Test configurations

For each test in this CE, it is mandatory to provide coding results under common test conditions (CTC) over VTM-6.0, and the results shall be compared against the anchor obtained using VTM-6.0 under CTC.

The following requirements apply to all tests defined in this CE:

- For experiments having an impact on the transform precision, i.e., modifications on transform cores, results using low-QP configurations, i.e., QP 2, 7, 12 and 17 shall be provided. For low-QP configuration, simulation results using All Intra configuration are mandatory and the total number of frames to be coded is modified using the following encoder configuration: --FramesToBeEncoded=100.

- Any encoder or decoder optimizations (e.g., SIMD or encoder optimizations) that are not an essential part of the proposed method shall be described, and individual experiment results on the related optimizations shall be reported.

Proponents and non-proponents are encouraged to report results under non-CTC conditions.

# Complexity metrics and algorithm analysis

For the understanding of complexity, the following information shall be provided for each test planned for this CE:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations as compared to the VTM, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores (in bytes).

- List of all combinations of transforms block size and transform type used for primary/secondary transformation that are different as compared to VTM.

For the understanding of training transform kernels applied in CE test, if a re-training is performed to obtain a new transform kernel(s) in a CE test, the following questionnaire shall be filled by the Proponent (or Tester) by T3, and the Crosschecker by T4.

|  | Tester | Crosschecker |
| --- | --- | --- |
| Training sequence set (sequences outside CTC test set should be used) | Which sequence set has been used for training the LFNST kernels? | Whether it has been confirmed that the training sequences provided by Tester are used for deriving the modified LFNST kernels applied in CE test. |
| Training software | Whether the software for deriving the LFNST kernels given the training data has been provided. | Whether the training software has been received and confirmed for deriving the modified LFNST kernels applied in CE test. |
| VTM software version used for retrieving training data (test configuration, e.g. CTC) | Provide the VTM software version which is used for retrieving the training data used for generating LFNST kernels. | Whether it has been confirmed that the VTM software version provided by Tester is used for retrieving training data which is used for generating the LFNST kernels applied in CE test. |

Proponents and non-proponents are encouraged to provide analysis on the orthogonality of LFNST kernels.

# Timeline

| T1 | Aug. 02, 2019 | - CE description is finalized and uploaded |
| --- | --- | --- |
| T2 | Sep. 02, 2019 Two weeks after VTM-6.0 software release | - Proponents provide initial software of CE tools - Cross-checkers may start initial study of tools/software |
| T3 | Sep. 10, 2019 Three weeks before the next JVET meeting or one week after T2, whichever date is later | - Software of each CE test is finalized - Cross-checkers start final cross checks |
| T4 | Sep. 17, 2019 One week before the regular document submission deadline | - CE contribution documents including specification text and complete results shall be uploaded to the JVET repository (particularly for proposals targeting to be promoted to the draft standard at the next meeting) - At the same time, crosscheckers shall send their findings out |
| T5 | Sep. 25, 2019 Regular document submission deadline | - The CE summary report shall be available. This shall include documentation about crosscheck of software, matching of CE description and confirmation of the appropriateness of the text changes as well as sufficient crosscheck results to create evidence about correctness (crosscheckers must send this information to the CE coordinator at least 3 days ahead of the document deadline) |

# References

- F. Bossen, J. Boyce, K. Sühring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.