| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0069-v2 |
| --- | --- |

| Title: | CE4: CE4-1.7, CE4-1.8: GEO and TPM Blending Off for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

It is proposed in the 15th Gothenburg JVET meeting that the TPM blending operation is over-smoothing when the content is known to include sharp edges, hence rendering TPM less useful for screen content application. GEO is proposed as an extension of TPM, which also contains the over smoothing issues. In CE4-1.7 and CE4-1.8, TPM blending off and GEO + TPM blending off is tested. For TGM class, the simulation results of CE4-1.7 demonstrate 0.73% and 0.94% luma BD-rate reduction for RA and LDB configurations respectively. For CE4-1.8, 2.38% and 3.11% luma BD-rate reduction for RA and LDB configurations respectively. It is asserted that the blending off operation visibly improves prediction by avoiding over smoothing.

# Introduction

TPM blending off for screen content is first proposed in [1] and defined as CE test (CE4-1.7) in [2]. In the meantime, proposed GEO [3] is also desirable to test blending off for screen content in CE (CE4-1.8).

# Proposal

## High level syntax changes

In order to turn off the blending operation of TPM and GEO, 1 sps level control flag and 1 slice level blending off enable flag is introduced:

7.3.2.3	Sequence parameter set RBSP syntax

…

| if( sps_triangle_enabled_flag ) |  |
| --- | --- |
| sps_blending_ctrl_flag | u(1) |

…

7.3.6.1	General slice header syntax

…

| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_minus1 ) { |  |
| --- | --- |
| max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if( sps_blending_ctrl_flag ) |  |
| slice_blending_off_flag | ue(v) |
| } |  |

## Blending off operation for TPM

It’s proposed for TPM when the slice_blending_off flag is equal to true, the blending operation is turned off, only hard edge between to prediction is used for TPM weighting

- if slice_ blending_off_flag is equal to 0, the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows: wValue = ( cIdx = = 0 ) ? Clip3( 0, 8, wIdx ) : Clip3( 0, 8, wIdx * 2 ) (8-826) - otherwise (slice_triangle_blending_enabled_flag is equal to 1), the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows: if y < (nCbH >>1) : wValue = ( cIdx = = 0 ) ? (wIdx <= 4 ? 0 : 8) : (wIdx * 2 <= 4 ? 0 : 8) (8-826) else : wValue = ( cIdx = = 0 ) ? (wIdx < 4 ? 0 : 8) : (wIdx * 2 < 4 ? 0 : 8) (8-826)

Figure 1 shows one example of the TPM blending off mask:

Figure 1: VTM6.0 TPM blending mask vs proposed TPM blending off mask

## Blending off operation for GEO

The blending mask of GEO mode is derived using look-up table based on the distance between the sample position and the split line. In blending off operation, look up table is replaced with the fix weighting value 8 as:

if slice_blending_off_flag is equal to 0: sampleWeightL[ x ][ y ] = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs] otherwiese (slice_blending_off_flag is equal to 1): sampleWeightL[ x ][ y ] = weightIdx <= 0 ? 8 : 0

# Simulation Results

# CE results

The CE4-1.7 and CE4-1.8 test results for ClassF, ClassTGM and 444 content under CE8 test condition [4] as summarized in Table 1. Details results are attached in excel tables. In both simulations, a hash based slice level encoder-side decision is implemented.

Table 1 Simulation results of CE4-1.7 and CE4-1.8

| #CE | Class | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-1.7 | ClassF | -0.02% | -0.02% | -0.03% | 100% | 100% | 0.02% | 0.00% | 0.48% | 99% | 100% |
|  | TGM420 (1080p) | -0.73% | -0.58% | -0.55% | 100% | 100% | -0.94% | -0.69% | -0.68% | 98% | 100% |
|  | TGM444 | -0.73% | -0.60% | -0.60% | 100% | 100% | -1.09% | -1.11% | -1.05% | 100% | 101% |
|  | 444Overall | -0.24% | -0.20% | -0.23% | 100% | 100% | -0.40% | -0.35% | -0.35% | 100% | 100% |
| CE4-1.8 | ClassF | -0.47% | -0.55% | -0.48% | 101% | 100% | -0.70% | -0.83% | -0.49% | 102% | 99% |
|  | TGM420 (1080p) | -2.35% | -2.01% | -1.98% | 100% | 100% | -3.11% | -2.77% | -2.72% | 99% | 100% |
|  | TGM444 | -2.44% | -2.45% | -2.43% | 100% | 100% | -3.88% | -3.89% | -3.82% | 100% | 101% |
|  | 444Overall | -1.06% | -1.07% | -1.12% | 101% | 100% | -1.65% | -1.69% | -1.64% | 102% | 100% |

## Supplementary results:

Table 2 show the supplementary results for CE4-1.8. In the supplementary results, the anchor is CE4-1.8 with blending on, test is CE4-1.8(Blending off). It shows that the major coding gain of screen content for CE4-1.8 is contributed by blending off operation, which indicated that the blending on operation is not design-wise optimized for SCC.

Table 2 CE4-1.8_BlendingOn_vs_BlendingOff

| Class | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| ClassF |  | -0.15% | -0.06% |  | -0.09% |  | 91% |  |  | 70% |  |  |  |  |  |  |  |  |  |  |
| TGM420 (1080p) |  | -1.95% | -1.38% |  | -1.36% |  | 90% |  |  | 67% |  | -2.65% |  | -2.20% | -2.13% |  | 89% |  | 62% |  |
| TGM444 |  | -1.97% | -1.82% |  | -1.78% |  | 89% |  |  | 59% |  | -3.31% |  | -3.20% | -3.14% |  | 88% |  | 55% |  |
| 444Overall |  | -0.69% | -0.64% |  | -0.66% |  | 92% |  |  | 64% |  |  |  |  |  |  |  |  |  |  |

Note that the encoding and decoding run time for the supplementary results is not accurate.

# References

- S. Esenlik, H. Gao, B. Wang, A. Kotra, Z. Zhao, E. Alshina, M. Blaeser, J. Sauer “Description of Core Experiment 1 (CE1): Reference picture resampling filters”, JVET-O0513, Gothenburg, SE, 3–12 July 2019

- C.-C. Chen, H. Yang, X. Xiu “Description of Core Experiment 1 (CE1): Reference picture resampling filters”, JVET-O2024, Gothenburg, SE, 3–12 July 2019

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, J. Sauer  “Non-CE4: Geometrical partitioning for inter blocks”, JVET-O0489, Gothenburg, SE, 3–12 July 2019

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, X. Xiu “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, Gothenburg, SE, 3–12 July 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.