| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0084 |
| --- | --- |

| Title: | Decoding Energy Assessment of VTM-6.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27105 matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

# Abstract

This contribution describes the decoding energy assessment of the VTM-6.0 decoder. We describe different possibilities to measure the energy of a video decoder on a desktop PC and a Raspberry Pi. The measurements are evaluated with the Bjøntegaard-Delta metric. The evaluation of the decoding energy measurements reveals that the energy demand is increased by approximately 72% in comparison to bit streams of the same sequence coded and decoded with HEVC at equal objective visual quality.

# Introduction

Energy-efficiency is important for mobile devices that have a limited battery lifetime. Besides, the energy-efficiency is not only important for a longer battery-time of smartphones, but also the viewing of online videos impacts the global greenhouse gas emission by a share of approximately 1% in 2018 [1].

The improvement of the bitrate-efficiency of the next-generation video codec Versatile Video Coding (VVC) in comparison to the High Efficiency Video Coding (HEVC) video codec is based on new sophisticated tools and algorithms (e.g. multi-type tree structure of CTUs, triangular prediction partitions) [2]. However, these new tools increase the computational complexity of the video decoding process, which leads to a higher decoding time and energy demand. An analysis of the decoding energy demand was performed for a previous version of the VTM decoder software [3]. In this respect, this document extends the energy assessment from [3] with the following contributions:

- Analysis of a recent version of VTM (VTM-6.0).

- Description of further energy measurement techniques.

- A more detailed analysis for different software versions, hardware, encoder configurations, and input sequences

Chapter 2 presents the measurement of the decoding energy. Chapter 3 shows the test setups for the evaluation of both video codecs. The decoding energy measurements are evaluated and the results for both video codecs are compared with different quality metrics in Chapter 4. Finally, Chapter 5 shows the evaluation for different implementations of HEVC on different devices.

# Energy Measurement

In this chapter, we will explain how the energy demand for the decoding of a bit stream can be measured. First, the fundamentals of energy measurements are described. Afterwards, the practical measurement setups are explained. Finally, the statistical test conditions are given that ensure the accuracy of the energy measurements.

# Fundamentals of Energy Measurements

In general, for the determination of the decoding energy, two measurements are needed. At first, the power demand during the decoding of a bit stream (Pdect) has to be measured. Afterwards, the power demand during the idle mode (Pidlet) has to be measured over an equal duration T. The requirement of two alternating measurements is given by the fact that electronic devices have a static power demand that is not caused directly by the decoding of the bit stream, e.g. the operating system, which we neglect in our considerations.

In order to determine the actual energy demand of the decoding process Edec, the difference between Pdec and Pidle is calculated with

| Edec= t=t0t0+ TPdectdt - t=t1t1+ TPidletdt, | (2.1) |
| --- | --- |

where t0 is the beginning of the measurement of Pdec, t1 of Pidle, and T the duration of the decoding process.

Figure 1: Measurement of the active power demand of a desktop PC during decoding of a bit stream with VTM-6.0 (blue curve) and in idle mode (red curve). The decoding energy corresponds to the grey area between both curves.

Figure 1: Measurement of the active power demand of a desktop PC during decoding of a bit stream with VTM-6.0 (blue curve) and in idle mode (red curve). The decoding energy corresponds to the grey area between both curves.

The decoding power of a bit stream with VTM-6.0 on a desktop PC is visualized in Figure 1. The blue curve shows the power demand during decoding and the red curve shows the power demand during idle mode. The grey area between both curves corresponds to the additional power demand of the decoding process. The decoding of the bit stream starts at approximately 6 s and ends at approximately 37 s. However, it can be seen that the power demand of the red curve is not constant and varies approximately between 30 W and 40 W. This is because an operating system runs background processes that affect the power demand of the desktop PC, even when it is idle.

# Measurement Setup

Figure 2: Measurement setup for a device-under-test (DUT) with an external power meter for Raspberry Pi (right side) and measurement of desktop PC (left side).

The measurements in this contribution are carried out on two different test devices. First, a desktop PC is used. The PC runs CentOS 7 as an operating system and has an Intel i7-8700 CPU, which has 6 cores with a base frequency of 3.20 GHz [4]. Another device is the Raspberry Pi 3B+, which has a quad-core CPU with a clock frequency of 1.4 GHz [5] and Raspbian Stretch as an operating system.

Both devices are measured with an external power meter as shown in Figure 2. The external power meter is the LMG95 high precision power meter by ZES ZIMMER. For the desktop PC, the setup on the left side is used to measure the decoding energy and for the Raspberry Pi, the setup on the right side is used. With the latter setup, it is also possible to measure other devices that are powered over an USB-cable or battery with a constant voltage, e.g. smartphones and evaluation boards. The desktop PC is supplied with an AC voltage of 230 V, whereas the Raspberry Pi is supplied with a DC voltage of 5V, which is shown on the right side of Figure 2.

Another possibility to measure the energy demand of the decoding process without an external power meter is Running Average Power Limit (RAPL), which is an integrated power meter within modern Intel CPUs. With this power meter, it is possible to measure the energy demand of the entire CPU socket and of each component, which are the CPU’s core and uncore components like the memory controller and last level caches [6]. The socket and each component have a counter that is accessible over the file system and counts the energy in units of µJ. For this contribution, only the counter of the complete socket is of interest. The counter for the entire socket can be accessed at:

/sys/class/powercap/intel-rapl/intel-rapl:0/energy_uj

With RAPL, no additional hardware is necessary to measure the energy demand. However, it is only possible to measure the energy demand of the CPU socket and not of other components of the desktop PC (e.g. hard drive, GPU). Nevertheless, it can be observed that the CPU has a major impact on the decoding energy for software-based decoders like HM or VTM.

# Statistical Test Conditions

The power measurement is affected by different disturbances, e.g. background processes of the operating system and the ambient temperature. Therefore, we assume that one measurement is not sufficient to obtain the actual Edec. To establish a certain quality level for the measurement of Edec, a series of measurements is performed. The accuracy of this measurement series is evaluated with a confidence interval test that is defined by

| 2⋅σm⋅tαm-1<β⋅Edec , | (2.2) |
| --- | --- |

where m is the number of measurements,  σ the standard deviation of the measurement, Edec the mean value of the energy measurements, β the allowed deviation of the actual decoding energy value, tα the critical t-value of the Student’s t-distribution, where α is the probability that the actual decoding energy is within the allowed deviation. In this contribution, the value of β is 0.02 and α is 0.99, which means that with a certainty of 99%, Edec is within a deviation of less than 2%.

# Evaluation Test Setup

In this chapter, the sequences for the generation of bit streams with different video codecs are presented. Therefore, the used codecs for the encoding with the corresponding configurations are explained at first. Furthermore, the original sequences and sequence-specific settings are given.

# Video Encoding & Decoding

The decoding energy is assessed for two different codecs. The state-of-the-art video codec High Efficiency Video Coding (HEVC) is compared to the next-generation video codec Versatile Video Coding (VVC) as a reference. For HEVC, the HEVC Test Model (HM) software is used with the version 16.20 [7] and for VVC, the VVC Test Model (VTM) software is used with the version 6.0 [8]. Both softwares encode the sequences and decode the generated bit streams. The Fast Forward MPEG (FFmpeg) software is another possibility for the decoding of HEVC encoded bit streams, which is possible with the included openHEVC implementation. For this contribution, the software version 4.1 [9] is used for the measurement of the decoding energy.

# Set of Sequences

For the evaluation of standard dynamic range (SDR) sequences, the recommendations of [10] are used. The sequences are shown in Table I with the corresponding properties of the class, the resolution, the frames per second (FPS), and the number of encoded sequences. The sequences are separated into seven different classes, which are A1, A2, and B-F. Sequences with a 4K-resolution are encoded with only 100 frames due to time constraints. For the generation of the bit streams, the three configurations intra, lowdelay, and randomaccess are used. The corresponding configuration files for each video codec are given in Table II. The configurations for HM-16.20 are given on the left side and the configurations for VTM-6.0 are given on the right side.

Table 1: List of all used sequences [10] with corresponding properties of class, resolution, frames per second (FPS) and numbers of encoded frames.

| Sequence | Class | Resolution | FPS | #Frames | Sequence | Class | Resolution | FPS | #Frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tango2 | A1 | 3840x2160 | 60 | 100 | PartyScene | C | 832x480 | 50 | 500 |
| FoodMarket4 | A1 | 3840x2160 | 60 | 100 | BasketballDrill | C | 832x480 | 50 | 500 |
| Campfire | A1 | 3840x2160 | 30 | 100 | RaceHorses | D | 416x240 | 30 | 300 |
| CatRobot1 | A2 | 3840x2160 | 60 | 100 | BQSquare | D | 416x240 | 60 | 600 |
| DaylightRoad2 | A2 | 3840x2160 | 60 | 100 | BlowingBubbles | D | 416x240 | 50 | 500 |
| ParkRunning3 | A2 | 3840x2160 | 50 | 100 | BasketballPass | D | 416x240 | 50 | 500 |
| MarketPlace | B | 1920x1080 | 60 | 600 | FourPeople | E | 1280x720 | 60 | 600 |
| RitualDance | B | 1920x1080 | 60 | 600 | Johnny | E | 1280x720 | 60 | 600 |
| Cactus | B | 1920x1080 | 50 | 500 | KristenAndSara | E | 1280x720 | 60 | 600 |
| BasketballDrive | B | 1920x1080 | 50 | 500 | ArenaOfValor | F | 1920x1080 | 60 | 600 |
| BQTerrace | B | 1920x1080 | 60 | 600 | BasketballDrillText | F | 832x480 | 50 | 500 |
| RaceHorses | C | 832x480 | 30 | 300 | SlideEditing | F | 1280x720 | 30 | 300 |
| BQMall | C | 832x480 | 60 | 600 | SlideShow | F | 1280x720 | 50 | 500 |

Table 2: Used configuration files for each used video codec.

| HM-16.20 [7] | VTM-6.0 [8] |
| --- | --- |
| encoder_intra_main10.cfg | encoder_intra_vtm.cfg |
| encoder_lowdelay_main10.cfg | encoder_lowdelay_vtm.cfg |
| encoder_randomaccess_main10.cfg | encoder_randomaccess_vtm.cfg |

# Evaluation of the Decoding Energy Measurements

As a scale for the improvement of a newly developed video codec, the Bjøntegaard-Delta method [11] is commonly used. Therefore, we compared the bitrate-efficiency of HM-16.20 and VTM-6.0 with different Bjøntegaard-Delta metrics. For the evaluation of the decoding energy demand, the Bjøntegaard-Delta-Decoding-Energy (BDDE) is used according to [12]. The BDDE value corresponds to the additional energy demand at an equal distortion. Therefore, a negative value implies that with equal objective visual quality, the decoding of a bit streams demands less energy.

Figure 3: Rate-distortion diagram for the sequence FoodMarket4 with the configuration randomacccess and the QP-values of 22, 27, 32, and 37. The red curve corresponds to HM encoding, the blue curve to VTM encoding.

Figure 3: Rate-distortion diagram for the sequence FoodMarket4 with the configuration randomacccess and the QP-values of 22, 27, 32, and 37. The red curve corresponds to HM encoding, the blue curve to VTM encoding.

In the following, both setups from Chapter 3 will be investigated with the RAPL energy measurements. The rate-distortion diagram in Figure 3 shows that the bit streams that are encoded with VTM-6.0 (blue curve) have a higher YUV-PSNR value at the same bitrate compared to corresponding bit streams that are encoded with HM-16.20 (red curve). The bitrate-efficiency of the sequence FoodMarket4 with configuration randomaccess is improved and the BDR-value is -29.11%.

In Table 3, the evaluation for all Bjøntegaard-Delta metrics is shown and separated according to the configuration and the mean value of all configurations. For the BDR, the difference in rate-distortion efficiency between HM and VTM is -27.55%, for the intra configuration it is -23.70%, for the lowdelay configuration -27.05%, and for the randomaccess configuration -31.95%.

|  | BDR | BDDE |
| --- | --- | --- |
| Intra | -23.70% | +76.11% |
| Lowdelay | -27.05% | +62.08% |
| Randomaccess | -31.91% | +77.85% |
| Mean | -27.55% | +72.01% |

Table 3: Results of the evaluation metrics for the comparison of HM and VTM. For each Bjøntegaard-Delta, the results for three configurations and the mean value over all bit streams are shown.

Figure 4: Decoding-energy-distortion diagram for the sequence DaylightRoad2 with the configuration lowdelay. The red curves correspond to the decoding energy of the decoded bit streams with HM-16.20 and the blue curves to the decoded sequences with VTM-6.0. The curves of the dashed lines are measured with the external power meter, and the other curves with RAPL.

Figure 4: Decoding-energy-distortion diagram for the sequence DaylightRoad2 with the configuration lowdelay. The red curves correspond to the decoding energy of the decoded bit streams with HM-16.20 and the blue curves to the decoded sequences with VTM-6.0. The curves of the dashed lines are measured with the external power meter, and the other curves with RAPL.

However, the improvement of the rate-distortion efficiency is at the cost of a higher decoding energy demand. In Figure 4, the decoding-energy-distortion diagram is shown for the bit streams of the sequences DaylightRoad2 with the configuration lowdelay. The diagram shows that the decoding of bit streams with HM (red curves) has a lower decoding energy demand at an equal objective visual quality than with VTM (blue curves). Choosing HM as a reference, the BDDE-value for the bit streams of Figure 4 is 68.60%. Therefore, the decoding energy demand of the VTM-decoder is increased significantly in comparison to the HM-decoder.

Furthermore, both measurement methods are shown in Figure 4. The measurements with an external power meter are depicted as dashed lines, the solid lines correspond to RAPL measurements. Both methods have a low difference in the determined decoding energy for each video codec. For the HM set, the measurement with LMG has on average an 1.16% higher decoding energy than with RAPL and for the VTM set, the decoding energy is 2.57% higher on average. The higher energy demand of the LMG measurements results from components that are not covered by the RAPL unit. However, as the linear correlation is high, the we assume in the following that the decoding energy can be measured with RAPL with sufficient accuracy and these results are taken for the following evaluation.

The BDDE values of Table 3 are 76.11% for the intra configuration, 62.08% for the lowdelay configuration, 77.85% for the randomaccess configuration, and on average the BDDE value is 72.01%. These findings show that in general, the decoding energy demand is increased significantly. However, there are differences among the configurations. The configuration intra and randomaccess have a similar BDDE-value despite the fact that the BDR-values of both configurations have a higher difference. These differences suggest that the improvement of the bitrate-distortion efficiency for intra-coding comes at a higher cost for the decoding energy efficiency than for randomaccess. It is interesting to note that the lowdelay configuration has the lowest value of BDDE even though the BDR of the lowdelay configuration is higher than for the intra configuration. In further investigations, these observations might help in finding the relation between the higher energy demand and the rate-distortion efficiency.

In Figure 5, the evaluation of BDR and BDDE is shown for each sequence and configuration. The bit-streams that are encoded with the intra configuration are shown as blue markers, with the lowdelay configuration as red markers, and with the randomaccess configuration as yellow markers. The plot shows that the randomaccess bit streams have a higher BDR- and BDDE-value than the bit streams with another configuration. Furthermore, the bit streams with lowdelay configuration have BDR-values between the other configurations and lower BDDE-values, which confirms the observations of Table 3. It is observed that the sequences SlideShow (BDR: -44.32%, BDDE: 48.04%) and SlideEditing (BDR: -37.58%, BDDE: 16.01%) have a low BDR and BDDE-value for the lowdelay configuration. Both of these are screen-content sequences.

Figure 5: Evaluation of all sequences with BDDE and BDR. Each point corresponds to one sequence with a specific configuration. The blue dots correspond to the intra configuration, the red dots to the lowdelay configuration, and the yellow dots to the randomaccess configuration.

Figure 5: Evaluation of all sequences with BDDE and BDR. Each point corresponds to one sequence with a specific configuration. The blue dots correspond to the intra configuration, the red dots to the lowdelay configuration, and the yellow dots to the randomaccess configuration.

# Evaluation of the Implementation on Different Devices

For HEVC encoded bit streams, there are several implementations for decoding. One alternative to the HM-decoder is software decoding with openHEVC, which has a higher energy efficiency. In Table 4, the BDDE values for the decoding with FFmpeg on the Intel CPU are given. For the determination of the BDDE values, the decoding energy measurements with RAPL and the HM-decoder are taken as a reference. In general, the BDDE value is -79.48%, for the intra configuration -82.23%, for the lowdelay configuration -78.11%, and for the randomaccess configuration -78.11%. Thereby, the decoding energy demand of an HEVC-coded bit stream takes approximately one fifth with FFmpeg.

The BDDE value for the Raspberry Pi with FFmpeg as a decoder is -80.69%, which is similar to the results of the Intel CPU. The results for the individual configurations for the Raspberry Pi in Table 4 also show similar results. In Figure 6, the results of each measurement for HEVC decoding are shown for the sequence Campfire with the configuration randomaccess. For these bit streams, the BDDE value is -79.26% with FFmpeg on the Intel CPU and -78.66% on the Raspberry Pi. We claim that future implementations of a VVC-decoder could also achieve similar improvements in comparison to the VTM-software.

Table 4: Evaluation of the BDDE for the decoding with FFmpeg and HM. For the determination of the BDDE, the measured decoding energy of the HM-decoder on the Intel-CPU with RAPL is taken as a reference.

|  | BDDE Intel FFmpeg | BDDE Raspberry FFmpeg |
| --- | --- | --- |
| Intra | -82.23% | -87.55% |
| Lowdelay | -78.11% | -78.09% |
| Randomaccess | -78.11% | -76.44% |
| Mean | -79.48% | -80.69% |

# Conclusion

With the decoding energy measurements, it is determined that the VTM decoder has an increased energy demand for comparable bit streams with equal objective visual quality as compared to HEVC. Furthermore, it could be determined

Figure 6: Evaluation of the decoding energy on different devices for the HEVC video codec for the sequences Campfire with the configuration randomaccess. The measurements on the desktop Pc are shown by the red and yellow curves and on the Raspberry Pi as green curve.

Figure 6: Evaluation of the decoding energy on different devices for the HEVC video codec for the sequences Campfire with the configuration randomaccess. The measurements on the desktop Pc are shown by the red and yellow curves and on the Raspberry Pi as green curve.

that the measurement of the decoding energy has sufficient accuracy with RAPL. Additionally, the following findings were observed:

- Measurement methods for different hardware

- VTM currently needs 70% more decoding energy

- Energy increase is highest for intra, lowest for lowdelay

- A practical implementation (FFmpeg) saves 80% of energy for HEVC, for VTM, a similar amount is expected

- The expected decoding energy for Intel and ARM CPUs is almost the same

- Precise knowledge about VTM decoding energy could finally be used to optimize the VVC video bit stream using decoding-energy-rate-distortion optimization as suggested in [12].

# References

[1] M. Efoui-Hess, “Climate Crisis: The Unsustainable Use of Online Video. The Practical Case for Digital Sobriety”, Jul. 2019.

[2] J. Chen, Y. Ye, S. H. Kim, “Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6)”, JVET-O2002-v1, Jul. 2019.

[3] C. Herglotz, A. Kaup, “Energy Assessment for Video Decoding”, JVET-K0106-v3, Jul. 2018.

[4] Intel, https://ark.intel.com/content/www/us/en/ark/products/126686/intel-core-i7-8700-processor-12m-cache-up-to-4-60-ghz.html, [Accessed: 29.08.2019].

[5] Raspberry Pi Foundation. Raspberry Pi 3 Model B+. www.raspberrypi.org/products/raspberry-pi-3-model-b-plus/, 2019. Accessed 2019-03-19.

[6] K. N. Khan, M. Hirki, T. Niemi, J. K. Nurminen, and Z. Ou. RAPL in Action: Experiences in Using RAPL for Power Measurements, ACM Trans. Model. Perform. Eval. Comput. Syst. 3(2), 9:1-9:26, Mar. 2018.

[7] Joint Collaborative Team on Video Coding (JCT-VC). HEVC test model reference software (HM-16.20). https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-16.20/, 2019. Accessed 2019-07-08.

[8] Joint Video Exploration Team (JVET). VVC test model reference software (VTM-6.0). https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0, 2019. Accessed 2019-08-17.

[9] Fast Forward MPEG (FFmpeg). http://ffmpeg.org/, 2019. Accessed 2019-07-08.

[10] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010-v1, Mar. 2019.

[11] G. Bjontegaard, “Calculation of average PSNR differences between RD-Curves”, VCEG-M33, ITU-T SG 16/Q 6, Austin, TX, Apr. 2001.

[12] C. Herglotz, A. Heindel, and A. Kaup. Decoding-Energy-Rate-Distortion Optimization for Video Coding. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 29(1), 2019.

[13] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack and A. Kaup. Modeling the Energy Consumption of the HEVC Decoding Process. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 28(1), 2018.

[14] T. F. Coleman and Y. Li. An interior trust region approach for nonlinear minimization subject to bounds. SIAM J. Optim., vol. 6, no. 2, pp. 418–445, 1996.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.