| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0087-v2 |
| --- | --- |

| Title: | Deblocking boundary strength modification for triangle and affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Frank Bossen2, Andrew Segall1 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com |
| Source: | 1Sharp Labs of America 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

There are two instances in the VVC draft where motion discontinuity based deblocking is performed only because an edge was marked as an internal transform edge.  It is asserted that this leads to an inconsistent application of deblocking.  This document proposes modifications to the deblocking process to address this design consistency.  The first modification applies to CUs using triangle mode.  In these CUs, the deblocking process performs motion discontinuity-based smoothing/deblocking at the internal transform boundaries. However, it is asserted that this is not well motivated, as the triangle mode makes use of sample wise blending of predictions, and the blending process itself smooths any motion discontinuities.  The second modification applies to CUs using the affine mode.  In these CUs, the current VVC design may perform motion discontinuity-based smoothing on a non-8x8 CU grid only when that boundary is also an internal transform boundary. This is asserted to be poorly motivated, as the transform boundary may correspond to two partitions that do not contain residual (and so deblocking at that specific non-8x8 grid location is inconsistent.)

To address the two above inconsistencies, this contribution proposes to skip motion discontinuity-based smoothing/deblocking for internal edges of (i) a triangle mode CU and (ii) at non 8x8-grid edges of an affine CU. The Bjønteegard Delta (BD) bitrate impact of this change is evaluated for Common Test Conditions (CTC) and found to be:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Random Access | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Low Delay B | -0.01% | 0.10% | -0.03% | 102% | 100% |
| Low Delay P | 0.00% | 0.02% | 0.05% | 99% | 99% |

The changes were also evaluated for ALF OFF and the BD rate impact is reported to be:

|  | Y | U | V |
| --- | --- | --- | --- |
| Random Access | 0.00% | -0.01% | 0.00% |
| Low Delay B | -0.01% | 0.09% | -0.08% |
| Low Delay P | -0.01% | -0.01% | -0.02% |

# Proposal

The contribution proposes the following two changes to the deblocking process in VVC draft

## Skip deblocking based on motion discontinuity for internal edges of Triangle mode CU

In the VVC draft, Coding Units (CUs) with triangle mode may perform motion discontinuity-based smoothing/deblocking at internal transform boundaries CU. However, in triangle mode makes use of sample wise blending of predictions and it is asserted that motion discontinuity-based smoothing is not needed.

## Skip deblocking based on motion discontinuity for non-8x8 CU grid aligned edges of an Affine mode CU

In the VVC draft, Coding Units (CUs) with triangle mode perform motion discontinuity-based deblocking on 8x8 CU grid. However, if a subblock transform edge lies on a non-8x8 CU grid, motion discontinuity-based deblocking is carried out for this edge as well. It is asserted that a more consistent design approach would be to limit motion discontinuity-based deblocking to 8x8 CU grid for all Affine mode CUs.

In the VVC draft, motion continuity-based boundary strength determination is executed as the last step. To skip the motion continuity-based based deblocking process for internal edges of Triangle mode CU and non-8x8 CU grid aligned edges of an Affine mode CU, we set the boundary strength in this step to 0. The relevant VVC draft text changes are included in Appendix of this contribution. Note, JVET contribution JVET-P0043 [1] also proposes a cleanup for de-blocking in the affine and TPM mode

# Experimental Results

The CE tests were implemented in VTM6 [2] and evaluated for CTC [3].  A summary of the results with ALF ON and ALF OFF is provided below.  Detailed results can be found in the attached Excel files.

## ALF ON

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 101% | 99% |
| Class A2 | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.00% | -0.02% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.01% | 101% | 96% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.08% | 0.05% | 103% | 99% |
| Class C | -0.02% | 0.00% | -0.12% | 102% | 101% |
| Class E | 0.01% | 0.26% | -0.05% | 100% | 98% |
| Overall | -0.01% | 0.10% | -0.03% | 102% | 100% |
| Class D | -0.02% | -0.01% | -0.08% | 101% | 101% |
| Class F | -0.02% | 0.23% | 0.25% | 102% | 100% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.03% | 97% | 98% |
| Class C | 0.01% | -0.04% | 0.12% | 99% | 100% |
| Class E | 0.00% | 0.05% | -0.02% | 100% | 99% |
| Overall | 0.00% | 0.02% | 0.05% | 99% | 99% |
| Class D | 0.02% | -0.01% | 0.04% | 98% | 99% |
| Class F | -0.03% | -0.25% | -0.22% | 100% | 99% |

## ALF OFF

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% |  |  |
| Class A2 | 0.00% | -0.04% | -0.02% |  |  |
| Class B | 0.00% | 0.00% | 0.00% |  |  |
| Class C | 0.00% | 0.00% | 0.00% |  |  |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% |  |  |
| Class D | 0.00% | 0.00% | 0.00% |  |  |
| Class F | 0.00% | 0.00% | 0.00% |  |  |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.06% | -0.06% |  |  |
| Class C | 0.00% | 0.02% | -0.16% |  |  |
| Class E | -0.03% | 0.23% | -0.02% |  |  |
| Overall | -0.01% | 0.09% | -0.08% |  |  |
| Class D | -0.01% | -0.05% | -0.04% |  |  |
| Class F | 0.06% | 0.18% | 0.21% |  |  |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.03% |  |  |
| Class C | -0.02% | -0.05% | -0.10% |  |  |
| Class E | 0.00% | 0.00% | -0.01% |  |  |
| Overall | -0.01% | -0.01% | -0.02% |  |  |
| Class D | 0.00% | 0.05% | -0.01% |  |  |
| Class F | -0.03% | 0.14% | 0.43% |  |  |

# Conclusion

It is proposed to skip the motion continuity-based based deblocking process for internal edges of Triangle mode CU and non-8x8 CU grid aligned edges of an affine mode CU. It is asserted that this addresses two design inconsistency in the deblocking design, and it is recommended to adopt these deblocking modifications into the VVC draft specification.

# References

[1] B. Heng, M. Zhou, “AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode,” JVET-P0043, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019

[2] VVC reference software (VTM) version 6.0, available at: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, "JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

Proposed modifications to VVC draft are highlighted below in yellow

      - 8.8.3.5 Derivation process of boundary filtering strength

Inputs to this process are:

- a picture sample array recPicture,

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered,

- a variable cIdx specifying the colour component of the current coding block,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags.

Output of this process is a two-dimensional (nCbW)x(nCbH) array bS specifying the boundary filtering strength.

The variables xDi, yDj, xN and yN are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(8-1049)

- If edgeType is equal to EDGE_VER,

xDi = ( i * gridSize )		(8-1050)

yDj = cIdx = = 0  ?  ( j  <<  2 )  :  ( j  <<  1 )		(8-1051)

xN is set equal to Max( 0, ( nCbW / gridSize ) − 1 )		(8-1052)

yN = cIdx = = 0  ?  ( nCbH / 4 ) − 1  :  ( nCbH / 2 ) − 1	(8-1053)

- Otherwise (edgeType is equal to EDGE_HOR),

xDi = cIdx = = 0  ?  ( i  <<  2 )  :  ( i  <<  1 )		(8-1054)

yDj = ( j * gridSize )		(8-1055)

xN = cIdx = = 0  ?  ( nCbW / 4 ) − 1  :  ( nCbW / 2 ) − 1	(8-1056)

yN = Max( 0, ( nCbH / gridSize ) − 1 )		(8-1057)

For xDi with i = 0..xN and yDj with j = 0..yN, the following applies:

- If edgeFlags[ xDi ][ yDj ] is equal to 0, the variable bS[ xDi ][ yDj ] is set equal to 0.

- Otherwise, the following applies:

- The sample values p0 and q0 are derived as follows:

    - If edgeType is equal to EDGE_VER, p0 is set equal to recPicture[ xCb + xDi − 1 ][ yCb + yDj ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

    - Otherwise (edgeType is equal to EDGE_HOR), p0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj − 1 ] and q0 is set equal to recPicture[ xCb + xDi ][ yCb + yDj ].

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a coding block with ciip_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 2.

    - Otherwise, if the block edge is also a transform block edge and the sample p0 or q0 is in a transform block which contains one or more non-zero transform coefficient levels, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the block edge is also a transform block edge, cIdx is greater than 0, and the sample p0 or q0 is in a transform unit with tu_joint_cbcr_residual_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if the prediction mode of the coding subblock containing the sample p0 is different from the prediction mode of the coding subblock containing the sample q0 (i.e. one of the coding subblock is coded in IBC prediction mode and the other is coded in inter prediction mode), bS[ xDi ][ yDj ] is set equal to 1.

    - Otherwise, if ( ( subblock_merge_flag[ xCb ][ yCb ] is equal to 1 and MotionModelIdc[ xCb ][ yCb ] is not equal to 0 ) or ( inter_affine_flag[ xCb ][ yCb ] is equal to 1 ) ), edgeType is equal to EDGE_VER and ( xDi % 8 ) is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if ( ( subblock_merge_flag[ xCb ][ yCb ] is equal to 1 and MotionModelIdc[ xCb ][ yCb ] is not equal to 0 ) or ( inter_affine_flag[ xCb ][ yCb ] is equal to 1 ) ), edgeType is equal to EDGE_HOR, and ( yDi % 8 ) is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, edgeType is equal to EDGE_VER, and xDi is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if MergeTriangleFlag[ xCb ][ yCb ] is equal to 1, edgeType is equal to EDGE_HOR, and yDj is not equal to 0, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is equal to 0 and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

      - For the prediction of the coding subblock containing the sample p0 different reference pictures or a different number of motion vectors are used than for the prediction of the coding subblock containing the sample q0.

NOTE 1 – The determination of whether the reference pictures used for the two coding sublocks are the same or different is based only on which pictures are referenced, without regard to whether a prediction is formed using an index into reference picture list 0 or an index into reference picture list 1, and also without regard to whether the index position within a reference picture list is different.

NOTE 2 – The number of motion vectors that are used for the prediction of a coding subblock with top-left sample covering ( xSb, ySb ), is equal to PredFlagL0[ xSb ][ ySb ] + PredFlagL1[ xSb ][ ySb ].

      - One motion vector is used to predict the coding subblock containing the sample p0 and one motion vector is used to predict the coding subblock containing the sample q0, and the absolute difference between the horizontal or vertical component of the motion vectors used is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors and two different reference pictures are used to predict the coding subblock containing the sample p0, two motion vectors for the same two reference pictures are used to predict the coding subblock containing the sample q0 and the absolute difference between the horizontal or vertical component of the two motion vectors used in the prediction of the two coding subblocks for the same reference picture is greater than or equal to 8 in units of 1/16 luma samples.

      - Two motion vectors for the same reference picture are used to predict the coding subblock containing the sample p0, two motion vectors for the same reference picture are used to predict the coding subblock containing the sample q0 and both of the following conditions are true:

        - The absolute difference between the horizontal or vertical component of list 0 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

        - The absolute difference between the horizontal or vertical component of list 0 motion vector used in the prediction of the coding subblock containing the sample p0 and the list 1 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples, or the absolute difference between the horizontal or vertical component of the list 1 motion vector used in the prediction of the coding subblock containing the sample p0 and list 0 motion vector used in the prediction of the coding subblock containing the sample q0 is greater than or equal to 8 in units of 1/16 luma samples.

    - Otherwise, the variable bS[ xDi ][ yDj ] is set equal to 0.