| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0167-v3 |
| --- | --- |

| Title: | CE7-related: Improved coding of user defined quantization matrices |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, an improved predictive coding of user defined quantization matrices (QMs) is proposed. The current design in VVC Draft 6 allows either referencing one of the previously coded QMs having the same sizeId as the current QM or DPCM coding of elements within the current QM. This contribution suggests simple changes on top of the current design to improve coding efficiency as follows.

- [Inter QM extended copy mode]
Allow referencing a previously coded QM with the same base size as the current QM, where the reference QM sizeId can be the same as or smaller than the current QM sizeId.

- [Inter QM prediction mode]
Allow coding element-to-element differences between the current QM and the reference QM.

- [Intra QM prediction mode]
When the previous two modes are impossible or inefficient, the original DPCM coding of elements within the current QM can still be applied.

The number of bits required for each of the three methods is computed, and the method requiring minimum number of bits will be chosen for coding of the current QM. The proposed method was implemented on top of VTM6.1 and tested on multiple scaling list sets, and the results are provided in the last section of this document. On average 43% bits for QMs are saved.

# Introduction

VVC, having a larger number of block sizes compared to HEVC, requires coding of more QMs. In VVC Draft 6, when user defined QMs are specified, current coding procedure is limited to the two methods:

- [Inter QM copy mode] reference one of the previously decoded QMs if it is equal to the current to-be-coded QM and has the same sizeId.

- [Intra QM prediction mode] apply DPCM coding of elements within the current to-be-coded QM.

# Proposed method

This contribution targets at improving coding efficiency for user defined QMs while keeping the design simple for understanding. It is proposed to:

- [Inter QM extended copy mode] allow referencing any of the previously coded QMs having the same base size as the current QM, while reference QM sizeId can be smaller than or equal to the current QM sizeId, and if all elements of the reference QM are equal to those of the current QM

- [Inter QM prediction mode] allow referencing a previously coded QM (using the same reference QM method as inter QM extended copy mode) and apply DPCM coding to element-to-element differences between the current QM and the reference QM

- [Intra QM prediction mode] keep the original DPCM coding of elements within the current QM in cases when both previous methods are either impossible or inefficient.

These aspects are described in more details in the following subsections below.

The number of bits required for each of the three methods is computed, and the method requiring minimum number of bits will be chosen for coding of the current QM.

## Inter QM extended copy mode

Table 1 shows an example of selected QMs for block sizes 8x8 and 16x16. As it can be seen, all three QMs are equal. In case of VVC Draft 6 design, QM for INTRA8X8_CHROMAU can be decoded by copying a previously decoded QM for INTRA8X8_LUMA. However, QM for INTRA16X16_LUMA cannot be decoded in the same way, since it has a different sizeId from that of 8x8, and prediction between QMs with different sizeIds is not allowed in VVC Draft 6.

Table 1. Selected QMs for block sizes 8x8 and 16x16 (HEVC default)

| INTRA8X8_LUMA = | INTRA8X8_CHROMAU = | INTRA16X16_LUMA = |
| --- | --- | --- |
| 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 | 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 | 16, 16, 16, 16, 17, 18, 21, 24 16, 16, 16, 16, 17, 19, 22, 25 16, 16, 17, 18, 20, 22, 25, 29 16, 16, 18, 21, 24, 27, 31, 36 17, 17, 20, 24, 30, 35, 41, 47 18, 19, 22, 27, 35, 44, 54, 65 21, 22, 25, 31, 41, 54, 70, 88 24, 25, 29, 36, 47, 65, 88, 115 |

In this proposal, it is suggested to permit referencing QMs with the same base size, while reference QM sizeId can be smaller than or equal to the current QM sizeId. This way, INTRA8X8_LUMA can be used for decoding QM for INTRA16X16_LUMA, which allows reducing bits needed for coding user defined QMs, especially for cases when same base QMs are used across different block sizes.

## Inter QM prediction mode

Inter QM prediction mode implies using any of the previously coded QM for decoding of the current QM. For that, a flag is sent to indicate whether inter QM prediction mode is enabled. Then, the reference sizeId and matrixId specified in Tables 2 and 3 are coded together with the delta QM. The delta QM is equal to the difference between the current QM and the reference QM. DPCM is applied for coding delta QM.

Table 2. Size identifier (VTM6.1)

| Luma | Chroma | sizeId |
| --- | --- | --- |
| - | - | 0 |
| - | 2x2 | 1 |
| 4x4 | 4x4 | 2 |
| 8x8 | 8x8 | 3 |
| 16x16 | 16x16 | 4 |
| 32x32 | 32x32 | 5 |
| 64x64 | - | 6 |

Table3. Matrix type identifier (VTM6.1)

| CuPredMode | cIdx (Colour component) | matrixId |
| --- | --- | --- |
| MODE_INTRA | 0 (Y) | 0 |
| MODE_INTRA | 1 (Cb) | 1 |
| MODE_INTRA | 2 (Cr) | 2 |
| MODE_INTER | 0 (Y) | 3 |
| MODE_INTER | 1 (Cb) | 4 |
| MODE_INTER | 2 (Cr) | 5 |

In cases when the QM used for prediction does not have a separate DC value but the current QM has a separate DC value, the top-left value of the reference QM is used to predict the separate DC value of the current QM. As shown in Table 4, INTER8X8_CHROMAV can still be used for predicting of INTRA16X16_LUMA with INTRA16X16_LUMA_DC=2, and the prediction for this value is the top-left element of INTER8X8_CHROMAV which is equal to 5.

Table 4. Selected QMs for block sizes 8x8 and 16x16 (asymmetric7)

| INTER8X8_CHROMAV = | INTRA16X16_LUMA = |
| --- | --- |
| 5, 5, 14, 32, 44, 52, 59, 62 5, 10, 17, 34, 50, 61, 68, 70 8, 12, 25, 37, 57, 70, 84, 91 10, 17, 34, 52, 73, 84, 95, 104 17, 26, 52, 66, 88, 98, 113, 116 23, 32, 57, 73, 97, 107, 118, 122 26, 35, 62, 84, 102, 118, 124, 125 30, 35, 64, 89, 107, 120, 124, 125 | 2, 2, 5, 11, 19, 21, 24, 24 2, 3, 7, 16, 25, 29, 31, 32 2, 4, 8, 21, 39, 44, 47, 47 3, 6, 11, 26, 53, 61, 63, 65 4, 9, 17, 34, 62, 74, 77, 77 6, 13, 22, 47, 71, 79, 84, 86 9, 16, 25, 53, 70, 83, 87, 88 9, 17, 30, 56, 76, 84, 88, 90 |

Although current default QMs are all flat (with the default value 16), the default QMs can also be used for predicting current matrices. No additional flags are needed for this. However, since the default QMs are all flat and do not bring any visual improvements, it is still suggested to remove them and support only user defined QMs.

## Intra QM prediction mode

When both previous methods are either impossible or inefficient, the original DPCM coding of elements within the current QM can be applied. This mode already exists in VVC Draft 6.

The final decision is made in the end: the number of bits required for each of the three methods is computed and the method requiring minimum number of bits will be chosen for coding of the current QM.

## Additional optimizations

Additional optimisations are made in order to remove some redundant signalling:

- The sizeId of the reference QM is signalled only for QMs with sizeId >3

- For 2x2 IntraChromaU, 4x4 IntraLuma, 8x8 IntraLuma, reference QM matrixId is not signalled, and inferred to 0. It should be noted that referencing the default QMs is still possible in this case. The number of bits required in this case will be compared versus the number of bits after DPCM coding, and the method requiring minimum number of bits will be chosen.

- Delta values are signalled for sizeId and matrixId when inter QM extended copy mode or inter QM prediction mode is used.

- If sps_max_luma_transform_size_64_flag equal to 0 (i.e., maximum transform size in luma samples is equal to 32), no QM for 64x64 has to be decoded.

# Coding efficiency results

The proposed method is implemented on top of VTM6.1 [1]. The QM sets used for testing of the coding efficiency were taken from JVET-O0223 [2] (VVC test set).

Table 5 reports the number of bits needed to encode scaling_list_data using three different methods: VTM6.1, VTM6.1 with inter QM extended copy mode, and the entire proposed method in this proposal.

Table 5.  Results of different QM coding methods for different QM sets

| Test name | VTM6.1 | VTM6.1 + ext. copy mode | VTM6.1 + ext. copy mode vs. VTM6.1 | Proposed | Proposed vs. VTM6.1 |
| --- | --- | --- | --- | --- | --- |
| JPEG-like | 4748 | 2459 | -2289(-48%) | 1841 | -2907(-61%) |
| MPEG2-like | 911 | 592 | -319(-35%) | 473 | -438(-48%) |
| h264-like | 1224 | 802 | -422(-34%) | 663 | -561(-46%) |
| DTT-like | 2940 | 1896 | -1044(-36%) | 1485 | -1455(-49%) |
| HEVC | 1504 | 530 | -974(-65%) | 533 | -971(-65%) |
| symmetry1 | 2658 | 1584 | -1074(-40%) | 1117 | -1541(-58%) |
| symmetry2 | 1620 | 1292 | -328(-20%) | 1119 | -501(-31%) |
| symmetry3 | 3220 | 2560 | -660(-20%) | 2211 | -1009(-31%) |
| symmetry4 | 3146 | 2418 | -728(-23%) | 2352 | -794(-25%) |
| symmetry7 | 4264 | 3252 | -1012(-24%) | 2679 | -1585(-37%) |
| asymmetry1 | 3078 | 1806 | -1272(-41%) | 1234 | -1844(-60%) |
| asymmetry2 | 1778 | 1404 | -374(-21%) | 1209 | -569(-32%) |
| asymmetry3 | 3624 | 2842 | -782(-22%) | 2415 | -1209(-33%) |
| asymmetry4 | 4344 | 3344 | -1000(-23%) | 3355 | -989(-23%) |
| asymmetry7 | 5406 | 3316 | -2090(-39%) | 2738 | -2668(-49%) |
| Average | 2964 | 2006 | -957(-33%) | 1694 | -1269(-43%) |

# References

- VTM6.1, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.1

- P. de Lagrange, F. Le Léannec, E. François, and K. Naser, “Non-CE7: Quantization matrices with single identifier and prediction from larger ones,” Document of Joint Video Experts Team, JVET-O0223.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).