| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0187-v2 |
| --- | --- |

| Title: | Comments on HLS of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes comments on high-level syntax of VVC WD.

Following is proposed:

- Proposal 1: A bug-fix is proposed for the for loop which derives the DirectDependentLayerIdx [i][j] for a j-th direct reference layer of the i-th layer.

- Proposal 2: It is proposed to fix the derived variables DirectDependencyLayerIdx[i][j]  to DirectReferenceLayerIdx[i][j]  as it is asserted that these specify the j-th direct reference layer for the  i-th layer and not the j-th dependent layer of the i-the layer.

- Proposal 3: Currently VVC WD is missing a NAL unit type for VPS. It is proposed to define a NAL unit type for VPS.

- Proposal 4: It is proposed to reserve more VCL NAL unit types for IRAP NAL units.

# Introduction

This document proposes comments on high-level syntax of VVC WD [1].  The following is proposed:

- Proposal 1: A bug-fix is proposed for the for loop which derives the DirectDependentLayerIdx [i][j] for a j-th direct reference layer of the i-th layer.

- Proposal 2: It is proposed to fix the derived variables DirectDependencyLayerIdx[i][j]  to DirectReferenceLayerIdx[i][j]  as it is asserted that these specify the j-th direct reference layer for the  i-th layer and not the j-th dependent layer of the i-the layer.

- Proposal 3: Currently VVC WD is missing a NAL unit type for VPS. It is proposed to define a NAL unit type for VPS.

- Proposal 4: It is proposed to reserve more VCL NAL unit types for IRAP NAL units.

# Proposals

## Proposal 1

A bug-fix is proposed for the for loop which derives the DirectDependentLayerIdx [i][j] for a j-th direct reference layer of the i-th layer.

It is asserted that without this modification the value of vps_direct_dependency_flag[ i ][ j ] needs to be signalled when i is equal to j.

The proposed changes are shown highlighted below compared to VVC WD. A few variants are included.

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0.

The variable DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent layer of the i-th layer, is derived as follows:

for( i = 1; i < vps_max_layers_minus1; i− − )
	if( !vps_independent_layer_flag[ i ] )
		for( j = i−1, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectDependentLayerIdx[ i ][ k++ ] = j

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(7-3)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

## Proposal 2

It is proposed to fix the derived variables DirectDependencyLayerIdx[i][j]  to DirectReferenceLayerIdx[i][j]  as it is asserted that these specify the j-th direct reference layer for the  i-th layer and not the j-th dependent layer of the i-the layer.

vps_direct_dependency_flag[ i ][ j ] equal to 0 specifies that the layer with index j is not a direct reference layer for the layer with index i. vps_direct_dependency_flag [ i ][ j ] equal to 1 specifies that the layer with index j is a direct reference layer for the layer with index i. When vps_direct_dependency_flag[ i ][ j ] is not present for i and j in the range of 0 to vps_max_layers_minus1, inclusive, it is inferred to be equal to 0.

The variable DirectReferenceLayerIdx DirectDependentLayerIdx[ i ][ j ], specifying the j-th direct dependent reference layer of the i-th layer, and variable NumDirectReferenceLayers[ i ], specifying the number of  direct reference layers for the i-th layer is are derived as follows:

for( i = 1; i < vps_max_layers_minus1; i− − ) {
	if( !vps_independent_layer_flag[ i ] )
		for( j = i, k = 0; j  >=  0; j− − )		(7-2)
			if( vps_direct_dependency_flag[ i ][ j ] )
				DirectReferenceLayerIdx DirectDependentLayerIdx[ i ][ k++ ] = j
}

NumDirectReferenceLayers[ i ] = (vps_independent_layer_flag[ i ] ? 0:k)

The variable GeneralLayerIdx[ i ], specifying the layer index of the layer with nuh_layer_id equal to vps_layer_id[ i ], is derived as follows:

for( i = 0; i  <=  vps_max_layers_minus1; i++ )		(7-3)
	GeneralLayerIdx[ vps_layer_id[ i ] ] = i

## Proposal 3

VVC WD6 is missing a NAL unit type for video parameter set.

It is proposed to include a VPS NAL unit type with a suitable number. For example:

nal_unit_type specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit as specified in Table 7-1.

NAL units that have nal_unit_type in the range of UNSPEC28..UNSPEC31, inclusive, for which semantics are not specified, shall not affect the decoding process specified in this Specification.

NOTE 1 – NAL unit types in the range of UNSPEC28..UNSPEC31 may be used as determined by the application. No decoding process for these values of nal_unit_type is specified in this Specification. Since different applications might use these NAL unit types for different purposes, particular care must be exercised in the design of encoders that generate NAL units with these nal_unit_type values, and in the design of decoders that interpret the content of NAL units with these nal_unit_type values. This Specification does not define any management for these values. These nal_unit_type values might only be suitable for use in contexts in which "collisions" of usage (i.e., different definitions of the meaning of the NAL unit content for the same nal_unit_type value) are unimportant, or not possible, or are managed – e.g., defined or managed in the controlling application or transport specification, or by controlling the environment in which bitstreams are distributed.

For purposes other than determining the amount of data in the decoding units of the bitstream (as specified in Annex C), decoders shall ignore (remove from the bitstream and discard) the contents of all NAL units that use reserved values of nal_unit_type.

NOTE 2 – This requirement allows future definition of compatible extensions to this Specification.

Table 7-1 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 2 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 3 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 4..7 | RSV_VCL_4.. RSV_VCL_7 | Reserved non-IRAP VCL NAL unit types | VCL |
| 8 9 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 10 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) | VCL |
| 11 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 12 13 | RSV_IRAP_VCL12 RSV_IRAP_VCL13 | Reserved IRAP VCL NAL unit types | VCL |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | AUD_NUT | Access unit delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 20 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 21 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 22, 23 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 24 | DPS_NUT | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 25 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 256..27 | RSV_NVCL25.. RSV_NVCL27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC28.. UNSPEC31 | Unspecified non-VCL NAL unit types | non-VCL |

## Proposal 4

It is proposed to reserve more VCL NAL unit types for IRAP VCL NAL units.

It is asserted that with the use of recent NAL unit types from IRAP range, only two reserved IARP VCL NAL unit types are remaining. It is proposed to assign more reserved VCL NAL units to IRAP as a balance.

It is proposed to make 14, 15 as reserved IRAP VCL NAL units instead of as VCL NAL units.

| 12 13 14 15 | RSV_IRAP_VCL12 RSV_IRAP_VCL13 RSV_VCL14. RSV_VCL15 | Reserved IRAP VCL NAL unit types | VCL |
| --- | --- | --- | --- |
| 14..15 | RSV_VCL14.. RSV_VCL15 | Reserved non-IRAP VCL NAL unit types | VCL |

It is asserted that this keeps 4 reserved IRAP VCL NAL unit types and 4 reserved non-IRAP VCL NAL unit types. Also, with this the VCL NAL unit range is evenly divided between IRAP VCL NAL unit types and non-IRAP VCL NAL unit types.

# Conclusion

This document proposes comments and asserted bug-fixes for VVC WD. It is proposed to adopt the proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019