| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0213 |
| --- | --- |

| Title: | AHG8: External independent layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes an external independent layer feature, which is asserted to be similar to the external base layer feature of HEVC. The proposed specification text changes are summarized as follows:

- The VPS includes vps_independent_layer_internal_flag[ i ] for each independent layer that is used as a reference layer for any other layer.

- When a reference picture list references an inter-layer reference picture with vps_independent_layer_internal_flag[ i ] equal to 0, a decoded reference picture is provided by external means. It is treated as a long-term reference in the decoding process. It is not stored in the DPB.

# Introduction

HEVC includes the external base layer feature, which can be used for hybrid codec scalability.

VVC does not treat nuh_layer_id equal to 0 in any special way but any independent layer can act as the "base" for inter-layer referencing (for layers with a greater nuh_layer_id value). Thus, in the VVC context, a feature similar to HEVC's external base layer feature but applying to any independent layers used as reference for predicting other layers could be called external independent layers.

This contribution proposes the support of external independent layers in VVC.

# Summary of the proposed changes

The following changes are proposed:

- The VPS includes vps_independent_layer_internal_flag[ i ] for each independent layer that is used as a reference layer for any other layer.

- When a reference picture list references an inter-layer reference picture with vps_independent_layer_internal_flag[ i ] equal to 0, a decoded reference picture is provided by external means. It is treated as a long-term reference in the decoding process. It is not stored in the DPB.

The detailed changes are indicated with change marks in the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).