| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0233 |
| --- | --- |

| Title: | AHG12: Signaling of tile and brick information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee (XRis Corp) Dongsan Jun (Kyungnam University) | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr dsjun9643@kyungnam.ac.kr |
| Source: | XRis Corporation, Kyungnam University |  |  |

_____________________________

- Abstract

This contribution proposes to reduce 1 syntax element (brick_height_minus1[i]) for the unification of brick related syntax elements.

In addition, it is proposed to conditionally signal uniform_brick_spacing_flag[i] based on the value of num_brick_row_minus2[i].

Introduction

There are 2 syntax elements to describe the number of brick and brick’s height. When uniform_brick_spacing_flag is equal to 1, brick_height_minus1[i] is used to indicate the the same brick height for all bricks of i-th tile. Otherwise, num_brick_rows_minus2[i] and brick_row_height_minus1[i][j] are used to represent the number of brick and the height of j-th brick of i-th tile.

In this contribution, it is proposed to unify syntax elements regarding the number of brick and brick’s height based on the value of num_brick_row_minus2[i] and the syntax, brick height_minus1[i], is eliminated.

When the number of bricks is equal to 2, at least two syntax elements among  uniform_brick_spacing_flag[i], brick_height_minus1[i], num_brick_rows_minus2[i] or brick_row_height_minus1[i][j] should be signalled in VVC draft6 [1]. If the number of bricks is 2, it is not needed to send any brick information except first brick’s height as shown in Fig. 1. In addition, the brick structure of Fig.1 (c), and (d) could be treat either uniform_brick_spacing_flag(1) or uniform_brick_spacing_flag(0).

- (b)                   (c)                  (d)

Figure 1

As shown in Table 1, for avoiding those ambiguous cases and simplifying the syntax elements, it is proposed to conditionally signal brick_row_height_minus1[ i ][ j ] based on the value of num_brick_row_minus2[i], without brick_height_minus1[i]. In addition, the proposed scheme is only signalled uniform_spacing_flag[i], when num_brick_rows_minus2[i] is greater than 0.

Proposal

As shown in Table 1, for avoiding those ambiguous cases and simplifying the syntax elements, it is proposed to conditionally signal brick_row_height_minus1[ i ][ j ] based on the value of num_brick_row_minus2[i], without brick_height_minus1[i]. The proposed scheme is only signalled uniform_brick_spacing_flag[i], when num_brick_rows_minus2[i] is greater than 0. In case that uniform_brick_spacing_flag[i] is equal to 1, brick_row_height_minus1[ i ][ j ] is inferred as below.

The proposed syntax and semantic changes are shown highlighted below compared with VVC draft6 [1]

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| brick_splitting_present_flag | u(1) |
| if( uniform_tile_spacing_flag && brick_splitting_present_flag ) |  |
| num_tiles_in_pic_minus1 | ue(v) |
| for( i = 0; brick_splitting_present_flag && i <= num_tiles_in_pic_minus1 ; i++ ) { |  |
| if( RowHeight[ i ] > 1 ) |  |
| brick_split_flag[ i ] | u(1) |
| if( brick_split_flag[ i ] ) { |  |
| if( RowHeight[ i ] > 2) |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if( uniform_brick_spacing_flag[ i ] ) |  |
| brick_height_minus1[ i ] | ue(v) |
| else { |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| num_brick_rows_minus2[ i ] | ue(v) |
| if( !num_brick_rows_minus2[i]) |  |
| brick_row_height_minus1[ i ][ 0 ] | ue(v) |
| else{ |  |
| uniform_brick_spacing_flag[ i ] | u(1) |
| if (!uniform_brick_spacing_flag[i]){ |  |
| for( j = 0; j <= num_brick_rows_minus2[ i ]; j++ ) |  |
| brick_row_height_minus1[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

- 6.5.1 CTB raster scanning, tile scanning, brick scanning, and subpicture scanning processes

for ( brickIdx = 0, i = 0; i < NumTilesInPic; i++ ) {
	tileX = i % ( num_tile_columns_minus1 + 1 )
	tileY = i / ( num_tile_columns_minus1 + 1 )
	if( !brick_split_flag[ i ] ) {
		BrickColBd[ brickIdx ] = tileColBd[ tileX ]
		BrickRowBd[ brickIdx ] = tileRowBd[ tileY ]
		BrickWidth[ brickIdx ] = colWidth[ tileX ]
		BrickHeight[ brickIdx ] = RowHeight[ tileY ]	(6-5)
		brickIdx++
	} else {
		if( uniform_brick_spacing_flag[ i ] ) {
			remainingHeightInCtbsY = RowHeight[ tileY ]
			j = 0
			while( remainingHeightInCtbsY > ( brick_row_height_minus1[i][j]+1) ) {
				rowHeight2[ j++ ] = brick_row_height_minus1[i][j]+1
				remainingHeightInCtbsY  −=  ( brick_row_height_minus1[i][j]+1 )
			}
			rowHeight2[ j ] = remainingHeightInCtbsY
			num_brick_rows_minus2[ i ] = j − 1
		} else {
			rowHeight2[ num_brick_rows_minus2[ i ] + 1 ] = RowHeight[ tileY ]
			for( j = 0; j  <=  num_brick_rows_minus2[ i ]; j++ ) {
				rowHeight2[ j ] = brick_row_height_minus1[ i ][ j ]+ 1
				rowHeight2[ num_brick_rows_minus2[ i ] + 1 ]  −=  rowHeight2[ j ]
			}
		}
		for( rowBd2[ 0 ] = 0, j = 0; j  <=  num_brick_rows_minus2[ i ] + 1; j++ )
			rowBd2[ j + 1 ] = rowBd2[ j ] + rowHeight2[ j ]
		for( j = 0; j  <=  num_brick_rows_minus2[ i ] + 1; j++ ) {
			BrickColBd[ brickIdx ] = tileColBd[ tileX ]
			BrickRowBd[ brickIdx ] = tileRowBd[ tileY ] + rowBd2[ j ]
			BrickWidth[ brickIdx ] = colWidth[ tileX ]
			BrickHeight[ brickIdx ] = rowHeight2[ j ]
			brickIdx++
		}
	}
}
NumBricksInPic = brickIdx

7.4.3.4 Picture parameter set RBSP semantics

brick_height_minus1[ i ] plus 1 specifies the height of the brick rows excluding the bottom brick in the i-th tile in units of CTBs when uniform_brick_spacing_flag[ i ] is equal to 1. When present, the value of brick_height_minus1 shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. When not present, the value of brick_height_minus1[ i ] is inferred to be equal to RowHeight[ i ] − 1.

brick_row_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th brick in the i-th tile in units of CTBs .

unifor_tile_spacing_flag is equal to 0. When not present, the value of brick_row_height_minus1[ i ][ j ] is derived as follows

- When num_brick_rows_minus2[i] is greater than 0 and uniform_brick_sapcing_flag is equal to 1, the height of the bricks is derived as follows

- if the brick is not the last brick of i-th tile (j is less than or equal to num_brick_rows_minus2), the height of all bricks except the last brick is derived only once as below

brick_row_height_minus1[i][j] = Ceil(RowHeight[i]/(num_brick_rows_minus2+2)) - 1

- otherwise

brick_row_height_minus1[i][j] = (RowHeight[i] % Ceil(RowHeight[i]/(num_brick_rows_minus2+2)) ) - 1

num_brick_rows_minus2[i] plus 2 specifies the number of bricks partitioning the i-th tile when uniform_brick_spacing_flag[ i ] is equal to 0. When present, the value of num_brick_rows_minus2[ i ] shall be in the range of 0 to RowHeight[ i ] − 2, inclusive. If brick_split_flag[ i ] is equal to 0, the value of num_brick_rows_minus2[ i ] is inferred to be equal to −1. Otherwise, when uniform_brick_spacing_flag[i] is equal to 1, the value of num_brick_rows_minus2[ i ] is inferred as specified in 6.5.1.

References

[1] JVET-O2001-vE, Versatile Video Coding (Draft 6), Gothenburg, July 2019

Patent rights declaration(s)

XRis Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kyungnam University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).