| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0289_r1 |
| --- | --- |

| Title: | Non-CE3: MIP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomohiro Ikai | Tel: Email: | +81 50-5123-6336 yasugi.yukinobu@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a simplification on matrix-based intra prediction (MIP). In VVC Draft 6, the intermediate prediction size of MIP (predMip), 4x4, 4x8, 8x4 or 8x8 can be different from that of the matrix output, 4x4 or 8x8. This difference needs additional processing in matrix calculation. This contribution propose to match the size by change MipSizeId derivation (matrix selection). Experimental results reportedly shows that the proposed method shows average luma BD-rates of 0.01 %, 0.01 %, and -0.04 % for AI, RA, and LDB cases, respectively.

Introduction

Matrix-based intra prediction (MIP) generates intermediate prediction predMip[][] (predW x predH) based on matrix mWeight[][] (predC x predC x inSize) and interpolates/upscales the predMip[][] to the prediction predSamples (nTbW x nTbH) as shown in Fig. 1.

Fig 1: Overview of Matrix-based intra prediction

However, in VVC Draft 6, the size of predMip can be different from that of mWeight (predW != predC or predH != predC). That means additional processing is necessary for matrix operation (multipy-add operation). This inconsistency is not desirable. The cause of this size mismatch is that the matrix selection (MipSize decision) can choose a matrix larger than target block size. i.e. 8x8 matrix (MipSizeId = 2) can be chosen in 16x4 and 4x16 blocks. It seems the current mismatched selection does not show any coding performance or computation benefit. It is proposed to remove that inconsistency by adjusting matrix selection.

Table 1: SizeId classification of input blocks

| sizeId | w=4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| h=4 | 0 | 1 | 2 | - | - |
| 8 | 1 | 1 | 2 | 2 | - |
| 16 | 2 | 2 | 2 | 2 | 2 |
| 32 | - | 2 | 2 | 2 | 2 |
| 64 | - | - | 2 | 2 | 2 |

Proposed method

In this contribution, we propose that size of intermediate prediction predMip[][] always match with mWeight matrix output size (predW = predH = predC), which enables much simpler matrix operation and removes several intermediate calculations.

The proposed change is to derive the MipSizeId (choose matrix size) as follows

MipSizeId = ( width == 4 && height == 4 ) ? 0 : ( width * height <= 64 ) ? 1: 2

Table 2 shows the resulting classification by the proposed method.

Table 2: SizeId classification of input blocks

| sizeId | w=4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| h=4 | 0 | 1 | 1 | - | - |
| 8 | 1 | 1 | 2 | 2 | - |
| 16 | 1 | 2 | 2 | 2 | 2 |
| 32 | - | 2 | 2 | 2 | 2 |
| 64 | - | - | 2 | 2 | 2 |

With the proposed change, both incH and incW become 1. Thus the matrix operation can be simplified as follows.

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> sW ) + pTemp[ 0 ]　	(8-67)

Here, the decimation (incH and incW) can be removed in matrix calculation.

Because predW, predH, predC, mipW and mipH becomes the same constant for each MipSize, WD text can be much simplified.

WD changes

8.4.1	General decoding process for coding units coded in intra prediction mode

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- Otherwise, if both cbWidth and cbHeight are less than or equal to 8cbWidth*cbHeight <= 64, MipSizeId[ x ][ y ] is set equal to 1.

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

or

- The variable MipSizeId[ x ][ y ] for x = xCb..xCb + cbWidth − 1 and  y = yCb..yCb + cbHeight − 1 is derived as follows:

- If both cbWidth and cbHeight are equal to 4, MipSizeId[ x ][ y ] is set equal to 0.

- Otherwise, if either cbWidth or cbHeight is equal to 4, or both cbWidth and cbHeight are less than or equal to 8 is set equal to 1.

- Otherwise, MipSizeId[ x ][ y ] is set equal to 2.

8.4.5.2.1	Matrix-based intra sample prediction

Table 8-4 – Specification of number of prediction modes numModes, boundary size boundarySize, and prediction sizes predW, predH and predC using MipSizeId

| MipSizeId | numModes | boundarySize | predW | predH | predC |
| --- | --- | --- | --- | --- | --- |
| 0 | 35 | 2 | 4 | 4 | 4 |
| 1 | 19 | 4 | 4 | 4 | 4 |
| 2 | 11 | 4 | Min( nTbW, 8 ) | Min( nTbH, 8 ) | 8 |

…

The variables mipW and mipH are derived as follows:

mipW = isTransposed  ?  predH  :  predW		(8-57)

mipH = isTransposed  ?  predW  :  predH		(8-58)

- The matrix-based intra prediction samples predMip[ x ][ y ], with x = 0..mipWpredC − 1, y = 0..mipHpredC − 1 are derived as follows:

oW = ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i )	(8-64)

incW = ( predC > mipW )  ?  2  :  1		(8-65)

incH = ( predC > mipH )  ?  2  :  1		(8-66)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> sW ) + pTemp[ 0 ]　	(8-67)

Experimental results

The proposed method has been implemented based on VTM-6.0 reference software and tested according to the common test condition defined in JVET-N1010 [2].

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.02% | 0.01% | 100% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.02% | -0.05% | 0.06% | 101% | 100% |
| Class E | 0.01% | -0.02% | 0.06% | 101% | 100% |
| Overall | 0.01% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | 0.02% | 0.06% | 100% | 100% |
| Class F | 0.01% | -0.01% | -0.11% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.16% | 100% | 100% |
| Class A2 | 0.03% | 0.06% | 0.10% | 100% | 100% |
| Class B | 0.00% | 0.05% | 0.05% | 100% | 100% |
| Class C | 0.03% | 0.00% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.05% | 100% | 100% |
| Class D | -0.07% | -0.23% | 0.19% | 100% | 100% |
| Class F | 0.01% | -0.02% | -0.02% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 (sharp) |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.13% | 0.15% | 100% | 100% |
| Class C | 0.03% | 0.01% | -0.05% | 100% | 99% |
| Class E | -0.19% | 1.01% | 0.70% | 100% | 100% |
| Overall | -0.04% | 0.31% | 0.22% | 100% | 100% |
| Class D | -0.03% | 0.17% | -0.32% | 100% | 100% |
| Class F | -0.08% | -0.34% | 0.66% | 100% | 100% |

Those experimental results show that the proposed method provides a simplified specification text with negligible BD-rate performance changes.

References

[1] B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, July 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring " JVET common test conditions and software reference configurations for SDR video," JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).