| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0318-v2 |
| --- | --- |

| Title: | Non-CE3: Context reduction for intra_luma_not_planar_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dohyeon Park Yong-Uk Yoon Jihoon Do Jae-Gon Kim (KAU) Jinho Lee Jungwon Kang (ETRI) | Tel: Email: | +82-2-300-0414 dhpark@kau.kr yuyoon@kau.kr jhdo@kau.kr jgkim@kau.ac.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Korea Aerospace University (KAU), ETRI |  |  |

_____________________________

- Abstract

The syntax element of intra_luma_not_planar_flag is coded in regular mode with two context models based on the value of the parsed intra_subpartitions_mode_flag. This contribution proposes a simplified regular coding method of a planar flag to reduce the number of context models with no dependency between ISP flag and planar flag. It also reduces memory of the context for the planar mode flag. The experimental results show 0.00% and 0.00% luma BD-rate performance for AI and RA, respectively.

Introduction

In the current VVC WD6, a unified MPM list for regular intra, MRL, and ISP is introduced with 6 MPMs [1]. In the current design of intra mode coding, the first index of 6 MPM is fixed to planar mode, and the syntax of intra_luma_not_planar_flag is newly adopted to indicate if the planar mode is used or not. The syntax element of intra_luma_not_planar_flag can be parsed when the value of intra_luma_mpm_flag is equal to 1, and the adjacent reference sample line is used. For intra mode coding, intra_luma_mpm_flag is coded in regular mode with one context model, and intra_luma_not_planar_flag is encoded using the regular coding mode with two context models which are selected according to the ISP mode of the current block below:

Table 1. Assignment of ctxInc to planar mode syntax in WD6

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_not_planar_flag[ ][ ] | intra_subpartitions_mode_flag | na | na | na | na | na |

Proposed method

Using 2 context models for planar mode coding may have a minor impact on a coding performance since the different occurrence probability of planar mode depending on ISP and regular is negligible. Moreover, there is a dependency of the ISP mode flag and planar mode flag in the planar mode coding. Thus, the number of context models of intra_luma_not_planar_flag can be reduced to one without any condition for selecting the context model such as intra_luma_mpm_flag. The memory to keep the context which is cut down is also reduced. The proposed method is to code planar mode in regular mode as shown in Table 2. The initial value of the kept context model is the same as that of the regular planar mode in WD6.

Table 2. Assignment of ctxInc to planar mode syntax in the proposed method

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_not_planar_flag[ ][ ] | 0 | na | na | na | na | na |

Experimental results

The proposed method is implemented on top of VTM6.0 and evaluated with VTM CTC configurations [2]. Table 3 shows the results for All Intra (AI) and Random Access (RA), respectively. The experimental results reportedly show 0.00%/-0.01%/-0.01% BD-rate changes for Y/U/V in AI, and 0.00%/0.01%/0.02% BD-rate changes for Y/U/V in RA, respectively.

Table 3. Experimental results of the proposed method over VTM6.0

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.02% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.01% | -0.01% | -0.02% | 101% | 100% |
| Class C | 0.00% | -0.06% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | -0.04% | 99% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.13% | 0.00% | 99% | 100% |
| Class F | -0.02% | -0.06% | -0.18% | 100% | 100% |
|  | Random access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.13% | -0.03% | 100% | 100% |
| Class A2 | 0.03% | 0.09% | 0.01% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Class C | 0.00% | 0.04% | 0.06% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Class D | -0.04% | -0.45% | -0.07% | 100% | 100% |
| Class F | 0.00% | -0.10% | 0.14% | 100% | 100% |

Draft text (based on JVET-O2001-vE)

| Table 9-82 – Assignment of ctxInc to syntax elements with context coded bins |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Syntax element | binIdx |  |  |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_luma_not_planar_flag[ ][ ] | intra_subpartitions_mode_flag 0 | na | na | na | na | na |

Conclusion

This contribution presents a reduction of the number of context models for planar mode entropy coding. In the proposed method, one context model is used for the planar flag with the existing initial value used for CABAC coding of the planar flag in regular mode. It is noted that the proposed modification is simpler regular coding method than the current design of intra_luma_not_planar_flag. As experimental results, it provides 0.00% and 0.00% BD-rate for AI and RA, respectively.

Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” JVET-O2001, July 2019.

- F. Bossen, J. Boyce, et al., “JVET common test conditions and software reference configurations for SDR video,” JVET document, JVET-N1010, May 2019.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).