| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0331 |
| --- | --- |

| Title: | NON-CE5 : CCALF design simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Taquet Patrice Onno Christophe Gisquet Guillaume Laroche Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 patrice.onno@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the design of the Cross Component Adaptive Loop Filter (CC-ALF) that is studied in CE5-2. There are two proposed simplifications. The first one is to use a single block size for the signaling of the activation map of CC-ALF which is the CTB size (i.e. maximum one flag is coded per Chroma CTB). The second proposed simplification is to remove the temporal mechanism that is used in addition to the APS.

Results are reported when enabling each simplification separately and both together.

The average Y/Cb/Cr BDR gains when using one flag per CTB over VTM-6.0 are 0.10%/-7.35%/-5.80% for the AI configuration, 0.15%/-10.45%/-9.84% for the RA configuration, 0.13%/-15.18%/-11.27% for the Low Delay B configuration, and 0.24%/-15.30%/-11.12% for the Low Delay P configuration.

The average Y/Cb/Cr BDR gains when disabling the temporal mechanism of CE5-2 over VTM-6.0 are 0.13%/-7.58%/-6.01% for the AI configuration, 0.16%/-10.67%/-10.03% for the RA configuration, 0.15%/-15.71%/-11.75% for the Low Delay B configuration, and 0.19%/-15.94%/-11.43% for the Low Delay P configuration.

The average Y/Cb/Cr BDR gains when using one flag per CTB and when disabling the temporal mechanism of CE5-2 over VTM-6.0 are 0.10%/-7.35%/-5.81% for the AI configuration, 0.15%/-10.44%/-9.85% for the RA configuration, 0.14%/-15.22%-11.12% for the Low Delay B configuration, and 0.20%/-15.41%/-10.86% for the Low Delay P configuration.

# Introduction

During the 14th JVET Meeting [1], Sharp has proposed a new Cross-Component Adaptive Loop Filter (CCALF) for Chroma in JVET-O0636 [4], which has been retained for study in CE5 [5][6].

This contribution provides results to be compared to CE5-2.1 test, where CCALF is used as an in-loop filter. Similar conclusion should be obtained when simplifying CCALF as a post-processing filter (CE5-2.2 test).

In JVET-O0636, the filter can be enabled or disabled on a block basis. The blocks size is transmitted in the slice header, and the activation map for the slice, and for the given block size, is encoded at the beginning of the first CTU.

In this contribution, it is recommended to use a fixed block size, which is the size of the Chroma CTB. It is shown by the obtained results (see section 6) that using that fixed block size does not really change CCALF coding gains. Furthermore, it enables unifying the coding of the activation map with existing signaling for ALF CTB flag: one flag is coded per Chroma component at the beginning of each CTU (if CCALF is enabled on that component).

In JVET-O0636, a temporal mechanism was also used to enable not always encoding filter coefficients, or using a previous APS and instead reuse the latest filter coefficients encoded for a previous frame at same temporal level.

In this contribution, it is suggested to remove this temporal mechanism, as it does not provide significant coding gains, as shown by the obtained results (see section 6).

# Experiments

## Experiment 1: CC-ALF activation map signalling with fixed block size

In this experiment the maximum and the minimum size of the activation map is set equal to the CTB size. This enable to have a single activation flag per CTB instead of having a specific map for CC-ALF.

## Experiment 2: Removal of temporal prediction for CC-ALF filters

The temporal mechanism proposed in JVET-O0636 is simply removed and the filter coefficients of CC-ALF is always coded for each frame.

## Experiment 3: Combination of Experiment 1 and Experiment 2

# Results over VTM6.0

The proposed configurations have been implemented on top of VTM-6.0 software, by modifying the implementation of CE5-2.1 Test [5]. Results of each configuration is reported using JVET common test conditions [3]. To illustrate the equivalent Luma gain, the YUV BDR measure in the results below has been computed with the following weights: YUV = (14*Y +U +V)/16.

## Experiment 1: CC-ALF activation map signalling at CTB level

## Experiment 2: Removal of the temporal filters

## Experiment 3: Combination of both

# Results over CE5-2.1 results

## Experiment 1: CC-ALF activation map signalling at CTB level

## Experiment 2: Removal of the temporal filters

# Experiment 3: Combination of both

# Conclusion

It is suggested to use a fixed block size for CC-ALF activation map, to unify the signaling of CC-ALF activation map with ALF CTB flag signaling; and to remove the temporal mechanism of CCALF. As they do not affect CC-ALF coding efficiency, it is recommended that the proposed simplifications be part of CC-ALF if it is adopted.

# References

- Meeting Report of the 15th JVET Meeting (Gothenburg, SE, 3–12 July 2019), JVET-O2000, G. J. Sullivan, J.-R. Ohm

- Versatile Video Coding (Draft 6), Document JVET-O2001_v7, B. Bross, J. Chen, S. Liu

- “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010

- Cross-Component Adaptive Loop Filter for Chroma, JVET-O0636, K. Misra, F. Bossen, A. Segall

- Description of Core Experiment 5 (CE5): Loop filtering, JVET-O2025, C.-Y. Chen, A. Norkin

- CE5-2.1, CE5-2.2: Cross Component Adaptive Loop Filter, JVET-P0080, K. Misra, F. Bossen and A. Segall.

# Syntax simplification

Changes to CE5-2 syntax, with first proposed simplification second proposed simplification

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_cross_component_alf_enabled_flag ) { |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_reuse_temporal_layer_filter | u(1) |
| if (!slice_cross_component_alf_cb_reuse_temporal_layer_filter) |  |
| slice_cross_component_alf_cb_aps_id | u(5) |
| slice_cross_component_alf_cb_log2_control_size_minus4 | ue(v) |
| } |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_ct_reuse_temporal_layer_filter | u(1) |
| if (!slice_cross_component_alf_cr_reuse_temporal_layer_filter) |  |
| slice_cross_component_alf_cr_aps_id | u(5) |
| slice_cross_component_alf_cr_log2_control_size_minus4 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).