| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0371r2 |
| --- | --- |

| Title: | AHG7/AHG15: signalling of corrective values for chroma residual scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Franck Galpin, Karam Naser, Philippe de Lagrange | Tel: Email: | edouard.francois@interdigital.com franck.galpin@interdigital.com karam.naser@interdigital.com philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution relates to LMCS. It proposes to enable signalling in the APS corrective values for the chroma residual scaling. In current VTM, chroma residual scaling values are inferred as the inverse values of the luma mapping slopes. The table of luma mapping slopes is signalled in the APS. In order to provide higher encoder control flexibility, it is proposed to add in the APS a corrective chroma residual scaling parameters table. The corrective parameters are determined at the encoder and signalled in the APS as additive values to the luma mapping slopes. The final chroma residual scaling values are obtained by inverting the corrected luma mapping slope values.

It is reported that the proposed chroma residual scaling control allows much finer tuning of the luma/chroma bitrate balance than using the delta QP control.

As an example, results of the tool-off test of LMCS in SDR, RA configuration, with VTM6.0, and with the proposal, are reported.

- VTM6.0 LMCS off:  	PSNR-Y, -U, -V	1.63%, -1.30%, -1.42%

- Proposal LMCS off:  	PSNR-Y, -U, -V	1.53%, -0.22%, -0.39%

Other test results for SDR and HDR PQ are also reported.

Encoding and decoding runtime impact is said to be negligible.

# Introduction

In current VVC specification, the parameters used in forward Luma Mapping (LM) are signalled in the APS. These parameters are used to derive the forward and inverse luma mapping functions (pivot points and slopes). The Chroma Residual Scaling (CRS) factors are set equal to the inverse luma mapping function slopes.

In this proposal, in order to have a better control of the CRS, it is proposed to signal in the APS, in addition to the luma mapping values, corrective values to be applied in the derivation of the CRS factors.

# Current syntax and process for deriving the CRS factors

In the current VVC specification, the LMCS parameters are signalled in the APS syntax as follows.

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| } |  |

The inverse luma mapping slope values InvScaleCoeff[ i ] are derived as follows (cf eq. 7-57):

for( i = 0; i <= 15; i++ ) {
  	if ( lmcsCW[ i ] = = 0 )	InvScaleCoeff[ i ] = 0
  	else						InvScaleCoeff[ i ] = OrgCW * (1 << 11) / lmcsCW[ i ]
}

where lmcsCW[ i ] is the ith slope of the forward luma mapping function, and orgCW is the value corresponding to a neutral slope value ((1 << BitDepthY ) / 16).

The CRS factors ChromaScaleCoeff[ i ] are computed from InvScaleCoeff[ i ] as follows (cf eq.7-58):

for( i = 0; i <= 15; i++ ) {
      if ( lmcsCW[ i ] = = 0 )		ChromaScaleCoeff[ i ] = (1 << 11)
      else 						ChromaScaleCoeff[ i ] = InvScaleCoeff[ i ]
}

It is observed that the CRS factors ChromaScaleCoeff[ i ] are inferred, without any option to modify them. Offering more flexibility would be beneficial for encoder control. CRS control is in spirit different from quantization control, as it is based on luma brightness (scaling factors are derived from luma samples), while quantization is spatially controlled. These are therefore two different dimensions (brightness, spatial localisation) enabling to control the chroma scaling/quantization. An additional point is the granularity of scaling control using delta QP and CRS factors. Scaling granularity using delta QPs is of the order of ( 2^(1/6) – 1 ), that is, around 0.122. CRS factors granularity is around 1/64 (for a 10-bit signal), that is, 0.016, that is around 7-8 times finer.

# Proposal

The proposal consists in adding in the APS a corrective table enabling to have a finer control of the CRS factors. The syntax and semantics of the added syntax elements are provided below.

| lmcs_data () { | Descriptor |
| --- | --- |
| lmcs_min_bin_idx | ue(v) |
| lmcs_delta_max_bin_idx | ue(v) |
| lmcs_delta_cw_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_cw[ i ] | u(v) |
| if ( lmcs_delta_abs_cw[ i ] ) > 0 ) |  |
| lmcs_delta_sign_cw_flag[ i ] | u(1) |
| } |  |
| lmcs_delta_crs_prec_minus1 | ue(v) |
| for ( i = lmcs_min_bin_idx; i <= LmcsMaxBinIdx; i++ ) { |  |
| lmcs_delta_abs_crs[ i ] | u(v) |
| if ( lmcs_delta_abs_crs[ i ] ) > 0 ) |  |
| lmcs_delta_sign_crs_flag[ i ] | u(1) |
| } |  |
| } |  |

The following semantics text is inserted:

lmcs_delta_crs_prec_minus1 plus 1 specifies the number of bits used for the representation of the syntax lmcs_delta_abs_crs[ i ]. The value of lmcs_delta_crs_prec_minus1 shall be in the range of 0 to BitDepthY − 2, inclusive. When lmcs_delta_crs_prec_minus1 is not present, it is set equal to 0.

lmcs_delta_abs_crs[ i ] specifies the absolute codeword value for the ith correction value of the chroma residual scaling. When lmcs_delta_abs_crs[ i ] is not present, it is set equal to 0.

lmcs_delta_sign_crs_flag[ i ] specifies the sign of the variable lmcsDeltaCrs[ i ]. When lmcs_delta_sign_crs_flag[ i ] is not present, it is set equal to 0.

The variable lmcsDeltaCrs[ i ], with i = lmcs_min_bin_idx..LmcsMaxBinIdx, is derived as follows:

lmcsDeltaCrs[ i ] = ( 1 − 2 * lmcs_delta_sign_crs_flag[ i ] ) * lmcs_delta_abs_crs[ i ]

It is a requirement of bitstream conformance that ( lmcsCW[ i ] + lmcsDeltaCrs[ i ] ) is in the range of (OrgCW>>3) to (OrgCW<<3 − 1), inclusive.

In addition, the following text is modified:

The variable ChromaScaleCoeff[ i ], with i = 0…15, is derived as follows:

if ( lmcsCW[ i ] = = 0 )
        ChromaScaleCoeff[ i ] = (1 << 11)
     else
        ChromaScaleCoeff[ i ] = OrgCW * (1 << 11) / ( lmcsCW[ i ] + lmcsDeltaCrs[ i ] )

# Experiments SDR

For the SDR experiments, the tested encoder tuning consisted in setting the corrective CRS values to a given value +/– CRS_offset as follows:

lmcsDeltaCrs[ i ] = CRS_offset       if lmcsCW[ i ] > OrgCW

lmcsDeltaCrs[ i ] =  –CRS_offset    otherwise

The CRS scaling is increased when CRS_offset is positive, and vice-versa. The table below reports results for different values of the parameter “CRS_offset” (reference VTM6.0).

Results using a chroma delta QP of 1/-1 are also provided (parameters CbQpOffset, CrQpOffset, CbQpOffsetDualTree, CrQpOffsetDualTree set to 1/-1). Those results show that the proposed CRS control provides a much finer granularity on luma/chroma balance than the delta chroma QP control.

|  |  | AI |  |  |  | RA |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CRS_offset | PsnrY | PsnrU | PsnrV |  | PsnrY | PsnrU | PsnrV |
| CRS_offset = 3 | 0.09% | -0.46% | -0.43% |  | 0.10% | -1.09% | -1.04% |
| CRS_offset = 1 | 0.03% | -0.16% | -0.14% |  | 0.03% | -0.36% | -0.34% |
| CRS_offset = -1 | -0.01% | 0.26% | 0.26% |  | -0.03% | 0.46% | 0.42% |
| CRS_offset = -2 | -0.03% | 0.43% | 0.45% |  | -0.06% | 0.84% | 0.65% |
| CRS_offset = -3 | -0.05% | 0.65% | 0.69% |  | -0.09% | 1.28% | 1.09% |
| chr QP set to 1(1) | -2.70% | 26.45% | 25.69% |  | -2.08% | 23.93% | 23.54% |
| chr QP set to -1(-1) | 3.68% | -19.99% | -19.51% |  | 2.69% | -17.75% | -17.50% |

Results for the case CRS_offset = 3 are attached in files “offset3_vs_VTM6.0.xlsm” (Reference: VTM6.0, Test: LMCS with CRS_offset = 3) and “LMCS_off_test_vs_Offset3.xlsm” (tool-off test, Reference: LMCS with CRS_offset = 3, Test: LMCS off).

In the tables below, “LMCS off” results are depicted (RA case), for 2 configurations: current VTM6.0 (results from AHG13), LMCS with proposal using CRS_offset=3. Whilst with the current VTM, LMCS tool-off tests result in 1.63% luma loss but chroma gains (-1.30% U, -1.42% V), correcting the CRS table by an offset CRS_offset set to 3 leads to limiting the chroma gain.

Ref: “LMCS on” – Test: “LMCS off” (RA)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 1.06% | -2.71% | -2.09% | 91% | 101% |
| Class A2 | 2.29% | -4.02% | -3.99% | 89% | 99% |
| Class B | 1.75% | 0.84% | 0.69% | 95% | 98% |
| Class C | 1.41% | -0.89% | -1.62% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 1.63% | -1.30% | -1.42% | 94% | 100% |
| Class D | 0.96% | -0.64% | -1.05% | 98% | 105% |
| Class F | 1.29% | 0.17% | -0.07% | 100% | 97% |

Ref: “LMCS with CRS_offset 3” – Test: “LMCS off” (RA)

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.97% | -1.29% | -0.83% |  |  |
| Class A2 | 2.17% | -3.05% | -2.99% |  |  |
| Class B | 1.69% | 1.39% | 1.25% |  |  |
| Class C | 1.25% | 0.71% | -0.14% |  |  |
| Class E |  |  |  |  |  |
| Overall | 1.53% | -0.22% | -0.39% |  |  |
| Class D | 0.87% | 0.37% | -0.16% |  |  |
| Class F | 1.06% | 1.07% | 0.80% |  |  |

# Additional results for HDR PQ

## Proposed tuning algorithm for HDR PQ

The rationale for applying a chroma residual scaling with factors equal to the luma mapping slopes is that the luma mapping is equivalent to a quantization of the luma signal. In a way, this corresponds to applying a delta QP noted here dQPy to the QP of luma (QPreal = QPy – dQPy). dQPy would be linked to the luma mapping function slope as follows.

dQPy =  – 6*Log2( slope )

In VVC the chroma QP is dependent on the luma QP. Therefore the scaling applying to luma because of the luma mapping can by approximation translate into a similar scaling of the chroma residual, applied in addition to the chroma inverse quantization, using a scaling factor equal to the luma mapping function slope.

Nevertheless, this approach does not consider the impact of the chroma QP mapping. The chroma QP QPc is approximately derived as ChromaQpTable[ QPy ] (we neglect the various clipping applying when deriving the chroma QP; we also for the sake of simplicity consider that one single chroma QP mapping table is used). But considering the impact of luma mapping, QPc should be set equal to ChromaQpTable[ QPreal ]. Or alternatively, the chroma scaling should be set equal to 2^( ChromaQpTable[ QPreal ] – ChromaQpTable[ QPy ] ).

A finer tuning algorithm for deriving the CRS factors is summarized below, which considers the impact of the chroma QP mapping. The correction process applies to each element (of index i) of the CRS table. It is controlled by an input QP value, noted QPref.

invScale = InvScaleCoeff[i] == 0 ? 2048 : InvScaleCoeff[i]

QPreal = QPref + 6 * ( Log2( invScale ) – 11 )

Qp0 = ChromaQpTable[ floor(QPreal) ]

Qp1 = ChromaQpTable[ floor(QPreal) + 1 ]

Frac = QPreal – floor(QPreal)

QPmodif = (1 – Frac) * Qp0 + Frac * Qp1

deltaQP = QPmodif – ChromaQpTable[ QPref ]

scale = 2^( - deltaQP / 6 )

The correction values are deduced as follows:

CRScorrection[i] = round(64*scale) - lmcsCW[i]

ChromaScaleCoeff[i] = OrgCW * (1 << 11) / ( lmcsCW[i] + CRScorrection[i] )

Note: when the value of invScale is close to 2048 (neutral value corresponding to a scaling of 1), the process does not modify the scale / inverse scale values. The correction value is 0. This is the case for the SDR and HDR HLG CTCs (which use scaling factors close to 2048). For HDR PQ, the default scaling values are more aggressive, and the proposed algorithm modifies the scale and inverse scale values.

Experiments are performed in for HDR PQ content. As explained above, the encoder tuning for HDR PQ can be controlled by the parameter QPref.

## HDR PQ setting 1

We first show results corresponding to a first setting, with QPref = sliceQPy for AI and QPref = ( sliceQPy + 3 ) for RA.

Results against VTM6.0 with LMCS on are shown below.

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over | vtm6.0 LMCS on |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -0.59% | 0.05% | 0.05% | -2.81% | -6.19% | 100% | 99% |
|  |  |  | Random | Access |  |  |  |
|  |  |  | Over | vtm6.0 LMCS on |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -0.75% | 0.05% | 0.05% | -3.09% | -7.67% | 101% | 100% |

Results against HM16.18 on are shown below.

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over | HM16.18 |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -45.89% | -24.74% | -24.19% | -57.40% | -60.63% |  |  |
|  |  |  | Random | Access |  |  |  |
|  |  |  | wPSNR |  |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -44.81% | -28.99% | -28.65% | -61.27% | -70.41% |  |  |

## HDR PQ setting 2

Results below show a different trade-off. The second setting corresponds to QPref = ( sliceQPy + 2 ) for both AI and RA cases.

Results against VTM6.0 with LMCS on are shown below.

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over | vtm6.0 LMCS on |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | 2.91% | -0.27% | -0.29% | 6.13% | 16.69% | #NUM! | #NUM! |
|  |  |  | Random | Access |  |  |  |
|  |  |  | wPSNR |  |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | 0.10% | -0.10% | -0.10% | 0.19% | -2.51% | 100% | 101% |

Results against HM16.18 on are shown below.

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over | HM16.18 |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -32.21% | -26.98% | -26.53% | -48.20% | -31.70% |  |  |
|  |  |  | Random | Access |  |  |  |
|  |  |  | Over | HM16.18 |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | -44.29% | -29.10% | -28.77% | -60.43% | -68.54% |  |  |

More generally, it is observed that luma/chroma balance can be controlled by using different settings of QPref, as illustrated in the table below (RA configuration, reference VTM6.0, LMCS on), depicting the results for QPref = sliceQPy + QPoffset.

Results using a chroma delta QP of 1 are also provided (parameters CbQpOffset, CrQpOffset, CbQpOffsetDualTree, CrQpOffsetDualTree set to 1). Those results show that the proposed CRS control provides a much finer granularity on luma/chroma balance than the delta chroma QP control.

| QPoffset | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV |
| --- | --- | --- | --- | --- | --- |
| 0 | 1.26% | -0.19% | -0.19% | 3.34% | 3.22% |
| 1 | 0.98% | -0.09% | -0.09% | 2.11% | 1.57% |
| 2 | 0.10% | -0.10% | -0.10% | 0.19% | -2.51% |
| 3 | -0.75% | 0.05% | 0.05% | -3.09% | -7.67% |
| 4 | -1.43% | 0.09% | 0.09% | -5.11% | -9.27% |
| chr QP set to 1(0) | 13.35% | -1.28% | -1.34% | 40.71% | 89.55% |
| chr QP set to 1(1) | 26.65% | -1.97% | -2.06% | 75.17% | 206.47% |

Using the alternate chroma QP tables from JVET-P0335

More results are provided below using the alternate chroma QP tables from JVET-P0335. Result against VTM6.0 with LMCS are reported below with the setting 2.

|  |  |  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over | vtm6.0 LMCS on |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | 4.60% | -0.20% | -0.21% | 3.58% | 1.62% |  |  |
|  |  |  | Random | Access |  |  |  |
|  |  |  | wPSNR |  |  |  |  |
|  | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV | EncT | DecT |
| Class H1 | 2.44% | -0.14% | -0.11% | 1.57% | -1.06% |  |  |

More results are reported below by using different settings of QPref, (RA configuration, reference VTM6.0, LMCS on), depicting the results for QPref = sliceQPy + offset.

| offset | DE100 | L100 | wPSNRY | wPSNRU | wPSNRV |
| --- | --- | --- | --- | --- | --- |
| 0 | 2.66% | -0.15% | -0.14% | 2.23% | -0.21% |
| 1 | 2.50% | -0.09% | -0.08% | 1.13% | -1.24% |
| 2 | 2.44% | -0.14% | -0.11% | 1.57% | -1.06% |
| 3 | 3.37% | -0.15% | -0.15% | 2.76% | 0.48% |
| 4 | 3.12% | -0.19% | -0.17% | 2.79% | 0.20% |

# Conclusion

It is proposed to add a corrective table for the CRS factors. This enables getting more control flexibility of the CRS tool. The corrective parameters are determined at the encoder and signalled in the APS as additive values to the luma mapping slopes. The chroma residual scaling values are obtained by inverting the corrected luma mapping slope values. This adaptation gives more flexible encoder scaling control, with finer granularity, and in an orthogonal dimension to QP control (brightness vs spatial).

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).