Merged JVET-P0410/469, as adopted in Track A (quantization control), Wednesday morning (oct. 9)

    - SPS syntax

| sequence_parameter_set( ) { | Descriptor |
| --- | --- |
| … |  |
| same_qp_table_for_chroma | u(1) |
| for( i = 0; i < same_qp_table_for_chroma ? 1 : 3; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_outdiff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

    - SPS semantics

[…]

same_qp_table_for_chroma equal to 1 specifies that only one chroma QP mapping table is signalled and this table applies to Cb and Cr residuals as well as joint Cb-Cr residuals. same_qp_table_for_chroma equal to 0 specifies that three chroma QP mapping tables are signalled in the SPS. When same_qp_table_for_chroma is not present in the bistream, the value of same_qp_table_for_chroma is inferred to be equal to 1.

qp_table_start_minus26[ i ] plus 26 specifies the starting luma and chroma QP used to describe the i-th chroma QP mapping table. The value of start_qp_minus26[ i ] shall be in the range of -26 - QpBdOffsetC to 36 inclusive. When start_qp_minus26[ i ] is not present in the bitstream, the value of start_qp_minus26[ i ] is inferred to be equal to 0.

num_points_in_qp_table_minus1[ i ] plus 1 specifies the number of points used to describe the i-th chroma QP mapping table. The value of num_points_in_qp_table_minus1[ i ] shall be in the range of 0 to 62 + QpBdOffsetC, inclusive. When num_points_in_qp_table_minus1[ 0 ] is not present in the bitstream, the value of num_points_in_qp_table_minus1[ 0 ] is inferred to be equal to 0.

delta_qp_in_val_minus1[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_in_val_minus1[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_in_val_minus1[ 0 ][ j ] is inferred to be equal to 0.

delta_qp_diff_val[ i ][ j ] specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. When delta_qp_diff_val[ 0 ][ j ] is not present in the bitstream, the value of delta_qp_diff_val[ 0 ][ j ] is inferred to be equal to 0.

The i-th chroma QP mapping table ChromaQpTable[ i ] for i = 0..same_qp_table_for_chroma ? 0 : 2 is derived as follows:

qpInVal[ i ][ 0 ] = −QpBdOffsetC + qp_table_start_minus26[ i ] + 26
qpOutVal[ i ][ 0 ] = −QpBdOffsetCqpInVal[ i ][ 0 ] + delta_qp_diff_val[ i ][ 0 ]
for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j + 1 ] = qpInVal[ i ][ j − 1 ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j + 1 ] = qpOutVal[ i ][ j − 1 ] + (delta_qp_in_val_minus1[ i ][ j ] ^ delta_qp_diff_val[ i ][ j ] )
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(7-31)
for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j + 1 ] + 1 ) >> 1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k <= qpInVal[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][ j + 1 ] − qpOutVal[ i ][ j ] ) * m + sh ) / ( delta_qp_in_val_minus1[ i ][j + 1] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] + 1 ] + 1; k <= 63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

When same_qp_table_for_chroma is equal to 1, ChromaQpTable[ 1 ][ k ] and ChromaQpTable[ 2 ][ k ] are set equal to ChromaQpTable[ 0 ][ k ] for k = −QpBdOffsetC..63.

It is a requirement of bitstream conformance that the values of qpInVal[ i ][ j ] and qpOutVal[ i ][ j ] shall be in the range of −QpBdOffsetC to 63, inclusive for i = 0..same_qp_table_for_chroma ? 0 : 2 and j = 0..num_points_in_qp_table_minus1[ i ] + 1.

[…]