| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0447_r1 |
| --- | --- |

| Title: | CE7-related: Rice parameter range extension in residual coding with transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Xin Zhao, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

When VTM-6.0 is encoded at low QPs with QP in [22, 27, 32, 37], experimental results show 0.30% AI, 0.38% RA, 0.03% LB luma BD-rate for Class F, and 0.46% AI, 0.49% RA, 0.41% LB luma BD-rate for Class TGM. This contribution reports the results of extending the range of Rice parameter for residual coding in transform skip mode to improve the coding efficiency at low QP values and at 1.75 bins per coefficient.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37], experimental results show 0.00% AI, -0.01% RA, 0.01% LB luma BD-rate for Class F, and 0.00% AI, -0.01% RA, -0.02% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17], experimental results show -0.14% AI, -0.12% RA, -0.15% LB luma BD-rate for Class F, and -0.43% AI, -0.72% RA, -0.30% LB luma BD-rate for Class TGM. Coding gain is observed at low QPs.

Compared to VTM-6.0 at standard QPs with QP in [22, 27, 32, 37] with the proposed at 1.75 bins/coefficient, experimental results show 0.10% AI, 0.07% RA, 0.13% LB luma BD-rate for Class F, and 0.26% AI, 0.21% RA, -0.05% LB luma BD-rate for Class TGM. The coding loss is similar to VTM-6.0 with 1.75 bins/coefficient. In VTM-6.0 with 1.75 bins/coefficient, experimental results show 0.11% AI, 0.07% RA, 0.05% LB luma BD-rate for Class F, and 0.26% AI, 0.20% RA, -0.02% LB luma BD-rate for Class TGM.

Compared to VTM-6.0 at low QPs with QP in [2, 7, 12, 17] with the proposed at 1.75 bins/coefficient, experimental results show 0.17% AI, 0.16% RA, -0.01% LB luma BD-rate for Class F, and 0.03% AI, -0.11% RA, 0.19% LB luma BD-rate for Class TGM. The coding loss is reduced from VTM-6.0 with 1.75 bins/coefficient. In VTM-6.0 with 1.75 bins/coefficient, experimental results show 0.30% AI, 0.38% RA, 0.03% LB luma BD-rate for Class F, and 0.46% AI, 0.49% RA, 0.41% LB luma BD-rate for Class TGM.

This revision is updated to report all low QP test results.

# Introduction

This contribution reports the results of extending the range of Rice parameter for residual coding in transform skip mode to improve the coding efficiency at 1.75 bins per coefficient.

In VVC Draft 6 [1], when transform skip is activated, it first calculates a local sum of a maximum of two neighboring absolute levels to produce locSumAbs. Then locSumAbs is clipped to between 0 and 31 as the followings

locSumAbs = 0
if( xC > 0 )
	locSumAbs += AbsLevel[ xC − 1 ][ yC ]
if( yC > 0 )
	locSumAbs += AbsLevel[ xC ][ yC − 1 ]

which is used as input to determine the rice parameter cRiceParam as in the following code:

.

const uint32_t auiGoRicePars[32] =

{

0, 0, 0, 0,

0, 0, 0, 0, 0, 0,

0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 2, 2, 2, 2, 2, 2, 2

};

return auiGoRicePars[ std::min(locSumAbs, 31) ];

Figure 1: Rice parameter table for transform skip in VTM-6.0.

# Rice parameter range extension

The contribution reports the results of removing the table auiGoRicePars[] in Figure 1 and replaceing it by a table with a larger range, i.e. the Rice parameter table for residual coding without transform skip in Figure 2.

Figure 2: Extended range rice parameter table for transform skip in JVET-O0613.

return g_auiGoRiceParsCoeff[ std::min( (locSumAbs>>1)+1, 31)];

# Results

The testing condition of the proposed follows the CE7 [2] testing conditions using standard and low QP configurations as defined in CE7 [2]. In particular, the anchor results are encoded at 2 bins per coefficient in transform skip mode. The proposed is tested at both 2 and 1.75 bins per coefficient in transform skip mode.

As reference, the results of VTM-6.0 [3] at 2 bins per coefficient is compared with VTM-6.0 at 1.75 bins per coefficient in Table 3 for standard QPs and Table 4 for low QPs to show the coding losses at 1.75 bins per coefficient.

A summary of all results for Class F and Class TGM is in the following Table. As shown in the following table, the coding loss at 1.75 bins/coefficient is reduced from VTM-6.0 encoding at 1.75 bins/coefficient at low QPs.

Table 3: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with standard QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 98% |
| Class E | 0.00% | 0.10% | -0.01% | 99% | 95% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 98% |
| Class D | 0.01% | 0.00% | -0.01% | 99% | 96% |
| Class F | 0.11% | 0.10% | 0.15% | 99% | 100% |
| Class TGM | 0.26% | 0.22% | 0.23% | 97% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.03% | 99% | 99% |
| Class A2 | 0.02% | 0.14% | 0.09% | 99% | 99% |
| Class B | 0.01% | 0.10% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.03% | 99% | 99% |
| Class D | -0.03% | -0.48% | 0.02% | 100% | 99% |
| Class F | 0.07% | 0.10% | 0.17% | 99% | 99% |
| Class TGM | 0.20% | 0.14% | 0.13% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.04% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 99% |
| Class E | -0.09% | 0.46% | 0.57% | 100% | 99% |
| Overall | -0.03% | 0.15% | 0.10% | 99% | 99% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 98% |
| Class F | 0.05% | 0.29% | 0.62% | 99% | 99% |
| Class TGM | -0.02% | -0.03% | -0.03% | 100% | 102% |

Table 4: A comparison of VTM-6.0 at 2 and 1.75 bins per coefficient with low QP values.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.01% | 100% | 98% |
| Class E | 0.05% | 0.00% | 0.02% | 99% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.00% | -0.01% | 0.01% | 99% | 98% |
| Class F | 0.30% | 0.14% | 0.14% | 98% | 99% |
| Class TGM | 0.46% | 0.42% | 0.41% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.00% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.01% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | 0.04% | 99% | 98% |
| Class F | 0.38% | 0.00% | 0.12% | 99% | 99% |
| Class TGM | 0.49% | 0.40% | 0.40% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class E | 0.03% | -0.02% | 0.02% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | -0.02% | 0.03% | -0.02% | 99% | 98% |
| Class F | 0.03% | 0.09% | 0.05% | 99% | 99% |
| Class TGM | 0.41% | 0.32% | 0.42% | 99% | 96% |

## Standard QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class F | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class TGM | 0.00% | 0.00% | -0.01% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.00% | 0.01% | 99% | 99% |
| Class F | -0.01% | -0.04% | 0.01% | 99% | 99% |
| Class TGM | -0.01% | -0.02% | -0.02% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | -0.03% | 99% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 99% | 97% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.01% | -0.01% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class F | 0.01% | -0.04% | 0.13% | 99% | 98% |
| Class TGM | -0.02% | -0.01% | -0.05% | 100% | 102% |

## Low QP results at 2 bins per coefficient

In this result, the anchor and the test are encoded at 2 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class B | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.01% | -0.02% | -0.02% | 99% | 98% |
| Overall | 0.00% | -0.01% | 0.00% | 99% | 99% |
| Class D | 0.00% | 0.01% | 0.01% | 99% | 99% |
| Class F | -0.14% | -0.17% | -0.17% | 98% | 99% |
| Class TGM | -0.43% | -0.34% | -0.34% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.01% | 0.03% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class D | 0.02% | 0.03% | -0.03% | 99% | 97% |
| Class F | -0.12% | -0.18% | -0.15% | 99% | 99% |
| Class TGM | -0.72% | -0.32% | -0.29% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.02% | 99% | 99% |
| Class C | 0.00% | 0.00% | -0.03% | 99% | 98% |
| Class E | 0.01% | -0.02% | 0.02% | 99% | 101% |
| Overall | 0.00% | 0.00% | -0.01% | 99% | 99% |
| Class D | 0.01% | 0.02% | -0.01% | 99% | 97% |
| Class F | -0.15% | -0.13% | -0.18% | 99% | 98% |
| Class TGM | -0.30% | -0.24% | -0.14% | 99% | 96% |

## Standard QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 100% | 99% |
| Class A2 | 0.00% | -0.02% | 0.03% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 98% |
| Class C | 0.02% | -0.03% | 0.04% | 100% | 98% |
| Class E | 0.00% | 0.10% | -0.01% | 99% | 97% |
| Overall | 0.00% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.01% | 0.00% | -0.01% | 99% | 97% |
| Class F | 0.10% | 0.10% | 0.16% | 99% | 99% |
| Class TGM | 0.26% | 0.23% | 0.23% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.04% | 99% | 99% |
| Class A2 | 0.02% | 0.14% | 0.09% | 99% | 99% |
| Class B | 0.01% | 0.10% | 0.01% | 99% | 99% |
| Class C | 0.00% | 0.06% | 0.03% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.08% | 0.04% | 99% | 99% |
| Class D | -0.03% | -0.48% | 0.02% | 100% | 98% |
| Class F | 0.07% | 0.12% | 0.15% | 99% | 98% |
| Class TGM | 0.21% | 0.14% | 0.13% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.11% | 0.03% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.20% | 99% | 95% |
| Class E | -0.09% | 0.46% | 0.57% | 99% | 98% |
| Overall | -0.03% | 0.15% | 0.09% | 99% | 98% |
| Class D | 0.03% | 0.31% | 0.80% | 100% | 98% |
| Class F | 0.13% | 0.03% | 0.56% | 99% | 96% |
| Class TGM | -0.05% | -0.07% | -0.03% | 100% | 101% |

## Low QP results at 1.75 bins per coefficient

In this result, the anchor is encoded at 2 bins per coefficient and the test is encoded at 1.75 bins per coefficient in transform skip mode.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.05% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.01% | -0.01% | 0.00% | 99% | 99% |
| Class C | 0.00% | 0.02% | 0.01% | 100% | 99% |
| Class E | 0.05% | 0.02% | 0.01% | 100% | 98% |
| Overall | 0.02% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.01% | -0.03% | -0.02% | 100% | 98% |
| Class F | 0.17% | 0.02% | 0.01% | 98% | 98% |
| Class TGM | 0.03% | 0.08% | 0.09% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.01% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Class C | 0.00% | -0.01% | -0.03% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.03% | -0.02% | 100% | 97% |
| Class F | 0.16% | -0.07% | -0.01% | 99% | 99% |
| Class TGM | -0.11% | 0.08% | 0.11% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | 0.00% | 99% | 99% |
| Class C | 0.01% | -0.01% | -0.03% | 100% | 100% |
| Class E | 0.02% | -0.04% | 0.03% | 99% | 101% |
| Overall | 0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.04% | 0.04% | -0.02% | 99% | 97% |
| Class F | -0.01% | -0.03% | 0.01% | 99% | 98% |
| Class TGM | 0.19% | 0.16% | 0.23% | 99% | 98% |

# Proposed Text

The proposed method is described in the form of text modifications relative to the specification text of VVC Draft v6 [1] shown below. The text changes are highlighted in yellow and texts with strikethrough indicates deleted texts.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

- If transform_skip_flag[ x0 ][ y0 ] is equal to 1, trafoSkip is set equal to 1 and the following applies:

locSumAbs = 0
if( xC > 0 )
	locSumAbs += AbsLevel[ xC − 1 ][ yC ]
if( yC > 0 )	(9-9)
	locSumAbs += AbsLevel[ xC ][ yC − 1 ]
locSumAbs = Clip3( 0, 31, (locSumAbs >>1) + 1 )

- Otherwise (transform_skip_flag[ x0 ][ y0 ] is equal to 0), trafoSkip is set equal to 0 and the following applies:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(9-10)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel [ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

The following applies:

- If baseLevel is equal to 0, the variable s is set equal to Max( 0, QState − 1 ) and given the variables locSumAbs, trafoSkip and s, the Rice parameter cRiceParam and the variable ZeroPos[ n ] are derived as specified in Table 9-78.

- Otherwise (baseLevel is greater than 0), given the variables locSumAbs and trafoSkip, the Rice parameter cRiceParam is derived as specified in Table 9-78.

Table 9-78 – Specification of cRiceParam and ZeroPos[ n ] based on locSumAbs, trafoSkip and s

| trafoSkip | s | locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| 1 |  | cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
|  | 0 | ZeroPos[ n ] | 0 | 0 | 0 | 0 | 0 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 4 | 4 | 4 | 4 |
|  | 1 | ZeroPos[ n ] | 1 | 1 | 1 | 1 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  | 2 | ZeroPos[ n ] | 1 | 1 | 2 | 2 | 2 | 3 | 4 | 4 | 4 | 6 | 6 | 6 | 8 | 8 | 8 | 8 |
|  |  | locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| 0 |  | cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |
| 1 |  | cRiceParam | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | 0 | ZeroPos[ n ] | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 8 | 8 | 8 | 8 | 8 | 16 | 16 | 16 | 16 |
|  | 1 | ZeroPos[ n ] | 4 | 4 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 |
|  | 2 | ZeroPos[ n ] | 8 | 8 | 12 | 12 | 12 | 12 | 12 | 12 | 12 | 16 | 16 | 16 | 16 | 16 | 16 | 16 |

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001-vE, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-O2027-v3, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-6.0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).