| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0456-v1 |
| --- | --- |

| Title: | Non-CE8: IBC chroma mode with dual-tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, IBC chroma mode is enabled for the dual-tree case. The proposed chroma IBC mode is a block level mode where samples of the entire chroma CU share the same BV, which is derived from the collocated luma area. This is guaranteed as a bitstream conformance condition. Simulation results report that:

- The BD rate changes for Class F average are 0.00%/-0.06%/0.00% for AI/RA/LB, separately, when compared to VTM-6.0 anchor.

- The BD rate for Class TGM average are -1.21%/-0.53%/0.00% for AI/RA/LB, separately, when compared to VTM-6.0 anchor.

Problem Statement

In current VVC specification [1], IBC chroma mode is disabled when dual-tree is used. The concern for using chroma IBC mode was due to the sub-block nature of chroma IBC mode which leads to complex BV derivation. Roughly 1.8% TGM AI loss was observed when chroma IBC mode was disabled from the last meeting.

Proposed method

In this contribution, a block level IBC chroma mode is proposed. From the decoder side, for dual-tree chroma coding, a block level IBC flag is signaled. If a chroma CU chooses IBC mode, the whole block will derive a single BV from the collocated luma area. From the encoder perspective, the encoder should guarantee that the entire collocated luma area are coded in IBC mode with the same luma BV.

In this way, the chroma IBC coded CU can still derive the BV from its collocated luma, which is safe for quality consideration. At the mean time, the decoding process can be the same with dual-tree or without dual-tree. It is asserted that no added decoding complexity for enabling such a chroma IBC mode.

Simulation results

The proposed method is implemented on top of VTM-6.0.

Results are compared to VTM-6.0 IBC on and VTM-6.0 IBC plus sub-block based chroma IBC mode, separately.

Table I: Performance of proposed method vs. VTM-6.0 IBC on and VTM-6.0 IBC plus subblock chroma IBC

Conclusion

In this contribution, a block based chroma IBC mode is proposed. From the simulation results, it shows about 2/3 of the coding benefit of using sub-block based chroma IBC mode has been regained, without imposing more decoding complexity to the design. It is recommended to include the block based IBC chroma mode into next VVC specification and VTM.

References

[1]	B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, July. 2019

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).