| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0458 |
| --- | --- |

| Title: | AHG17: NAL unit type mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Karsten Suehring Yago Sanchez | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add a new NAL unit type, referred to as “mappable” NAL unit type, that is to mapped to a NAL unit type indicated within the SPS. A solution is proposed for merging scenarios where the sub-bitstreams merged together have different random access properties.

# Introduction

When combining bitstreams, the random access properties of the different sub-bitstreams may be different. For instance, 360-degree video streaming use cases, where the content is tiled into multiple resolutions so that it is streamed depending on the current viewing direction, the content is typically offered at several versions with different RAP periods. Thus, when the viewing orientation of the user changes, the tile configuration (tiles that are downloaded at high or low resolutions) changes to adapt to the viewing orientation. In such cases, only tiles that experience a change are downloaded with a RAP. This tile configuration change is illustrated in the following figure.

Other scenarios, where different sub-bitstreams can have different random access properties may occur in packed formats using different resolutions in 360-degree use cases, where it may be desirable to encode the different resolutions with different random access period.

The result of such a streaming scenario, where a server may select different pre-encoded segments for different view directions and re-combine those into a bitstream to be jointly decoded is shown in the following.

A decoder that decodes the combined stream though should not have to take care of random access properties of specific regions. Currently, the way such RAP period mixing has been implemented for HEVC (e.g. OMAF use cases) is by rewriting NAL unit types (and slice headers). Although at first sight rewriting might not be seen as a cumbersome operation, system layer integration has become more a more complex having to implement some kind of rewriting operation by means of extractors.

If there are means that avoid such operations to be implemented into the system layer it is desirable to specify them in the video layer.

# Proposal

A simple way to reduce the pain of rewriting NAL unit types in each affected NAL unit header is to define a mappable NAL unit type of the VCL class with an indirection defined in a parameter set since parameter set rewriting is typically unavoidable anyways:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| […] | […] | […] | […] |
| 12 | MAP_NUT | Mappable VCL NAL unit type | VCL |
| […] | […] | […] | […] |

The mapped NAL unit type is signalled in the SPS:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| […] |  |
| mapped_nut | u(5) |
| […] |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

mapped_nut indicates the NalUnitType of NAL units with nal_unit_type equal to MAP_NUT.

A new variable NalUnitType is defined as:

The variable NalUnitType is defined as follows:

When nal_unit_type != MAP_NUT

NalUnitType is set equal to nal_unit_type

Otherwise (nal_unit_type == MAP_NUT)

NalUnitType is set equal to mapped_nut

and being used instead of directly referencing nal_unit_type.

So, when a (sub-)bitstream is combined with another one, the merging process involves only rewriting the mapped NAL type indirection in the SPS, which is required anyway, and thus the value of NalUnitType can easly change without having to change nal_unit_type of several NAL units within the bitstream.

The following constraints are added:

- The value of mapped_nut in the SPS shall be equal to IDR_N_LP or TRAIL_N.

- An access unit may contain NAL unit type MAP_NUT when it contains IDR_N_LP or TRAIL_NUT.

- When an access unit contains NAL unit with type IDR_N_LP or TRAIL_NUT, it is a requirement of bitstream conformance that mapped_nut is the respective type.

The value of IDR_W_RADL has been intentionally left out of the above proposal. However, if it is desirable that mapped_nut should also be allowed to signal IDR_W_RADL, i.e. enable mixing RAP periods with leading picture structures, it would be required to add two instances of mapped_nut in the SPS and reserve two NAL unit types as suggested below:

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| […] | […] | […] | […] |
| 12 | MAP_NUT_1 | Mappable VCL NAL unit type | VCL |
| 13 | MAP_NUT_2 | Mappable VCL NAL unit type | VCL |
| […] | […] | […] | […] |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).