#### Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| min_qp_prime_ts_minus4 | ue(v) |
| min_qp_prime_chroma_ts_minus4 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| if( sps_max_sub_layers_minus1 > 0 ) |  |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| …… |  |

#### Sequence parameter set RBSP semantics

bit_depth_luma_minus8 specifies the bit depth of the samples of the luma array BitDepthY and the value of the luma quantization parameter range offset QpBdOffsetY as follows:

BitDepthY      = 8 + bit_depth_luma_minus8		(7-8)

QpBdOffsetY = 6 * bit_depth_luma_minus8		(7-9)

bit_depth_luma_minus8 shall be in the range of 0 to 8, inclusive.

bit_depth_chroma_minus8 specifies the bit depth of the samples of the chroma arrays BitDepthC and the value of the chroma quantization parameter range offset QpBdOffsetC as follows:

BitDepthC      = 8 + bit_depth_chroma_minus8		(7-10)

QpBdOffsetC = 6 * bit_depth_chroma_minus8		(7-11)

bit_depth_chroma_minus8 shall be in the range of 0 to 8, inclusive.

min_qp_prime_ts_minus4 specifies the minimum allowed quantization parameter for transform skip and palette mode as follows:

QpPrimeTsMin = 4 + min_qp_prime_ts_ minus4		(7-12)

min_qp_prime_chroma_ts_minus4 specifies the minimum allowed quantization parameter for transform skip and palette mode as follows:

QpPrimeChromaTsMin = 4 + min_qp_prime_chroma_ts_ minus4	(7-13)

…..

#### 8.4.5.3 Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1 and cu_transquant_bypass_flag is equal to 0), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Max( 0 QpPrimeTsMin, Qp′Y )		(8-272)

- Otherwise, if cIdx is equal to 1,

qP = Max( 0 QpPrimeTsMinChroma, Qp′Cb )		(8-273)

- Otherwise (cIdx is equal to 2),

qP = Max( 0 QpPrimeTsMinChroma, Qp′Cr )		(8-274)

