| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0448 |
| --- | --- |

| Title: | Non-CE7 : On updating a variable for context model selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jungah Choi Jaehyun Lim Seung Hwan Kim | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the residual coding part in VVC, a syntax element indicating absolute level in the first pass (i.e., AbsLevelPass1) is referenced when deriving the context model index of sig_coeff_flag, par_flag, and gtx_flag. Considering that the maximum value of the AbsLevelPass1 is defined by 5, this variable should be updated for the next CG after the coefficient levels are reconstructed completely. Specifically, this contribution introduces two alternative methods to fix a specification on updating AbsLevelPass1 for the syntax elements in residual coding as follows:

- Method 1: Specification bug fix by adding AbsLevelPass1 update after abs_level coding

- Method 2: No update process of AbsLevelPass1 by restriction of the value of AbsLevelPass1 equal to or less than 4

For Method 2, AbsLevelPass1 is updated for the next CG when the number of context coded bins (CCBs), remBinsPass1, is greater than 4, since AbsLevelPass1 is referred when deriving the context model index of sig_coeff_flag[ ], par_flag[ ], and gtx_flag[ ][0]. By restricting AbsLevelPass1 to be less than or equal to 4 for each coordination, the update process can be removed. The experiment shows the impact of the aspect 2, 0.00% (AI) / X.XX% (RA) / X.XX% (LDB), respectively, for the regular QPs, and 0.00% (AI) / X.XX% (RA) / X.XX% (LDB), for the low QPs, correspondingly.

Problem Statement

In the current VVC [1], the absolute level in pass 1 in RC, AbsLevelPass1, is referenced when deriving the context model index of sig_coeff_flag. If the remaining CCBs, remBinsPass1, is greater than 4 at the stage that abs_remainder is parsed, AbsLevelPass1 should be updated for the next CG because the value can be 0 to 5. It can be 5 when the absolute value is an odd number and greater than 4 whereas the maximum value of the AbsLevelPass1 is 4.

Proposed method

Bugfix #1 of the current spec for method 1

7.3.8.11 	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| (…) |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| abs_level_gtx_flag[ n ][ 1 ] | ae(v) |
| remBinsPass1− − |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( dep_quant_enabled_flag ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| if( remBinsPass1 < 4 ) |  |
| firstPosMode1 = n − 1 |  |
| } |  |
| for( n = numSbCoeff − 1; n >= firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevel Pass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel [ xC ][ yC ] & 1 ), AbsLevel [ xC ][ yC ] ) |  |
| } |  |
| (…) |  |
| } |  |  |

- 7.4.9.11  Residual coding semantics

(…)

dec_abs_level[ n ] is an intermediate value that is coded with Golomb-Rice code at the scanning position n. Given ZeroPos[ n ] that is derived in clause 9.3.3.2 during the parsing of dec_abs_level[ n ], the absolute value of a transform coefficient level at location ( xC, yC ) AbsLevel[ xC ][ yC ] is derived using as follows:

- If dec_abs_level[ n ] is equal to ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to 0.

- Otherwise, if dec_abs_level[ n ] is less than ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ] + 1;

- Otherwise (dec_abs_level[ n ] is greater than ZeroPos[ n ]), AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ].

The value of AbsLevelPass1[ xC ][ yC ] is derived as follows:

AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel[ xC ][ yC ] & 1 ), AbsLevel[ xC ][ yC ] ) 	(7-176)

It is a requirement of bitstream conformance that the value of dec_abs_level[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

(…)

Bugfix #2 of the current spec for method 1

- 7.4.9.11  Residual coding semantics

(…)

abs_remainder[ n ] is the remaining absolute value of a transform coefficient level that is coded with Golomb-Rice code at the scanning position n. When abs_remainder[ n ] is not present, it is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of abs_remainder[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

The value of AbsLevelPass1[ xC ][ yC ] is derived as follows:

AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel[ xC ][ yC ] & 1 ), AbsLevel[ xC ][ yC ] ) 	(7-176)

dec_abs_level[ n ] is an intermediate value that is coded with Golomb-Rice code at the scanning position n. Given ZeroPos[ n ] that is derived in clause 9.3.3.2 during the parsing of dec_abs_level[ n ], the absolute value of a transform coefficient level at location ( xC, yC ) AbsLevel[ xC ][ yC ] is derived using as follows:

- If dec_abs_level[ n ] is equal to ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to 0.

- Otherwise, if dec_abs_level[ n ] is less than ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ] + 1;

- Otherwise (dec_abs_level[ n ] is greater than ZeroPos[ n ]), AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ].

The value of AbsLevelPass1[ xC ][ yC ] is derived as follows:

AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel[ xC ][ yC ] & 1 ), AbsLevel[ xC ][ yC ] ) 	(7-176)

It is a requirement of bitstream conformance that the value of dec_abs_level[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

(…)

Restriction of the maximum value by 4 for AbsLevelPass1 (Method 2)

If the maximum value of AbsLevelPass1 is restricted by 4 for each coordination, the update process can be removed, since AbsLevelPass1 can be updated by 4 at most after all the syntax elements are parsed in pass 1.

- 7.4.9.11 Residual coding semantics

(…)

abs_remainder[ n ] is the remaining absolute value of a transform coefficient level that is coded with Golomb-Rice code at the scanning position n. When abs_remainder[ n ] is not present, it is inferred to be equal to 0.

It is a requirement of bitstream conformance that the value of abs_remainder[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

dec_abs_level[ n ] is an intermediate value that is coded with Golomb-Rice code at the scanning position n. Given ZeroPos[ n ] that is derived in clause 9.3.3.2 during the parsing of dec_abs_level[ n ], the absolute value of a transform coefficient level at location ( xC, yC ) AbsLevel[ xC ][ yC ] is derived using as follows:

- If dec_abs_level[ n ] is equal to ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to 0.

- Otherwise, if dec_abs_level[ n ] is less than ZeroPos[ n ], AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ] + 1;

- Otherwise (dec_abs_level[ n ] is greater than ZeroPos[ n ]), AbsLevel[ xC ][ yC ] is set equal to dec_abs_level[ n ].

The value of AbsLevelPass1[ xC ][ yC ] is derived as follows:

AbsLevelPass1[ xC ][ yC ] = Min( 4 + ( AbsLevel[ xC ][ yC ] & 1 ), AbsLevel[ xC ][ yC ] ) 	(7-176)

It is a requirement of bitstream conformance that the value of dec_abs_level[ n ] shall be constrained such that the corresponding value of TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] is in the range of CoeffMin to CoeffMax, inclusive.

(…)

Experimental results

For the method 2, the coding efficiency can be affected by the proposed change. Therefore, the coding performance for the method 2 is given below. The simulation is constructed using VTM-6.0 with the common test conditions [2] and low Qps (2, 7, 12, 17), as required in CE7 [3].

Test results for CTC Qps (22, 27, 32, 37)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.04% | 99% | 101% |
| Class A2 | 0.01% | 0.01% | -0.04% | 100% | 100% |
| Class B | 0.01% | 0.01% | -0.05% | 99% | 102% |
| Class C | 0.00% | 0.01% | 0.08% | 97% | 99% |
| Class E | -0.01% | 0.01% | 0.08% | 97% | 102% |
| Overall | 0.00% | 0.00% | 0.01% | 98% | 101% |
| Class D | 0.01% | 0.06% | -0.14% | 96% | 92% |
| Class F | -0.01% | 0.03% | 0.00% | 98% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.07% | -0.02% | 97% | 100% |
| Class C | -0.01% | 0.14% | -0.05% | 95% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.09% | -0.10% | 94% | 91% |
| Class F | 0.01% | -0.06% | -0.02% | 96% | 98% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.04% | 0.08% | 0.05% | 98% | 100% |
| Class E | 0.05% | 0.20% | 0.74% | 98% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.03% | 0.30% | 0.29% | 98% | 98% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test results for CTC Qps (2, 7, 12, 17)

Reference(s)

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” JVET-O2027, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).