| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0502 |
| --- | --- |

| Title: | AHG18: Interaction between lossless coding and max transform size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In AHG18, coding tools for lossless and near-lossless coding are being studied for VVC. The initial VTM-6 based SW supporting lossless coding mode has been released through JVET reflector, and the changes include a configuration on setting maximum transform size to be 32x32 because 64-length transform cannot guarantee lossless coding. In this contribution, it is proposed to avoid redundant signaling of lossless coding flag if transform size is greater than 32.

# Problem Statement

Support on lossless coding is being studied in AHG18. An initial SW (based on VTM-6.0) has been released through JVET reflector with the following changes:

- Disable JCCR locally for lossless

- Disable SBT locally for lossless

- Disable LCMS completely. It was not possible to disable LCMS locally without further delay.

- Includes fix for ticket 484

- Adjust maximum TB size to 32x32

Among the above changes, max transform block size is configured as 32x32 because 64-length transform, which drops high frequency coefficients, cannot guarantee lossless coding. However, if the max transform size is set as 64x64, in VTM-6.0, the lossless mode flag (cu_transquant_bypass_flag) is still being redundantly signaled for CU size greater than 32x32.

# Proposal

It is proposed to avoid signaling cu_transquant_bypass_flag if 64-length transform is to be used for the current coding unit.

The proposed spec text changes are highlighted as follows,

#### 7.3.8.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( transquant_bypass_enabled_flag && !(max(cbWidth, cbHeight) > 32 && MaxTbLog2SizeY > 32) ) |  |
| cu_transquant_bypass_flag |  |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| …… |  |

# Conclusion

In this contribution, it is proposed to avoid redundant signaling of lossless coding flag in case the current CU applies 64-length transform. It is recommended to include the proposed changes into next VTM software and VVC spec text.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).