| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0543-v2 |
| --- | --- |

| Title: | CE5-related: Cleanup on cross-component adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jangwon Choi Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, jangwon84.choi, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

A cross component adaptive loop filter (CC-ALF) is being investigated in CE5-2.1. Unlike VVC ALF, CC-ALF allows ALF coefficients used for each chroma component of a slice to be reused in temporal sub-layer. In addition, CC-ALF is controlled on a variable block size, which is signalled at the slice-level. This contribution proposes to keep the concept of temporal prediction for ALF is to be controlled by signaling APS id. Also, CC-ALF is to be controlled at the CTU-level as current ALF in VVC. From experiment results, changes of coding performance are negligible.

Introduction

In the last meeting, CC-ALF was proposed [1] and it is being investigated in CE5-2.1 and CE5-2.2 at this meeting [2]. The syntax design of CC-ALF is slightly different from ALF in VVC. Firstly, CC-ALF coefficients used for each chroma component of a slice are reused corresponding to temporal sub-layer. It is facilitated using slice-level flags. Secondly, the usage of the CC-ALF filters is controlled on a variable block size which is signaled at the slice-level. In this contribution, those two aspects are removed.

Proposed method

Aspect 1: Remove temporal prediction for CC-ALF

In VVC, a concept of temporal prediction for ALF is included by signaling APS id. To unify ALF and CC-ALF, temporal prediction for CC-ALF are removed as follows,

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_reuse_temporal_layer_filter_flag | u(1) |
| if (!slice_cross_component_alf_cb_reuse_temporal_layer_filter_flag) |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_alf_cb_log2_control_size_minus4 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_reuse_temporal_layer_filter_flag | u(1) |
| if (!slice_cross_component_alf_cr_reuse_temporal_layer_filter_flag) |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_alf_cr_log2_control_size_minus4 | ue(v) |
| } |  |
| … |  |

Aspect 2: Instead of variable block size, CC-ALF is controlled at the CTU-level as current

In VVC, application for ALF is controlled at the CTU-level. However, CC-ALF has own control block size. To unify ALF and CC-ALF, CC-ALF is also controlled at CTU level as follows,

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_reuse_temporal_layer_filter_flag | u(1) |
| if (!slice_cross_component_alf_cb_reuse_temporal_layer_filter_flag) |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_alf_cb_log2_control_size_minus4 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_reuse_temporal_layer_filter_flag | u(1) |
| if (!slice_cross_component_alf_cr_reuse_temporal_layer_filter_flag) |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_alf_cr_log2_control_size_minus4 | ue(v) |
| } |  |
| … |  |

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| xEnd = xCtb + (1 << CtbLog2SizeY ) |  |
| xEnd = ( xEnd >= pic_width_in_luma_samples ) ? pic_width_in_luma_samples : xEnd |  |
| yEnd = yCtb + (1 << CtbLog2SizeY ) |  |
| yEnd = ( yEnd >= pic_height_in_luma_samples ) ? pic_height_in_luma_samples : yEnd |  |
| if ( slice_cross_component_alf_cb_enabled_flag ) |  |
| for ( yL = yCtb; yL < yEnd; yL += CcAlfHeightCbL ) |  |
| for ( xL = xCtb; xL < xEnd; xL += CcAlfWidthCbL ) |  |
| alf_cross_component_cb_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] [ xL / CcAlfWidthCbL ][ yL / CcAlfHeightCbL ] | ae(v) |
| if ( slice_cross_component_alf_cr_enabled_flag ) |  |
| for ( yL = yCtb; yL < yEnd; yL += CcAlfHeightCrL ) |  |
| for ( xL = xStartC; xL < xEndC; xL += CcAlfWidthCrL ) |  |
| alf_cross_component_cr_flag[ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] [ xL / CcAlfWidthCrL ][ yL / CcAlfHeightCrL ] | ae(v) |
| … |  |

Experimental results

CTC results

The proposed methods are implemented on top of CE5-2.1 software. The simulations are conducted on shifts the chroma gain to luma by adjusting the chroma QP offset increased by 1 and Lagrangian cost parameter lambda used by the loop.

Test 1: Remove temporal prediction for CC-ALF

Table 1. Performance of test 1 in AI

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.19% |  | 5.37% |  | 3.75% |  | 111% |  | 106% |  |
| Class A2 | -2.04% | -0.29% | -0.74% | 105% | 105% |
| Class B | -0.74% | -1.74% | 0.92% | 107% | 109% |
| Class C |  | -0.76% |  | 3.98% |  | 4.50% |  | 105% |  | 111% |  |
| Class E |  | -0.46% |  | -2.81% |  | 4.71% |  | 107% |  | 111% |  |
| Overall | -0.99% | 0.78% | 2.54% | 107% | 109% |
| Class D |  | -0.75% |  | 5.36% |  | 4.85% |  | 103% |  | 106% |  |
| Class F | -0.73% | -1.03% | -1.29% | 104% | 109% |

Table 2. Performance of test 1 in RA

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.81% |  | 5.32% |  | 4.14% |  | 104% |  | 104% |  |
| Class A2 |  | -2.05% |  | -1.17% |  | -6.17% |  | 104% |  | 105% |  |
| Class B |  | -0.54% |  | -6.33% |  | -2.27% |  | 103% |  | 104% |  |
| Class C |  | -0.77% |  | 5.70% |  | 6.58% |  | 102% |  | 102% |  |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.24% | 0.59% | 103% | 104% |
| Class D |  | -0.66% |  | 6.42% |  | 6.44% |  | 103% |  | 103% |  |
| Class F | -0.93% | 1.88% | 2.37% | 104% | 104% |

Table 3. Performance of test 1 in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | -0.50% |  | -9.16% |  | -6.51% |  | 101% |  | 107% |  |
| Class C |  | -0.76% |  | 3.92% |  | 5.78% |  | 101% |  | 108% |  |
| Class E |  | -0.31% |  | -5.39% |  | 7.32% |  | 105% |  | 106% |  |
| Overall |  | -0.54% |  | -3.86% |  | 1.04% |  | 102% |  | 107% |  |
| Class D |  | -0.56% |  | 4.84% |  | 7.35% |  | 100% |  | 104% |  |
| Class F | -0.90% | -2.54% | -0.24% | 102% | 106% |

Test 2: Remove variable block size to control the application

Table 4. Performance of test 2 in AI

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.20% |  | 5.40% |  | 3.99% |  | 104% |  | 107% |  |
| Class A2 | -2.12% | 0.13% | -0.29% | 100% | 105% |
| Class B | -0.77% | -1.22% | 1.31% | 103% | 110% |
| Class C |  | -0.78% |  | 4.32% |  | 4.71% |  | 102% |  | 111% |  |
| Class E |  | -0.47% |  | -2.66% |  | 4.79% |  | 102% |  | 112% |  |
| Overall | -1.02% | 1.10% | 2.82% | 102% | 109% |
| Class D |  | -0.78% |  | 5.73% |  | 5.26% |  | 101% |  | 105% |  |
| Class F | -0.83% | 0.16% | -0.52% | 102% | 110% |

Table 5. Performance of test 2 in RA

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.82% |  | 5.29% |  | 4.21% |  | 100% |  | 104% |  |
| Class A2 |  | -2.09% |  | -0.93% |  | -5.93% |  | 100% |  | 105% |  |
| Class B |  | -0.56% |  | -5.92% |  | -1.81% |  | 100% |  | 104% |  |
| Class C |  | -0.80% |  | 6.14% |  | 6.95% |  | 99% |  | 103% |  |
| Class E |  |  |  |  |  |
| Overall | -1.18% | 0.53% | 0.91% | 100% | 104% |
| Class D |  | -0.75% |  | 6.97% |  | 7.21% |  | 101% |  | 105% |  |
| Class F | -0.99% | 2.59% | 2.80% | 100% | 103% |

Table 6. Performance of test 2 in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | -0.54% |  | -8.72% |  | -6.13% |  | 101% |  | 107% |  |
| Class C |  | -0.80% |  | 5.27% |  | 7.18% |  | 100% |  | 107% |  |
| Class E |  | -0.14% |  | -5.47% |  | 7.53% |  | 102% |  | 108% |  |
| Overall |  | -0.52% |  | -3.25% |  | 1.72% |  | 101% |  | 107% |  |
| Class D |  | -0.65% |  | 7.04% |  | 9.76% |  | 100% |  | 104% |  |
| Class F | -0.95% | -1.52% | 0.29% | 101% | 106% |

Test 3: Test 1 + Test 2

Table 7. Performance of test 3 in AI

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.21% |  | 5.40% |  | 3.99% |  | 106% |  | 108% |  |
| Class A2 | -2.12% | 0.13% | -0.29% | 101% | 105% |
| Class B | -0.78% | -1.22% | 1.31% | 104% | 110% |
| Class C |  | -0.79% |  | 4.32% |  | 4.70% |  | 102% |  | 111% |  |
| Class E |  | -0.48% |  | -2.67% |  | 4.78% |  | 104% |  | 112% |  |
| Overall | -1.02% | 1.10% | 2.82% | 103% | 109% |
| Class D |  | -0.78% |  | 5.73% |  | 5.26% |  | 101% |  | 107% |  |
| Class F | -0.83% | 0.16% | -0.52% | 103% | 111% |

Table 8. Performance of test 3 in RA

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  | -1.81% |  | 5.35% |  | 4.20% |  | 101% |  | 104% |  |
| Class A2 |  | -2.11% |  | -0.86% |  | -5.93% |  | 101% |  | 105% |  |
| Class B |  | -0.56% |  | -5.94% |  | -1.77% |  | 101% |  | 104% |  |
| Class C |  | -0.82% |  | 6.11% |  | 6.84% |  | 100% |  | 102% |  |
| Class E |  |  |  |  |  |
| Overall | -1.19% | 0.55% | 0.89% | 100% | 104% |
| Class D |  | -0.72% |  | 7.08% |  | 7.23% |  | 101% |  | 104% |  |
| Class F | -0.99% | 2.62% | 2.85% | 101% | 103% |

Table 9. Performance of test 3 in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  | -0.54% |  | -8.74% |  | -5.87% |  | 102% |  | 108% |  |
| Class C |  | -0.78% |  | 5.41% |  | 7.42% |  | 101% |  | 108% |  |
| Class E |  | -0.23% |  | -5.20% |  | 7.58% |  | 103% |  | 107% |  |
| Overall |  | -0.54% |  | -3.14% |  | 1.92% |  | 102% |  | 108% |  |
| Class D |  | -0.62% |  | 6.91% |  | 9.66% |  | 101% |  | 104% |  |
| Class F | -0.90% | -1.36% | 0.73% | 101% | 106% |

Additional results

The proposed methods are implemented on top of CE5-2.1 software. The simulations are conducted on shifts the chroma gain to luma by adjusting the chroma QP offset increased by 1 and Lagrangian cost parameter lambda used by the loop. The Anchor is also CE5-2.1 with on shifts the chroma gain to luma by adjusting the chroma QP and Lagrangian cost parameter lambda used by the loop.

Test 1: Remove temporal prediction for CC-ALF

Table 10. Performance of test 1 in AI compared with CC-ALF

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 99% |

Table 11. Performance of test 1 in RA compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.06% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | -0.02% | 0.00% | 100% | 101% |
| Class B | 0.00% | 0.02% | 0.00% | 100% | 101% |
| Class C | 0.01% | -0.05% | 0.01% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | -0.02% | 0.36% | 0.03% | 99% | 100% |
| Class F | -0.01% | 0.03% | 0.03% | 100% | 101% |

Table 12. Performance of test 1 in LD compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.14% | 0.14% | 100% | 103% |
| Class C | -0.03% | 0.05% | 0.33% | 99% | 99% |
| Class E | -0.07% | 0.25% | -0.23% | 101% | 98% |
| Overall | -0.01% | 0.14% | 0.11% | 100% | 100% |
| Class D | 0.00% | -0.08% | 0.01% | 100% | 101% |
| Class F | -0.06% | 0.15% | -0.01% | 99% | 101% |

Test 2: Remove variable block size to control the application

Table 13. Performance of test 2 in AI compared with CC-ALF

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.25% | 97% | 101% |
| Class A2 | -0.09% | 0.45% | 0.47% | 98% | 100% |
| Class B | -0.03% | 0.58% | 0.42% | 99% | 100% |
| Class C | -0.03% | 0.32% | 0.18% | 99% | 101% |
| Class E | -0.01% | 0.16% | 0.07% | 99% | 100% |
| Overall | -0.03% | 0.34% | 0.29% | 98% | 100% |
| Class D | -0.03% | 0.34% | 0.37% | 99% | 101% |
| Class F | -0.10% | 1.94% | 1.00% | 99% | 100% |

Table 14. Performance of test 2 in RA compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | 0.07% | 99% | 101% |
| Class A2 | -0.05% | 0.23% | 0.29% | 99% | 101% |
| Class B | -0.03% | 0.49% | 0.48% | 99% | 101% |
| Class C | -0.02% | 0.37% | 0.34% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.32% | 0.32% | 99% | 101% |
| Class D | -0.11% | 0.84% | 0.74% | 99% | 102% |
| Class F | -0.08% | 1.17% | 0.67% | 99% | 101% |

Table 15. Performance of test 2 in LD compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.70% | 0.45% | 99% | 103% |
| Class C | -0.06% | 1.44% | 1.71% | 99% | 99% |
| Class E | 0.10% | 0.16% | -0.05% | 97% | 100% |
| Overall | 0.00% | 0.81% | 0.75% | 99% | 101% |
| Class D | -0.09% | 2.06% | 2.25% | 99% | 101% |
| Class F | -0.12% | 1.54% | 0.62% | 99% | 100% |

Test 3: Test 1 + Test 2

Table 16. Performance of test 3 in AI compared with CC-ALF

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.24% | 99% | 101% |
| Class A2 | -0.09% | 0.45% | 0.47% | 99% | 100% |
| Class B | -0.03% | 0.58% | 0.42% | 99% | 100% |
| Class C | -0.03% | 0.32% | 0.18% | 99% | 100% |
| Class E | -0.02% | 0.16% | 0.06% | 100% | 101% |
| Overall | -0.04% | 0.34% | 0.28% | 99% | 100% |
| Class D | -0.03% | 0.34% | 0.37% | 100% | 102% |
| Class F | -0.10% | 1.94% | 1.00% | 100% | 100% |

Table 17. Performance of test 3 in RA compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | 0.05% | 100% | 101% |
| Class A2 | -0.06% | 0.30% | 0.28% | 100% | 101% |
| Class B | -0.02% | 0.48% | 0.53% | 99% | 100% |
| Class C | -0.04% | 0.33% | 0.23% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.33% | 0.31% | 100% | 101% |
| Class D | -0.09% | 0.97% | 0.76% | 99% | 101% |
| Class F | -0.07% | 1.20% | 0.72% | 99% | 100% |

Table 18. Performance of test 3 in LD compared with CC-ALF

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.66% | 0.86% | 100% | 103% |
| Class C | -0.05% | 1.59% | 1.93% | 100% | 99% |
| Class E | 0.01% | 0.43% | -0.02% | 98% | 99% |
| Overall | -0.01% | 0.91% | 1.00% | 99% | 101% |
| Class D | -0.06% | 1.94% | 2.19% | 100% | 101% |
| Class F | -0.06% | 1.61% | 1.06% | 99% | 101% |

Conclusion

In this contribution, two modification are proposed to unify syntax signaling for CC-ALF in CE5-2.1. From experiments, coding affect is minor. It is recommended to adopt proposed modifications with CE5-2.1.

Reference

- K. Misra, F. Bossen, A. Segall, “Cross-Component Adaptive Loop Filter for chroma,” JVET-O0636, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- K. Misra, F. Bossen, A. Segall, “CE5-2.1, CE5-2.2: Cross-Component Adaptive Loop Filter,” JVET-P0080, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 October 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).