| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0550-v2 |
| --- | --- |

| Title: | Non-CE3 : Cleanup of intra reference sample filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Jangwon Choi Junghak Nam hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, jangwon84.choi, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution simplifies intra reference sample filter; one reference sample filter and two interpolation filters (Gaussian and DCT-IF) selection process. In Test 1, the only interpolation filter is selected based on the block size. Test 1 provide the coding performance 0.03% and 0.01% BD-rate in AI and RA configuration, respectively. In Test 2, both the reference sample filter and the interpolation filter are derived according to the block size. Test 2 provides the coding performance 0.03% and 0.01% BD-rate in AI and RA configuration, respectively. Encoding and decoding run-times are unaffected.

Introduction

In the last meeting, JVET-O0277 [1] was adopted to simplify and unify the intra reference sample filter selection. The contribution uses the modified size-dependent threshold to simply determine intra reference sample filters (reference sample filter and interpolation filter). However, since the contribution still uses the size-dependent threshold to select intra reference sample filters, intra reference sample filters should be selected in difference process according to the size-dependent threshold. Moreover, the process of determining the intra reference sample filter has still multiple checks, as intra mode and sub-partition size should be considered to determine the size-dependent threshold. This contribution proposes to remove the size-dependent threshold to simplify and unify the intra reference sample filter selection.

Proposed method

This contribution proposes to determine intra reference sample filter selection without using the size-dependent threshold. Table 1 and Table 2 represent the proposed intra reference sample filter selection process for Test 1 and Test 2, respectively.

In Table 1 and Table 2, the refFilterFlag and the interpolationFlag represent the reference sample filter flag and the interpolation filter flag, respectively. The nTbS is set equal to (Log2 ( nTbW ) + Log2 ( nTbH )) >> 1, where nTbW and nTbH means the transform block width and the tranform block height, respectively. If the refFilterFlag is on/off, the intra reference sample smoothing filter is enabled/disabled. In addition, if the interpolationFlag is true/false, Gaussian/DCT-IF filter is applied. The intraMode, HorMode, and VerMode represent the current intra mode, horizontal mode, and vertical mode. If the current intra mode is one of {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80} (the intra prediction mode refers to integer reconstructed boundary samples), isIntegerSlop is true. Otherwise, isIntegerSlop returns false.

The only difference between Test 1 and Test 2 is the refFilterFlag selection process.

Table 1. Proposed intra reference sample filter selection process for Test 1

|  | VTM-6.0 |  | Proposed |
| --- | --- | --- | --- |
| Chroma, ISP, MIP, MRL, DC | - refFilterFlag = false - interpolationFlag = DCT-IF |  |  |
| BDPCM | - refFilterFlag = false - interpolationFlag = DCT-IF |  |  |
| Planar | - refFilterFlag = luma CB size > 32 ? true : false |  |  |
| Angular | diff = min( abs(intraMode – HorMode), abs(intraMode – VerMode) ) |  | - refFilterFlag = isIntegerSlop - interpolationFlag = (nTbS > 3) ? !isIntegerSlop : DCT-IF |
|  | diff > *size-dependent threshold | diff ≤ *size-dependent threshold |  |
|  | - refFilterFlag = isIntegerSlop - interpolationFlag = !isIntegerSlop | - refFilterFlag = false - interpolationFlag = DCT-IF |  |

Table 2. Proposed intra reference sample filter selection process for Test 2

|  | VTM-6.0 |  | Proposed |
| --- | --- | --- | --- |
| Chroma, ISP, MIP, MRL, DC | - refFilterFlag = false - interpolationFlag = DCT-IF |  |  |
| BDPCM | - refFilterFlag = false - interpolationFlag = DCT-IF |  |  |
| Planar | - refFilterFlag = luma CB size > 32 ? true : false |  |  |
| Angular | diff = min( abs(intraMode – HorMode), abs(intraMode – VerMode) ) |  | - refFilterFlag = (nTbS >= 3) ? isIntegerSlop : false - interpolationFlag = (nTbS > 3) ? !isIntegerSlop : DCT-IF |
|  | diff > *size-dependent threshold | diff ≤ *size-dependent threshold |  |
|  | - refFilterFlag = isIntegerSlop - interpolationFlag = !isIntegerSlop | - refFilterFlag = false - interpolationFlag = DCT-IF |  |

* The size-dependent threshold represents the intraHorVerDisThres[ nTbS ] in Table 8-7 in VVC text.

Table 8-7 – Specification of intraHorVerDistThres[ nTbS ] for various transform block sizes nTbS

|  | nTbS = 2 | nTbS = 3 | nTbS = 4 | nTbS = 5 | nTbS = 6 | nTbS = 7 |
| --- | --- | --- | --- | --- | --- | --- |
| intraHorVerDistThres[ nTbS ] | 24 | 14 | 2 | 0 | 0 | 0 |

In Test 1 and Test 2, in all other intra predictions except for the angular mode, the proposed method determines the intra reference sample filter in the same way as VTM-6.0.

With the Test 1, the intra reference sample filter for the angular mode is selected as follows.

- The refFilterFlag is determined by isIntegerSlop.

- The interpolationFlag is derived as follows:

- If nTbS is greater than 3, the interpolationFlag is determined by isIntegerSlop.

- Otherwise, the interpolationFlag is set to DCT-IF.

With the Test 2, the intra reference sample filter for the angular mode is selected as follows.

- The refFilterFlag is derived as follows:

- If nTbS is greater than or equal to 3, the refFilterFlag is determined by isIntegerSlop.

- Otherwise, the refFilterFlag is set to false.

- The interpolationFlag is derived as follows:

- If nTbS is greater than 3, the interpolationFlag is determined by isIntegerSlop.

- Otherwise, the interpolationFlag is set to DCT-IF.

Experimental result

The proposed method has been implemented using VTM-6.0 and under the common test conditions [2].

- Test 1: the only interpolation filter is derived according to the block size.

Table 3. Coding performance of the proposed method (AI)

Table 4. Coding performance of the proposed method (RA)

- Test 2: both the reference sample filter and interpolation filter are derived according to the block size.

Table 5. Coding performance of the proposed method (AI)

Table 6. Coding performance of the proposed method (RA)

Conclusion

This contribution proposes to remove the size-dependent threshold to simplify and unify the intra reference sample filter selection. The proposed method provides the minor coding performance change without increasing the complexity of encoding and decoding. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- C. Tsai, et al., “CE3-related: Simplification and unification for intra reference sample filtering,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-O0277, 15th Meeting, Gothenburg, SE, 3–12 July 2019.

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

- B. Bross, el al., “Versatile Video Coding (Draft 6),” JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).