| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0621 |
| --- | --- |

| Title: | Motion information comparison |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Antoine Robert, Fabrice Le Léannec, Franck Galpin, Tangi Poirier 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 antoine.robert@interdigital.com, fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

It is proposed to consider the half-pel interpolation filter flag in the motion information comparisons used in merge list constructions and HMVP list update and reading processes.

The proposed method has been tested based on the VTM-6.0 and the common test conditions. It is reported that it achieves a BD-rate loss of 0.01% in RA and a gain of -0.03% in LDB configuration. By extending the proposed method to HMVP candidates in merge list construction, the BD-rates become 0.00% in RA and -0.04% in LDB.

# Introduction

In VTM-6.0 [1], a half-pel precision of motion vector difference for AMVR coupled with an adaptive interpolation filter switching has been adopted in JVET-O0057 [2]. In case the resolution of the motion vector difference is HALF_PEL and the motion vector values are half-pel, the adaptive interpolation filter (AIF) allows selecting one interpolation filer among two different ones for motion compensation. The selection of the filter depends on the AIF flag which indicates if switching is possible, and on the motion vector magnitude.

The adaptive interpolation filter flag is stored into the motion information buffer after the CU has been reconstructed for spatial inheritance purpose. Although impacting the prediction made by using such motion information, AIF flag is not considered in motion information comparisons.

# Proposals

## Motion information comparison (test a)

During the construction of regular, IBC and TPM merge lists and during HMVP list updates, some pruning involving motion information comparisons, aka “==” operator, is used to reduce the motion data redundancy in those lists. As two motion compensated predicted blocks are different if their AIF flag are different, even with identical motion vectors and reference frames, it is proposed to consider this AIF flag in the motion information comparisons.

## Extension to HMVP candidate (test b)

In a merge list construction, some HMVP candidates can be added. In that case, some pruning is also performed by comparing HMVP candidates to merge candidates. In VTM-6.0, this pruning is not performed with the same function (“xCheckSimilarMotion”) as the other motion information comparisons as HMVP list update (“==” operator), while it is described in the same way in the draft text [1]:

For each candidate in HmvpCandList[ hMvpIdx ] with index hMvpIdx = 1..NumHmvpCand, the following ordered steps are repeated until numCurrMergeCand is equal to MaxNumMergeCand − 1:

- The variable sameMotion is derived as follows:

  - If all of the following conditions are true for any merging candidate N with N being A1 or B1, sameMotion and isPrunedN are both set equal to TRUE:

  - hMvpIdx is less than or equal to 2.

  - The candidate HmvpCandList[ NumHmvpCand − hMvpIdx] is equal to the merging candidate N.

  - isPrunedN is equal to FALSE.

  - Otherwise, sameMotion is set equal to FALSE.

…

Figure 1: Extract of the VVC draft 6 - 8.5.2.6 Derivation process for history-based merging candidates
In yellow the part corresponding to the “xCheckSimilarMotion” function

The candidate list HmvpCandList is modified using the candidate hMvpCand by the following ordered steps:

- The variable identicalCandExist is set equal to FALSE and the variable removeIdx is set equal to 0.

- When NumHmvpCand is greater than 0, for each index hMvpIdx with hMvpIdx = 0..NumHmvpCand − 1, the following steps apply until identicalCandExist is equal to TRUE:

  - When hMvpCand is equal to HmvpCandList[ hMvpIdx ], identicalCandExist is set equal to TRUE and removeIdx is set equal to hMvpIdx.

…

Figure 2: Extract of the VVC draft 6 - 8.5.2.16 Updating process for the history-based motion vector predictor candidate list
In yellow the part corresponding to the “==” operator

To be consistent with the specification, test b proposes to extend the proposed motion information comparison, which includes AIF flags comparisons, to the HMVP candidates pruning during the regular merge list construction (“xCheckSimilarMotion”).

## General remarks on motion information comparisons

As presented above, in VTM-6.0, the HMVP list update and reading processes do not use the same function to perform the motion information comparisons while it is described in the same way in the VVC draft.

Furthermore, HMVP list update and spatial merge candidate derivation which use the same function, the “==” operator, are not described in the same way in the VVC draft specification.

- The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows:

- If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and bcwIdxB1 is set equal to 0:

  - availableB1 is equal to FALSE.

  - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices.

- Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made:

…

Figure 3: Extract of the VVC draft 6 - 8.5.2.3 Derivation process for spatial merging candidates
In yellow the part corresponding to the “==” operator

These three parts of the VVC draft specification and of the VTM should be harmonized.

# Experimental Results

The simulations are conducted using the VTM-6.0 reference software with VTM configuration files and under common test conditions.

Table 1 shows the simulation results of the proposed motion information comparisons involving the AIF flag in random access and low delay B configurations. The BD-rate performances decrease a bit in random access (0.01%) but increase in low delay B (-0.03%) with the same encoding/decoding time as VTM-6.0.

Table 1: summary of proposed motion information comparison (test a).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.04% | 0.02% | 100% | 100% |
| Class A2 | 0.02% | 0.03% | 0.03% | 100% | 100% |
| Class B | 0.01% | 0.07% | -0.06% | 99% | 99% |
| Class C | 0.00% | 0.03% | 0.06% | 99% | 100% |
| Overall | 0.01% | 0.03% | 0.01% | 100% | 100% |
| Class D | -0.02% | -0.07% | 0.03% | 100% | 99% |
| Class F (optional) | 0.00% | -0.02% | -0.01% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.05% | -0.12% | 100% | 99% |
| Class C | -0.04% | 0.02% | 0.13% | 100% | 100% |
| Class E | -0.05% | 0.47% | 0.41% | 101% | 100% |
| Overall | -0.03% | 0.10% | 0.10% | 100% | 100% |
| Class D | 0.01% | 0.28% | -0.07% | 100% | 101% |
| Class F (optional) | -0.02% | 0.24% | 0.51% | 99% | 101% |

Table 2 shows the simulation results of the proposed extended motion information comparisons involving the AIF flag in both configurations. The VTM-6.0 performances are unchanged in random access (0.00%) and are improved in low delay B (-0.04%) while the encoding/decoding time are still not impacted.

Table 2: summary of proposed extended motion information comparison (test b).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 100% | 99% |
| Class A2 | 0.02% | 0.07% | 0.05% | 99% | 100% |
| Class B | -0.01% | 0.05% | -0.10% | 100% | 99% |
| Class C | -0.01% | -0.03% | 0.08% | 100% | 100% |
| Overall | 0.00% | 0.02% | 0.00% | 99% | 99% |
| Class D | -0.01% | -0.02% | 0.07% | 100% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.02% | -0.09% | 100% | 99% |
| Class C | -0.05% | 0.12% | -0.09% | 100% | 100% |
| Class E | -0.11% | 0.35% | -0.03% | 99% | 100% |
| Overall | -0.04% | 0.12% | -0.07% | 100% | 100% |
| Class D | 0.05% | 0.35% | -0.06% | 100% | 99% |
| Class F (optional) | -0.01% | 0.39% | 0.33% | 100% | 101% |

# Conclusion

The proposed motion information comparison involving the alternative interpolation filter flag shows BD-rate changes of 0.01% in RA and -0.03% in LDB configuration (test a). BD-rates can be improved to 0.00% in RA and -0.04% in LDB by extending the proposed motion information comparison to HMVP candidates (test b).

It is suggested to adopt the proposed extended motion information comparison into future VTM-7.0.

# References

- Text of Versatile Video Coding (Draft 6), document JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- A.Henkel et al., “CE4: Switchable interpolation filter,” document JVET-O0057, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).