| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0669-v1 |
| --- | --- |

| Title: | AHG6: Wrap-around motion vector prediction at the picture boundary |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) | Minhun Lee Jongseok Lee Juntaek Park Donggyu Sim Seung-Jun Oh Woong Lim Gun Bang | Email: | minhun@kw.ac.kr, suk2080@kw.ac.kr, juntaek@kw.ac.kr, dgsim@kw.ac.kr, sjoh@kw.ac.kr, woong.lim@etri.re.kr, gbang@etri.re.kr, |
| Source: | Kwangwoon University (KWU) and ETRI |  |  |

_____________________________

- Abstract

In this contribution, a wrap-around motion vector prediction at the picture boundary is proposed. The proposed method adds a wrap-around motion vector candidate with the above left or above right motion vector candidate at a picture boundary for coding efficiency. Simulation results show that the wrap-around motion vector prediction at the picture boundary yields 0.xx%, 0.xx%, and 0.xx% BD-rate gain in end-to-end WS-PSNR with no complexity burden for PERP with horizontal geometry padding of reference pictures.

Introduction

In WD6[1], a merge candidate list is constructed for inter skip/merge modes and a AMVP candidate list is also constructed for inter AMVP mode using the motion vectors of the neighboring blocks to decode the current CU’s motion vectors, and also, sub-block based prediction methods has been adopted, sub-block temporal motion vector prediction, which constructing motion vector candidate using neighboring block of current CU.

When the position of the neighboring block is out of the left or right picture boundaries or the position of neighboring block’s motion vector is outside the boundary of picture, the motion vector candidate is excluded from the candidate list. Therefore, in the case of the CU adjacent to the picture boundary, the coding efficiency is reduced as compared with the other blocks due to the lack of merge candidates.

To alleviate this problem, this contribution employs a wrap-around motion vector prediction candidate which is a motion vector of the block of wrap-around offset position into the candidate list for the blocks adjacent to the picture boundary.

The boundary of the picture, in general, belongs to a background, and the motion of the background is likely to be denoted a global motion. If so, the motions of the left and right boundary regions in a picture are similar. In addition, the picture reformed om ERP or PERP formats has higher motion similarity than in non-360 pictures because the left and right picture boundaries are connected to each other. In VVC6.0, the horizontal wrap-around motion compensation technology has already been adopted [2].

WD6

In WD6, the process of constructing the spatial and temporal candidate list for merge/skip and AMVP modes of the current CU proceeds in the order of A0, B0, B1, A1, B2 and C0, C1 in Fig. 1, Fig. 2, respectively.

Figure 1. Positions of spatial candidate in WD6[1]

Figure 2. Candidate positions for temporal candidate, C0 and C1 in WD6[1]

And the process of constructing sbTMVP (sub-block Temporal Motion Vector Prediction) candidate for the current CU proceeds as follow:

- The temporal motion vector is derived by searching a motion vector pointing to collocated picture from the motion information of left spatial neighboring block A1 in Fig. 3.

- The location of collocated block is derived based on the location of current CU and the temporal motion vector of 1.

- If the collocated block of current CU is encoded of inter prediction, the collocated motion vector is derived and assigned to the motion vector of current sub-block.

Figure 3. The sbTMVP process in WD6[1]

If the current CU is at the left picture boundary, at merge/skip and AMVP modes, MVs at A0, A1 and B2 in Fig. 1. cannot be used as merge and AMVP candidates. On the contrary, MVs at B1 and C0 in Fig. 1. cannot be used as merge/skip and AMVP candidates if the current CU is adjacent to the right picture boundary.

In sbTMVP mode, If the position of temporal motion vector which is derived by searching a motion vector pointing to collocated picture from the motion information of left spatial neighboring block A1 in Fig. 3. or the location of collocated block which is derived based on the location of current CU and the temporal motion vector is at the out of left or right boundary, then the temporal motion vector or the location of collocated block is clipped in the picture boundary.

Proposed Methods

In the proposed method, the motion vector of the wrap-around position by adding sps_ref_wraparound_offset[3] or subtracting it from the position of the out-of-boundary candidate in the x direction is added in the merge/skip, AMVP and sbTMVP modes for constructing motion vector candidates list.

CU

CU

B1

B2

C0

B1

C0

B2

Picture boundary

sps_ref_warparound_offset

sps_ref_warparound_offset

sps_ref_warparound_offset

Figure 4. Example of wrap-around motion vector candidate at the picture boundary in merge/skip and AMVP modes

Motion vector of the wrap-around position with sps_ref_wraparound_offset[3] for B1, B2 or C0 in Fig. 4 locations of merge/skip and AMVP modes can be used at the picture boundary.

And in the same way, in the sbTMVP mode, the temporal motion vector or the location of collocated block of the wrap-around position by adding sps_ref_wraparound_offset[3] or subtracting it from the out-of-boundary position the x direction can be used adding the motion vector candidate from the right position, as shown Fig. 5 and Fig. 6.

Co-located CU

Reference picture

Picture boundary

sps_ref_warparound_offset

sps_ref_warparound_offset

Co-located CU

Figure 5. Example of wrap-around motion vector candidate at sbTMVP mode

Co-located CU

Picture boundary

Reference picture

Co-located CU

sps_ref_warparound_offset

sps_ref_warparound_offset

Figure 6. Example of wrap-around motion vector candidate at sbTMVP mode

Note that since the PERP format uses the horizontal geometry padding, the wrap-around offset (ERP width)[3] which is not the picture width should be used to find the corresponding wrap-around motion vector candidate.

Because motion vectors of wrapped-around A0 and A1 in Fig 4 positions for merge/AMVP modes have not decoded, they are not used for the proposed method.

Simulation Results

The proposed method is implemented on VTM-6.0 with 360Lib-9.1 software and tested under common test conditions[4]. Table 1 show RD performance and running time of the test model with the proposed wrap-around motion vector prediction at the picture boundary over VTM-6.0 with 360Lib-9.1.

Table 1. RD performance of wrap-around motion vector prediction at the picture boundary  on the top of VTM-6.0 with 360Lib-9.1

| Format: PERP |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference: VTM-6.0 |  | Over VTM-6.0 |  |  |  |  |
| Tested: Proposed |  | Y | U | V | EncT | DecT |
| S1 | SkateboardInLot | -0.06% | -0.20% | -0.41% | 100% | 100% |
|  | ChairLift | 0.00% | 0.04% | 0.03% | 100% | 99% |
|  | KiteFlite | -0.02% | 0.00% | 0.07% | 100% | 100% |
|  | Harbor | -0.01% | -0.05% | -0.13% | 100% | 100% |
|  | Trolley | -0.03% | 0.00% | -0.02% | 100% | 100% |
|  | GasLamp | -0.01% | -0.01% | 0.00% | 101% | 99% |
| S2 | Balboa | 0.00% | -0.02% | -0.41% | 100% | 100% |
|  | Broadway | -0.01% | -0.04% | -0.01% | 101% | 100% |
|  | Landing2 | 0.00% | -0.03% | -0.15% | 100% | 100% |
|  | BranCastle2 | -0.02% | -0.09% | -0.12% | 100% | 100% |
| Overall |  | -0.02% | -0.04% | -0.12% | 100% | 100% |

Patent rights declaration(s)

ETRI and Kwangwoon university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

References

- Benjamin Bross, Jianle Chen, Shan Liu “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019, Gothenburg, SE.

- Philippe Hanhart, Yuwen He, Yan Ye “CE13: PERP with horizontal geometry padding of reference pictures (Test 3.3)”, JVET-L0231, October 2018, Macau, CN.

- Byeongdoo Choi, Weiwei Feng, Shan Liu “AHG8: On wrap around motion compensation”, JVET-M0225, Jan 2019, Marrakech, MA.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, “ JVET common test conditions and evaluation procedures for 360° video”, JVET-L1012, October 2018, Macau, CN.