| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1007 |
| --- | --- |

| Title: | Crosscheck of JVET-P0412: CE5-related: On further modification of Hadamard filter from CE5-3.2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck |  |  |
| Author(s) or Contact(s): | Jacob Ström | Email: | jacob.strom@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A partial crosscheck of JVET-P0412 has been carried out. A bug in the original software was found and reported, specifically affecting RA. Therefore, in a first round, the AI/LDB/LDP configurations were tested, and for those the BDR figures are reported to match. An updated version of the software that worked also for RA was provided. For the updated software, the BDR figures are reported to match for RA. It is also reported that while the anchor uses SAO in about 50% of the frames, the proposed method uses SAO in about 20% of the frames.

# Timeline

The crosscheck was requested on Wed Sept 25 and obtained the code was sent on Sat Sept 28. A bug was found and was reported on Sun Sept 29. This affected all four configurations (AI, RA, LDB and LDP) but affected RA the most since it was not possible to encode bit streams in parallel without getting decoder mismatches. A bug-fixed version of the code was obtained on Wed Oct 2. Since this was after the start of the meeting, we have not had the possibility to run simulations on all the tests.

# BD rate results

Due to the delay in obtaining a bug-free version of the code, only a partial cross-check was done on the updated code, for RA, test1. Here all the BDR numbers match.

|  | Over VTM-6.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | 0.38% | 0.19% | 105% | 106% |
| Class A2 | -0.59% | 0.37% | 0.36% | 103% | 99% |
| Class B | -0.39% | 0.38% | 0.42% | 108% | 106% |
| Class C | -0.47% | 0.32% | 0.41% | 107% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.42% | 0.36% | 0.36% | 106% | 105% |
| Class D | -0.63% | 0.14% | 0.35% | 106% | 103% |
| Class F | -0.21% | 0.27% | 0.39% | 118% | 108% |

AI, LDB and LDP was possible to encode with the original version of the code. Also in this case, all BDR figures match:

AI:

|  | Over VTM-6.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | 0.56% | 0.45% | 106% | 107% |
| Class A2 | -0.49% | 0.63% | 0.43% | 106% | 105% |
| Class B | -0.32% | 0.52% | 0.52% | 105% | 101% |
| Class C | -0.54% | 0.34% | 0.55% | 109% | 104% |
| Class E | -0.47% | 0.25% | 0.25% | 109% | 107% |
| Overall | -0.44% | 0.46% | 0.46% | 107% | 104% |
| Class D | -0.47% | 0.50% | 0.38% | 107% | 100% |
| Class F | -0.28% | 0.42% | 0.38% | 102% | 103% |

LDB:

|  | Over VTM-6.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | 1.00% | 0.77% | 101% | 102% |
| Class C | -0.40% | 0.53% | 0.87% | 100% | 99% |
| Class E | 0.02% | 0.94% | 1.24% | 101% | 98% |
| Overall | -0.16% | 0.83% | 0.92% | 101% | 100% |
| Class D | -0.60% | 1.51% | 1.28% | 104% | 103% |
| Class F | 0.02% | 0.60% | 1.09% | 100% | 103% |

LDP:

|  | Over VTM-6.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | 0.89% | 1.29% | 103% | 102% |
| Class C | -0.43% | 0.55% | 0.84% | 103% | 100% |
| Class E | -0.17% | 0.20% | 1.11% | 97% | 96% |
| Overall | -0.27% | 0.60% | 1.09% | 102% | 100% |
| Class D | -0.65% | 0.67% | 0.92% | 101% | 100% |
| Class F | -0.08% | 0.37% | 1.37% | 101% | 100% |

# SAO turned off

Comparing the Hadamard CE contribution with SAO turned on (CE5-3.2.1), and the Hadamard CE contribution with SAO turned off (CE5-3.2.2), it is clear that the BDR performance of the CE5-3.2.1 method is degraded when combined with SAO. The current proposal helps this situation in several ways. However, we have noticed that the proposal has the side-effect of turning off SAO in more cases than the anchor does. In this crosscheck, we compared the number of frames where SAO is enabled in the anchor, compared to the number of frames where SAO is enabled in the proposed method. The result is that SAO is used in 50% of the frames in the anchor, versus around 20% of the frames for the proposed method. This analysis was performed on LDB for the original version of the code (obtained on Sat Sept 28). Any subjective gain from SAO would therefore risk being diminished in this proposal.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).