| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0051 |
| --- | --- |

| Title: | Analysis of the Energy-Time Relationship for VVC and HEVC Software Video Decoders |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informative |  |  |
| Author(s) or Contact(s): | Matthias Kränzler Christian Herglotz André Kaup | Tel: Email: | +49 9131 85 27105 matthias.kraenzler@fau.de christian.herglotz@fau.de andre.kaup@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander Universität Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution compares the decoding energy and time of VTM-7.0 with HM-16.20. The measurements are evaluated with different metrics of the Bjøntegaard-Delta calculus. The evaluation of the decoding energy measurements reveals that the energy demand is increased by up to 83% and the decoding time by up to 71% for the VTM-7.0 decoder compared to HM-16.20. Finally, the decoding energy demands are modeled exploiting the decoding times of the bit streams with a mean accuracy of 1.41% for HM-16.20 and 1.05% for VTM-7.0, respectively.

# Introduction

Conventional assessment of the computational complexity of encoding and decoding is usually performed using the execution time returned by Linux functions or C++-commands. However, it is useful to also consider other metrics like the power or the energy consumption, which are of high interest for end-users. A method for measuring and evaluating the energy requirements of encoding and decoding was proposed in [1] and [2].

This document presents an analysis of the relation between the decoding time and the decoding energy and provides the following insights:

- Using a simple linear model, the decoding energy can be estimated accurately for the reference software implementations using the decoding time as an input parameter.

- For FFmpeg, the time model has an estimation error of over 50%, which shows the need for an alternative modeling function (e.g. bit stream feature-based models)

Chapter 2 presents decoding energy and decoding time results from measurements. Chapter 3 shows a linear model describing the mathematical relation between the decoding energy and the decoding time.

Figure 2: Diagram of the energy and time demand of the VTM decoder. Each blue marker represents the decoding time and the decoding energy of one bit stream. The red line corresponds to the linear estimation function.

Decoding Time and Energy Dependency

Next to a joint assessment of the decoding energies and decoding times, it is interesting to investigate the relation between both. In Figure 2, measured energies of the VTM-decoder are shown by blue markers. We can see that energy and time have an approximately linear relationship. The red curve in the diagram shows the fitted curve, which will be explained in detail in the following. According to [3], the decoding energy can be estimated by

| Edec=E0+P⋅tⅆec , | (3.1) |
| --- | --- |

where E0 is a constant energy offset that is needed to initialize the decoding process, P is the constant power during the decoding of a bit stream, and tⅆec is the duration of the decoding process. For the training of the parameters of (3.1), a least-squares curve fitting is performed with a trust-region reflective algorithm [4]. In order to evaluate the modeling accuracy, the mean estimation error  ε is calculated by

| ε =1Nn=0N-1Edec,n-Edec,nEdec,n , | (3.2) |
| --- | --- |

where N is the number of bit streams in the evaluated setup, n the index of the bit stream within the setup, Edec,n is the estimated decoding energy for a specific bit stream n, and Edec,n is the measured decoding energy for a specific bit stream n.

With this model, the HM and the VTM setup have an estimation error of 1.41% and 1.05%, respectively. The modeled values of P are 21.26 W for HM and 22.09 W for VTM, which corresponds to an increase of approximately 4%. The higher value of P indicates that the decoding of equal bit streams with the same decoding time results in a higher decoding energy demand for the VTM decoder. Therefore, the increased value of P explains the difference of the higher values of Bjøntegaard-Delta Decoding-Energy (BDDE) compared to Bjøntegaard-Delta-Decoding-Time (BDDT) in [1].

Another possibility to decode HEVC bit streams is FFmpeg, which uses openHEVC as an optimized decoder [5]. In this contribution, we use FFmpeg 4.2. With FFmpeg it is possible to decode HEVC bit streams with multiple cores. In Figure 3, the decoding energies and respective decoding times are shown by blue markers and the estimated decoding energy by a red curve. The modeling of the decoding energy for FFmpeg has an estimation error of 51.02%. Thus, the linear relationship between decoding energy and time can not be assumed for FFmpeg, which can be seen by the estimation errors between the blue markers and the red curve. Hence, for other implementations, the decoding time is not sufficient for accurately estimating the decoding energy, such that other approaches like the use of feature-based models [6] must be applied

Figure 3: Diagram of the energy and time demand of the FFmpeg decoder. The red curve corresponds to the estimated decoding energy demand and the blue markers correspond to the measured decoding energies.

Conclusion

With the help of decoding energy measurements, it is determined that the VTM decoder has a disproportionately increased decoding energy demand in comparison to the increase in the decoding time.  Furthermore, it is shown that the decoding time can be used for accurate estimation of the decoding energy for the HM- and VTM-decoder. For FFmpeg, the modeling of the decoding energy is highly inaccurate. For a practical implementation like FFmpeg, the decoding time is not suited for decoding energy estimation such that other means are needed. One solution is the use of feature-based models as presented in [6].

# References

[1] M. Kränzler, C. Herglotz, A. Kaup, “AHG13: Decoding Time and Energy Assessment of VTM-7.0”, JVET-Q0050, Jan. 2020.

[2] C. Herglotz, A. Kaup, “Energy Assessment for Video Decoding”, JVET-K0106-v3, Jul. 2018.

[3] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup. Modeling the Energy Consumption of the HEVC Decoding Process. IEEE Transactions on Circuits and Systems for Video Technology, Vol. 28(1), 2018.

[4] T. F. Coleman and Y. Li. An interior trust region approach for nonlinear minimization subject to bounds. SIAM J. Optim., vol. 6, no. 2, pp. 418–445, 1996.

[5] Fast Forward MPEG (FFmpeg) version 4.2. Accessed: 2019-10-31. [Online]. Available: http://ffmpeg.org

[6] M. Kränzler, C. Herglotz, A. Kaup, “Modeling of the Decoding Energy for VTM-7.0 with a Bit Stream Feature-Based Model”, JVET-Q0052, Jan. 2020.

# Patent rights declaration

The Friedrich-Alexander-Universität Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.