| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0055 |
| --- | --- |

| Title: | On MTS index signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Draft VVC standard allows MTS transform coefficients only inside the lowest frequency 16x16 area of a transform block. However, signalling of the mts_idx is conditioned to the last coefficient position which does not guarantee existence of coefficients only inside the valid area. Thus, in some cases mts_idx is signalled even though it could be inferred to be zero. There is also a possibility for a non-conforming encoder to signal MTS transform coefficients in positions outside the 16x16 area and still indicate one of the MTS modes to be active.

It is proposed to add normative checks to the transform coefficient parsing and indicate mts_idx only if all non-zero transform coefficients are within the valid MTS coefficient area. Bitrate and complexity impact of the proposed change is asserted to be negligible. In the case of AI, RA and LD-B the average BD-rate changes are reported to be 0.00%, 0.00 % and 0.00 %, respectively.

# Proposed change

In order to avoid the possibility of indicating transform coefficients outside of the valid MTS coefficient area when MTS is active, it is proposed to change the current last significant position based validity check with a coefficient or coefficient subblock based check. In the proposed text the check is performed at the coefficient subblock level, but identical operation can be also implemented with checking for each coefficient individually if any of those fall outside of the valid MTS area.

# Proposed specification text

residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {

…

if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 )

MtsZeroOutSigCoeffFlag = 0

QState = 0

for( i = lastSubBlock; i >= 0; i− − ) {

startQStateSb = QState

xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ]

yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ]

inferSbDcSigCoeffFlag = 0

if( i < lastSubBlock && i > 0 ) {

coded_sub_block_flag[ xS ][ yS ]

inferSbDcSigCoeffFlag = 1

}

if( ( coded_sub_block_flag[ xS ][ yS ] ¦ ¦ i = = lastSubBlock ) && ( xS > 3 ¦ ¦ yS > 3 ) && cIdx = = 0 ) {

MtsZeroOutSigCoeffFlag = 0

}

…

# Objective results

Results according to the JVET CTC are provided in the attached spreadsheet. The proposed changes appear to have negligible impact in both coding efficiency and runtimes. Summary of the results is provided below.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | -0.03 % | 0.01 % | 99 % | 100 % |
| Class A2 | -0.01 % | -0.01 % | 0.00 % | 99 % | 100 % |
| Class B | 0.00 % | 0.01 % | -0.03 % | 99 % | 100 % |
| Class C | 0.00 % | 0.00 % | 0.00 % | 100 % | 100 % |
| Class E | 0.00 % | 0.00 % | -0.03 % | 100 % | 101 % |
| Overall | 0.00 % | 0.00 % | -0.01 % | 100 % | 100 % |
| Class D | 0.00 % | 0.03 % | 0.00 % | 100 % | 101 % |
| Class F | 0.00 % | 0.01 % | 0.00 % | 100 % | 103 % |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01 % | -0.07 % | 0.00 % | 100 % | 99 % |
| Class A2 | -0.05 % | 0.02 % | -0.02 % | 100 % | 99 % |
| Class B | 0.02 % | 0.02 % | 0.06 % | 100 % | 98 % |
| Class C | 0.00 % | -0.04 % | 0.03 % | 100 % | 98 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | -0.01 % | 0.02 % | 100 % | 98 % |
| Class D | 0.00 % | 0.01 % | -0.03 % | 100 % | 100 % |
| Class F | 0.00 % | -0.02 % | -0.02 % | 100 % | 99 % |
|  |  |  | Low delay B |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00 % | -0.15 % | 0.01 % | 101 % | 101 % |
| Class C | 0.01 % | -0.12 % | -0.05 % | 101 % | 101 % |
| Class E | 0.01 % | -0.01 % | -0.07 % | 101 % | 100 % |
| Overall | 0.00 % | -0.10 % | -0.03 % | 101 % | 101 % |
| Class D | 0.01 % | -0.05 % | 0.04 % | 101 % | 100 % |
| Class F | -0.01 % | -0.03 % | 0.01 % | 101 % | 101 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).