| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0056_v2 |
| --- | --- |

| Title: | CE3-related: Level prediction for transform skip samples |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema | Email: | first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Current draft VVC standard uses level prediction for transform skip samples when entropy coding is in non-bypass mode. In the current approach, the absolute value of a transform skip sample is predicted by selecting the maximum of the absolute values of its top and left neighbors. In bypass mode this “absLevel prediction” is disabled as in that case the prediction process was already earlier reported to have a negative impact on coding efficiency.

This contribution proposes to use an average based prediction for the values of transform skip coefficients when entropy coding engine is in bypass mode. This is reported to have -3.07 %, -1.05 % and -0.87 % BD-rate impact in lossless AI, RA and LD-B configurations, respectively. In lossy coding, for both standard QP range and low QP range, all configurations are reported have average coding efficiency impact within 0.01 % from the anchors.

Additional results with different configurations of the proposed change and CE3 tests are provided. For example, when the proposed change is applied together with CE3-2.8c and the VTM absLevel prediction for non-bypass transform skip samples is switched off, the lossless coding results are reported as -8.67 %, -5.20 % and -4.60 % for AI, RA and LD-B configurations, respectively.

# Proposed change

It is proposed to apply average based sample value prediction for transform skip coefficients in the transform skip residual coding when operating in bypass mode. That is, when the scan position of a transform skip sample has exceeded the position of “lastScanPosPass1” in “residual_ts_coding” function.

The proposed sample prediction process uses the same reference samples as the existing absLevel prediction process for the non-bypass samples. The proposed bypass sample prediction is enabled when the TU is coded in intra mode and when both the reference samples have the same sign.

# Proposed specification text

residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) {

…

/* remainder scan pass */

for( n = 0; n <= numSbCoeff − 1; n++ ) {

…

if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { (*)

absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0

absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0

predCoeff = Max( absLeftCoeff, absAboveCoeff )

if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 )

AbsLevel[ xC ][ yC ] = predCoeff

else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff )

AbsLevel[ xC ][ yC ] −= 1

}

if( CuPredMode[ chType ][x0][y0] = = MODE_INTRA && n > lastScanPosPass1) {

leftCoeff = xC > 0 ? TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC − 1 ][ yC ] : 0

aboveCoeff = yC > 0 ? TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC − 1 ] : 0

predCoeff = leftCoeff * aboveCoeff >= 0 ? ( leftCoeff + aboveCoeff ) / 2 : 0

level = predCoeff + ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ]

AbsLevel[ xC ][ yC ] = abs( level )

coeff_sign_flag[ n ] = level < 0 ? 1 : 0

}

TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ]

}

}

}

# Objective results (lossless)

## Proposed approach

Summary of results according to the CE3 test conditions for lossless coding are provided below. Detailed results can be found in the attached spreadsheet.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test |  | VTM7 | Test |  | VTM7 | Test |  |
| Class A1 | 2.2 | 2.3 | -1.85 % | 2.2 | 2.3 | -1.39 % |  |  |  |
| Class A2 | 1.6 | 1.6 | -2.51 % | 1.7 | 1.7 | -0.43 % |  |  |  |
| Class B | 2.2 | 2.3 | -3.62 % | 2.3 | 2.4 | -1.37 % | 2.3 | 2.4 | -1.25 % |
| Class C | 1.9 | 2.0 | -3.09 % | 2.4 | 2.5 | -0.85 % | 2.4 | 2.4 | -0.68 % |
| Class D | 1.9 | 2.0 | -3.79 % | 2.8 | 2.8 | -0.75 % | 2.7 | 2.8 | -0.48 % |
| Class E | 2.8 | 2.9 | -3.88 % |  |  |  | 3.1 | 3.1 | -0.49 % |
| Class F | 5.3 | 5.5 | -2.68 % | 33.7 | 33.9 | -0.68 % | 50.7 | 50.9 | -0.52 % |
| TGM | 11.8 | 11.9 | -0.04 % | 107.1 | 106.9 | 0.08 % | 124.9 | 124.7 | 0.10 % |
| Overall | 2.1 | 2.2 | -3.07 % | 2.2 | 2.2 | -1.05 % | 2.6 | 2.6 | -0.87 % |
| Enc Time[%] | 101 % |  |  | 101 % |  |  | 99 % |  |  |
| Dec Time[%] | 96 % |  |  | 102 % |  |  | 94 % |  |  |

## Extra test 1: Proposed + CE3-2.8c

In this experiment the proposed change is applied in CE3-2.8c software. CE3-2.8c includes the CE3-1.1c rice parameter modification for TSRC and applying BDPCM for luma and chroma.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test |  | VTM7 | Test |  | VTM7 | Test |  |
| Class A1 | 2.2 | 2.4 | -6.70 % | 2.2 | 2.4 | -6.16 % |  |  |  |
| Class A2 | 1.6 | 1.8 | -11.43 % | 1.7 | 1.8 | -7.38 % |  |  |  |
| Class B | 2.2 | 2.3 | -6.81 % | 2.3 | 2.4 | -3.05 % | 2.3 | 2.4 | -2.96 % |
| Class C | 1.9 | 2.1 | -7.77 % | 2.4 | 2.5 | -3.57 % | 2.4 | 2.5 | -3.36 % |
| Class D | 1.9 | 2.1 | -10.04 % | 2.8 | 2.9 | -3.79 % | 2.7 | 2.8 | -3.37 % |
| Class E | 2.8 | 3.1 | -10.07 % |  |  |  | 3.1 | 3.3 | -5.10 % |
| Class F | 5.3 | 6.0 | -9.76 % | 33.7 | 36.1 | -5.40 % | 50.7 | 53.3 | -4.75 % |
| TGM | 11.8 | 13.0 | -8.63 % | 107.1 | 118.5 | -8.18 % | 124.9 | 137.4 | -7.57 % |
| Overall | 2.1 | 2.3 | -8.32 % | 2.2 | 2.3 | -4.67 % | 2.6 | 2.7 | -3.63 % |
| Enc Time[%] | 123 % |  |  | 112 % |  |  | 108 % |  |  |
| Dec Time[%] | 94 % |  |  | 99 % |  |  | 101 % |  |  |

## Extra test 2: Proposed + CE3-2.8c + trSkip absLevel prediction off

In this experiment the proposed change is applied in CE3-2.8c software, and in addition the VTM absLevel prediction for non-bypass transform skip samples is switched off. Switching the absLevel prediction off could be implemented with the following change in the table given in the specification text section (substituting the line marked with (*)) and setting variable absLevelPredictionEnabled to false based on e.g. SPS level signalling:

if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 && absLevelPredictionEnabled ) {

Switching the VTM level prediction off seems to provide on average 0.5 – 1 % improvement in natural lossless video, but 1 – 4 % loss in lossless TGM category.

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test |  | VTM7 | Test |  | VTM7 | Test |  |
| Class A1 | 2.2 | 2.4 | -6.87 % | 2.2 | 2.4 | -6.33 % |  |  |  |
| Class A2 | 1.6 | 1.8 | -11.50 % | 1.7 | 1.8 | -7.47 % |  |  |  |
| Class B | 2.2 | 2.3 | -7.19 % | 2.3 | 2.4 | -3.61 % | 2.3 | 2.4 | -3.53 % |
| Class C | 1.9 | 2.1 | -8.21 % | 2.4 | 2.6 | -4.64 % | 2.4 | 2.5 | -4.43 % |
| Class D | 1.9 | 2.1 | -10.24 % | 2.8 | 2.9 | -4.88 % | 2.7 | 2.9 | -4.48 % |
| Class E | 2.8 | 3.1 | -10.69 % |  |  |  | 3.1 | 3.3 | -6.63 % |
| Class F | 5.3 | 6.0 | -9.97 % | 33.7 | 36.1 | -6.14 % | 50.7 | 53.2 | -5.46 % |
| TGM | 11.8 | 12.8 | -7.48 % | 107.1 | 112.1 | -4.60 % | 124.9 | 128.1 | -3.26 % |
| Overall | 2.1 | 2.3 | -8.67 % | 2.2 | 2.3 | -5.20 % | 2.6 | 2.7 | -4.60 % |
| Enc Time[%] | 120 % |  |  | 109 % |  |  | 104 % |  |  |
| Dec Time[%] | 92 % |  |  | 100 % |  |  | 102 % |  |  |

## Extra test 3.0: Proposed + CE3-1.1c + CE3-2.5

In this experiment the proposed change is applied in combined CE3-1.1c + CE3-2.5 software. CE3-1.1c includes the rice parameter modification for TSRC and CE3-2.5 includes possibility to switch between TSRC and RRC at CU level (CE3_2_5 macro was set to 1 in the tests).

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test |  | VTM7 | Test |  | VTM7 | Test |  |
| Class A1 | 2.2 | 2.4 | -7.01 % | 2.2 | 2.4 | -6.80 % |  |  |  |
| Class A2 | 1.6 | 1.8 | -11.72 % | 1.7 | 1.9 | -8.75 % |  |  |  |
| Class B | 2.2 | 2.3 | -6.39 % | 2.3 | 2.5 | -4.48 % | 2.3 | 2.4 | -4.43 % |
| Class C | 1.9 | 2.1 | -7.35 % | 2.4 | 2.6 | -6.00 % | 2.4 | 2.6 | -5.79 % |
| Class D | 1.9 | 2.1 | -8.23 % | 2.8 | 2.9 | -5.80 % | 2.7 | 2.9 | -5.53 % |
| Class E | 2.8 | 3.0 | -7.48 % |  |  |  | 3.1 | 3.3 | -6.01 % |
| Class F | 5.3 | 5.7 | -6.76 % | 33.7 | 35.4 | -5.27 % | 50.7 | 52.7 | -4.77 % |
| TGM | 11.8 | 12.6 | -5.91 % | 107.1 | 117.0 | -7.31 % | 124.9 | 136.6 | -7.21 % |
| Overall | 2.1 | 2.3 | -7.78 % | 2.2 | 2.4 | -6.20 % | 2.6 | 2.7 | -5.28 % |
| Enc Time[%] | 111 % |  |  | 116 % |  |  | 113 % |  |  |
| Dec Time[%] | 107 % |  |  | 106 % |  |  | 108 % |  |  |

## Extra test 3.2: Extra test 3 with BDPCM for luma and chroma

This experiment is identical to test 3.0 except that BDPCM is applied in test 3.2. BDPCM encoder parameter is set to 2 in this test enabling BDPCM for both luma and chroma (value of 0 was used in extra test 3.0).

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7 | Test |  | VTM7 | Test |  | VTM7 | Test |  |
| Class A1 | 2.2 | 2.4 | -7.26 % | 2.2 | 2.4 | -7.09 % |  |  |  |
| Class A2 | 1.6 | 1.8 | -12.44 % | 1.7 | 1.9 | -9.07 % |  |  |  |
| Class B | 2.2 | 2.3 | -7.22 % | 2.3 | 2.5 | -4.83 % | 2.3 | 2.5 | -4.80 % |
| Class C | 1.9 | 2.1 | -8.11 % | 2.4 | 2.6 | -6.30 % | 2.4 | 2.6 | -6.13 % |
| Class D | 1.9 | 2.1 | -10.18 % | 2.8 | 2.9 | -6.28 % | 2.7 | 2.9 | -5.93 % |
| Class E | 2.8 | 3.1 | -10.28 % |  |  |  | 3.1 | 3.4 | -7.41 % |
| Class F | 5.3 | 6.0 | -10.17 % | 33.7 | 36.4 | -6.86 % | 50.7 | 53.5 | -6.07 % |
| TGM | 11.8 | 13.1 | -9.44 % | 107.1 | 119.0 | -8.81 % | 124.9 | 137.2 | -7.95 % |
| Overall | 2.1 | 2.3 | -8.81 % | 2.2 | 2.4 | -6.52 % | 2.6 | 2.7 | -5.90 % |
| Enc Time[%] | 128 % |  |  | 122 % |  |  | 120 % |  |  |
| Dec Time[%] | 101 % |  |  | 108 % |  |  | 106 % |  |  |

## Summary of extra tests and related CE tests

| CE3 test | Changes | BD PCM | AI |  |  | RA |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | CTC | Class F | TGM | CTC | Class F | TGM | CTC | Class F | TGM |
|  | Proposed | 0 | -3.07 % | -2.68 % | -0.04 % | -1.05 % | -0.68 % | 0.08 % | -0.87 % | -0.52 % | 0.10 % |
| 2.8c-2 |  | 2 | -6,80 % | -8,68 % | -8,97 % | -3,96 % | -5,16 % | -8,37 % | -3,18 % | -4,61 % | -7,77 % |
| 2.8c-2 | Test 1 | 2 | -8.32 % | -9.76 % | -8.63 % | -4.67 % | -5.40 % | -8.18 % | -3.63 % | -4.75 % | -7.57 % |
| 2.8c-2 | Test 2 | 2 | -8.67 % | -9.97 % | -7.48 % | -5.20 % | -6.14 % | -4.60 % | -4.60 % | -5.46 % | -3.26 % |
| 1.1c+2.5 |  | 0 | -5.22 % | -3.59 % | -5.89 % | -5.58 % | -4.34 % | -7.67 % | -4.96 % | -4.17 % | -7.58 % |
| 1.1c+2.5 | Test 3.0 | 0 | -7.78 % | -6.76 % | -5.91 % | -6.20 % | -5.27 % | -7.31 % | -5.28 % | -4.77 % | -7.21 % |
| 1.1c+2.5 | Test 3.2 | 2 | -8.81 % | -10.17 % | -9.44 % | -6.52 % | -6.86 % | -8.81 % | -5.90 % | -6.07 % | -7.95 % |

# Objective results (lossy)

Results according to the JVET CTC are provided in the attached spreadsheets, separately for normal CTC QP range (22, 27, 32, 37) and the low QP range specified in the CE3 description (2, 7, 12, 17). The proposed approach appears to have negligible impact in both coding efficiency and runtimes for lossy coding. Summary of the results is provided below.

## CTC – Lossy – Normal QP range (22, 27, 32, 37)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | -0.03 % | 0.01 % | 99 % | 101 % |
| Class A2 | 0.00 % | 0.00 % | -0.02 % | 99 % | 100 % |
| Class B | 0.00 % | 0.03 % | -0.03 % | 99 % | 100 % |
| Class C | 0.01 % | 0.04 % | 0.08 % | 100 % | 98 % |
| Class E | 0.00 % | -0.01 % | 0.00 % | 99 % | 101 % |
| Overall | 0.00 % | 0.01 % | 0.01 % | 99 % | 100 % |
| Class D | -0.01 % | 0.04 % | -0.03 % | 99 % | 100 % |
| Class F | 0.01 % | 0.04 % | 0.07 % | 100 % | 101 % |
| TGM | 0.06 % | 0.06 % | 0.05 % | 99 % | 100 % |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01 % | -0.02 % | -0.01 % | 100 % | 99 % |
| Class A2 | 0.00 % | 0.09 % | 0.01 % | 100 % | 100 % |
| Class B | 0.02 % | -0.03 % | 0.08 % | 100 % | 99 % |
| Class C | 0.01 % | -0.11 % | -0.14 % | 100 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | 0.01 % | -0.03 % | -0.01 % | 100 % | 99 % |
| Class D | -0.01 % | -0.26 % | -0.10 % | 100 % | 99 % |
| Class F | 0.03 % | 0.11 % | -0.05 % | 100 % | 100 % |
| TGM | 0.05 % | -0.02 % | 0.04 % | 99 % | 100 % |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00 % | -0.23 % | 0.05 % | 100 % | 100 % |
| Class C | 0.02 % | -0.10 % | -0.20 % | 99 % | 97 % |
| Class E | -0.03 % | 1.01 % | -0.46 % | 99 % | 97 % |
| Overall | 0.00 % | 0.12 % | -0.16 % | 99 % | 98 % |
| Class D | -0.06 % | 0.45 % | 0.03 % | 99 % | 100 % |
| Class F | 0.03 % | -0.37 % | 0.24 % | 99 % | 97 % |
| TGM | -0.07 % | -0.20 % | -0.12 % | 98 % | 98 % |

## CTC – Lossy – Low QP range (2, 7, 12, 17)

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 LowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | 0.03 % | 0.02 % | 100 % | 97 % |
| Class A2 | 0.00 % | -0.01 % | -0.01 % | 100 % | 99 % |
| Class B | -0.01 % | 0.00 % | 0.01 % | 100 % | 98 % |
| Class C | -0.01 % | -0.01 % | -0.01 % | 100 % | 96 % |
| Class E | -0.02 % | -0.03 % | -0.05 % | 100 % | 97 % |
| Overall | 0.00 % | 0.00 % | -0.01 % | 100 % | 98 % |
| Class D | 0.00 % | -0.03 % | -0.03 % | 100 % | 91 % |
| Class F | -0.17 % | 0.05 % | 0.03 % | 100 % | 94 % |
| TGM | 0.38 % | 0.30 % | 0.30 % | 100 % | 98 % |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 LowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | 0.02 % | 0.00 % | 99 % | 99 % |
| Class A2 | 0.00 % | 0.00 % | 0.00 % | 99 % | 99 % |
| Class B | 0.00 % | -0.01 % | -0.01 % | 99 % | 101 % |
| Class C | 0.00 % | -0.02 % | -0.06 % | 99 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | 0.00 % | -0.02 % | 99 % | 99 % |
| Class D | -0.01 % | -0.03 % | -0.09 % | 100 % | 98 % |
| Class F | -0.85 % | 0.10 % | 0.02 % | 99 % | 100 % |
| TGM | 0.33 % | 0.30 % | 0.31 % | 100 % | 98 % |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 LowQP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01 % | -0.01 % | 0.02 % | 99 % | 97 % |
| Class C | 0.00 % | 0.00 % | -0.01 % | 100 % | 93 % |
| Class E | -0.02 % | 0.02 % | 0.06 % | 100 % | 95 % |
| Overall | 0.00 % | 0.00 % | 0.02 % | 100 % | 95 % |
| Class D | -0.01 % | 0.01 % | -0.04 % | 100 % | 92 % |
| Class F | 0.04 % | -0.04 % | -0.12 % | 100 % | 93 % |
| TGM | 0.00 % | 0.04 % | 0.07 % | 100 % | 97 % |

# Software

## Software for extra tests 1 and 2

Software implementing the proposed change on top of the CE3-1.1/CE3-2.8 software package is provided as an attachment (JVET-Q0056-source-tests1to2.zip). In order to compile the software in different configurations, the following settings should be made in the TypeDef.h file.

Enable the proposed change (default configuration in the software):

#define TS_CODING_BYPASS_DELTA                            1 // JVET-Q0056

#define ENABLE_CHROMA_BDPCM                               0

#define TS_CODING_VTM_PRED_OFF                            0

#define CE3_1_1_c                                         0

Extra test 1: Proposed + CE3-2.8c

#define TS_CODING_BYPASS_DELTA                            1 // JVET-Q0056

#define ENABLE_CHROMA_BDPCM                               1

#define TS_CODING_VTM_PRED_OFF                            0

#define CE3_1_1_c                                         1

Extra test 2: Proposed + CE3-2.8c + absLevel prediction disabled

#define TS_CODING_BYPASS_DELTA                            1 // JVET-Q0056

#define ENABLE_CHROMA_BDPCM                               1

#define TS_CODING_VTM_PRED_OFF                            1

#define CE3_1_1_c                                         1

As both extra test 1 and extra test 2 enable Chroma BDPCM, also the VTM encoder option for BDPCM needs to be set to 2 for these tests as instructed by the CE3 test conditions.

## Software for extra tests 3.0 and 3.2

Software implementing the proposed change on top of the CE3-1.1 + CE2.5 software package is provided as an attachment (JVET-Q0056-source-test3.zip). Software can be compiled with its default settings in TypeDef.h file for both extra test 3.0 and 3.2.

As both extra test 3.2 enable Chroma BDPCM, also the VTM encoder option for BDPCM needs to be set to 2 for this tests as instructed by the CE3 test conditions.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).