| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0061_v2 |
| --- | --- |

| Title: | CE4-related: Combined Test of CE4-1, CE4-2 and CE4-3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Kevin Reuzé, Han Huang, Ted Hsieh, Vadim Seregin, Chun-Chi Chen, Hilmi Enes Egilmez, Wei-Jung Chien, Marta Karczewicz (Qualcomm) Ru-Ling Liao, Jie Chen, Yan Ye, Jiancong Luo (Alibaba) Liying Xu, Fangdong Chen, Li Wang (Hikvision) Zhipin Deng, Li Zhang, Hongbin Liu, Kai Zhang, Yang Wang (Bytedance) Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | han.gao@huawei.com kreuz@qti.qualcomm.com ruling.lrl@alibaba-inc.com xuliying5@hikvision.com zhipin.deng@bytedance.com blaeser@ient.rw.th-aachen.de |
| Source: | Huawei Technologies Co., LTD, Qualcomm Inc, Alibaba Group, Hikvision, Bytedance Inc., RWTH Aachen University |  |  |

_____________________________

# Abstract

CE4-1, CE4-2 and CE4-3 are all aiming to simplify the GEO design in the common based of CE4. In this proposal two combination are tested, i.e., CE4-1+CE4-2.1+CE4-3.1 (test1) and CE4-1+CE4-2.1+CE4-3.2 (test2)

The simulation results shows:

In test1, 0.03% RA, 0.12% LDB luma loss compared with CE4 common base; 0.25% RA, 0.59% LDB luma gain compared with VTM7.0; 0.63%.RA, 1.49% LDB luma gain compared with VTM7.0 with TPM off case.

In test2, 0.01% RA luma gain, 0.06% LDB luma loss compared with CE4 common base; 0.29% RA, 0.65% LDB luma gain compared with VTM7.0; 0.67%.RA, 1.55% LDB luma gain compared with VTM7.0 with TPM off case.

# Introduction

Geometric inter prediction is firstly proposed as a merge mode extension in [1] of 15th JVET Meeting in Gothenburg, SE. It agreed that the combined test proposal of [2], [3] are used as common base of CE4 [4] of 16th JVET Meeting in Brussels, BE. There are several CE tests are designed in purpose of reducing the complexity of CE4 common base.

CE4-1 is aiming to reduce the redundant GEO partition modes

CE4-2 is designed to reduce the number GEO allowed block shapes

CE4-3 is constraining SBT modes for GEO. CE4-3.1: Disallow SBT for GEO mode and CE4-3.2: Disallow SBT for GEO mode when displacement is 0.

# Combined Test

In this test, two combined test will be presented, i.e., CE4-1+CE4-2.1+CE4-3.1 (test1) and CE4-1+CE4-2.1+CE4-3.2 (test2).

# Simulation Results

Following tables show simulation results compared with CE4 common base, VTM7.0 and VTM7.0 with TPM off as the test condition in CE4 description [4]. Details results are attached in the excel sheets.

Table 1, Simulation results of CE4-1 + CE4-2.1 + CE4-3.1

| Anchor | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-CB | 0.03% | 0.01% | 0.02% | 98% | 98% | 0.12% | 0.07% | 0.39% | 98% | 96% |
| VTM7.0 | -0.25% | -0.35% | -0.32% | 101% | 99% | -0.59% | -0.60% | -0.38% | 101% | 96% |
| VTM7.0 TPMoff | -0.63% | -0.98% | -0.99% | 103% | 98% | -1.49% | -1.86% | -1.60% | 104% | 95% |

Table 2, Simulation results of CE4-1 + CE4-2.1 + CE4-3.2

| Anchor | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-CB | -0.01% | 0.04% | 0.06% | 98% | 98% | 0.06% | 0.15% | 0.24% | 98% | 97% |
| VTM7.0 | -0.29% | -0.31% | -0.28% | 101% | 99% | -0.65% | -0.53% | -0.53% | 101% | 97% |
| VTM7.0 TPMoff | -0.67% | -0.94% | -0.95% | 103% | 98% | -1.55% | -1.79% | -1.75% | 104% | 95% |

# Conclusion

The combined tests of CE4-1+CD4-2.1 +CE4-3.1 and CE4-1+CD4-2.1 +CE4-3.2 simplify the CE4 common based GEO design. It proposed to adopt one of the combinations in the next version of VVC draft.

# Reference

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, J. Sauer  “Non-CE4: Geometrical partitioning for inter blocks”, JVET-O0489, Gothenburg, SE, 3–12 July 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P0884, Geneva, CH, 1–11 October 2019

- K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz, R.-L Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0885, Geneva, CH, 1–11 October 2019

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.