| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0068 |
| --- | --- |

| Title: | CE3-1.3: Using RRC for lossless coding without state transition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Alican Nalci, Hilmi E. Egilmez, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Mohammed Golam Sarwer, Ruling Liao, Jiancong Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com analci@qti.qualcomm.com m.sarwer@alibaba-inc.com |
| Source: | Kwai Inc., MediaTek Inc., Qualcomm Incorporated, Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 3-1.3 [1]. In this CE test, two subtests are proposed to enable 1-bit high level syntax to select regular residual coding (RRC) or transform-skip residual coding (TSRC) since the cu_transquant_bypass_mode is not used in CE and the CE mandate. In the first test, one PPS flag and one slice flag are added to indicate which residual coding scheme is selected with VTM7[2]. In the second test, one syntax in picture header is added to indicate which residual coding scheme is selected and implemented in VTM7.1 [3].

Results are reported as below:

Test 1: PPS and slice flag

CTC: -5.23% AI. -5.58% RA, -4.97% LDB,

Class F: -1.33% AI, -2.35% RA, -2.36% LDB

Class TGM: 0.00% AI, 0.00% RA, 0.00% LDB

EncT: 93% AI, 97% RA, 98% LDB

DecT: 102% AI, 97% RA, 106% LDB

Test 2: Picture Header syntax in VTM7.1 [3]

CTC: -5.23% AI, -5.58% RA, -4.96% LDB

Class F: -1.46% AI, -2.39% RA, -2.36% LDB

Class TGM: 0.00% AI, 0.00% RA, 0.00% LDB

EncT: 89% AI, 97% RA, 97% LDB

DecT: 98% AI, 96% RA, 104% LDB

# Introduction

In current VVC [3] for lossless coding, TSRC is always applied since the lossless coding is achieved by forced transform skipped. However, it is asserted that TSRC is not optimal coding scheme for natural content. In this CE test, two subtests are implemented to select RRC/TSRC for VVC lossless. In the first subtest, a PPS flag and a slice flag are signaled to select residual coding scheme. In the second subtest, a picture header syntax is signaled and implemented in VTM7.1 [3].

# Simulation Results

In subtest1, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

## Test 1: PPS and Slice syntax

Table 1: Results of Subtest1 for VVC lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.42% | 2.2 | 2.4 | -6.25% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 1.9 | -8.13% |  |  |  |
| Class B | 2.2 | 2.3 | -4.01% | 2.3 | 2.4 | -3.84% | 2.3 | 2.4 | -3.85% |
| Class C | 1.9 | 2.0 | -4.97% | 2.4 | 2.6 | -5.36% | 2.4 | 2.6 | -5.29% |
| Class D | 1.9 | 2.0 | -4.63% | 2.8 | 2.9 | -5.25% | 2.7 | 2.9 | -5.24% |
| Class E | 2.8 | 2.9 | -3.82% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.3 | -1.33% | 33.7 | 33.8 | -2.35% | 50.7 | 50.8 | -2.36% |
| TGM | 11.8 | 11.8 | 0.00% | 107.1 | 107.1 | 0.00% | 124.9 | 124.9 | 0.00% |
| Overall | 2.1 | 2.2 | -5.23% | 2.2 | 2.3 | -5.58% | 2.6 | 2.7 | -4.97% |
| Enc Time[%] | 93% |  |  | 97% |  |  | 98% |  |  |
| Dec Time[%] | 102% |  |  | 97% |  |  | 106% |  |  |

In subtest2, it is implemented based on the VTM-7.1 reference software [3] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

## Test 2: Picture header

Table 2: Results of Subtest2

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.1 | TEST |  | VTM7.1 | TEST |  | VTM7.1 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.42% | 2.2 | 2.4 | -6.25% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 1.9 | -8.13% |  |  |  |
| Class B | 2.2 | 2.3 | -4.01% | 2.3 | 2.4 | -3.82% | 2.3 | 2.4 | -3.83% |
| Class C | 1.9 | 2.0 | -4.97% | 2.4 | 2.6 | -5.36% | 2.4 | 2.6 | -5.29% |
| Class D | 1.9 | 2.0 | -4.63% | 2.8 | 2.9 | -5.25% | 2.7 | 2.9 | -5.24% |
| Class E | 2.8 | 2.9 | -3.82% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.4 | -1.46% | 33.8 | 33.9 | -2.39% | 50.8 | 50.8 | -2.36% |
| TGM | 11.8 | 11.8 | 0.00% | 107.0 | 107.0 | 0.00% | 124.4 | 124.4 | 0.00% |
| Overall | 2.1 | 2.2 | -5.23% | 2.2 | 2.3 | -5.58% | 2.6 | 2.7 | -4.96% |
| Enc Time[%] | 89% |  |  | 97% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  | 104% |  |  |

# References

- T. Nguyen T. C. Ma A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).