| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0069 |
| --- | --- |

| Title: | CE3-1.4: On residual scanning order for lossless coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Xianglin Wang | Email: | {tsung-chuanma, yiwenchen, xiaoyuxiu, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment (CE) 3-1.4 [1]. In this CE test, it is proposed to test the rotation of residual blocks if regular residual coding (RRC) is applied for lossless residual.

Results are reported as below:

CTC: -5.63% AI. -5.72% RA, -5.09% LDB,

Class F: -1.86% AI, -2.45% RA, -2.40% LDB

Class TGM: 0.00% AI, 0.00% RA, 0.00% LDB

EncT (all class): 98% AI, 97% RA, 100% LDB

DecT (all class): 105% AI, 100% RA, 110% LDB

Additional result is provided. In this additional test, the anchor is VVC lossless with RRC as in CE 3-1.3 [1].

Additional result is reported as following:

CTC: -0.42% AI, -0.14% RA, -0.13% LDB

Class F: -0.55% AI, -0.11% RA, -0.05% LDB

Class TGM: 0.00% AI, 0.00% RA, 0.00% LDB

EncT (all class): 105% AI, 100% RA, 103% LDB

DecT (all class): 103% AI, 103% RA, 104% LDB

# Introduction

In current VVC [3] for lossless coding, the residual samples from intra prediction located at the bottom-right area of the block tend to have larger amplitude than those located at the top-left area, because of the distance between the reconstructed pixels that are used for prediction and the predicted pixels. When regular residual coding is applied for lossless residual coding, rotating the residual blocks (i.e. flipping the blocks along both horizontal and vertical direction) would place the statistically large amplitude residue at the top left area of the TU, which is more suitable for regular coefficient coding. In this CE, it is proposed to test the rotation of residual blocks if RRC is applied for lossless residual.

# Implementation

In this CE test, one PPS and one slice flag are signaled to indicate RRC is selected. Once the RRC is selected, the residual rotation is applied for all blocks.

# Simulation Results

In this test, it is implemented based on the VTM-7.0 reference software [2] and tested using test condition and evaluation criteria that are used in CE3 [1] are applied.

Table 2: Results of test for VVC lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VVC7.0 | TEST |  | VVC7.0 | TEST |  | VVC7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.62% | 2.2 | 2.4 | -6.41% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.87% | 1.7 | 1.9 | -8.16% |  |  |  |
| Class B | 2.2 | 2.3 | -4.33% | 2.3 | 2.4 | -3.95% | 2.3 | 2.4 | -3.95% |
| Class C | 1.9 | 2.1 | -5.62% | 2.4 | 2.6 | -5.58% | 2.4 | 2.6 | -5.48% |
| Class D | 1.9 | 2.0 | -5.34% | 2.8 | 2.9 | -5.40% | 2.7 | 2.9 | -5.33% |
| Class E | 2.8 | 2.9 | -4.59% |  |  |  | 3.1 | 3.3 | -6.48% |
| Class F | 5.3 | 5.4 | -1.86% | 33.7 | 33.8 | -2.45% | 50.7 | 50.8 | -2.40% |
| TGM | 11.8 | 11.8 | 0.00% | 107.1 | 107.1 | 0.00% | 124.9 | 124.9 | 0.00% |
| Overall | 2.1 | 2.3 | -5.63% | 2.2 | 2.4 | -5.72% | 2.6 | 2.7 | -5.09% |
| Enc Time[%] | 98% |  |  | 97% |  |  | 100% |  |  |
| Dec Time[%] | 105% |  |  | 100% |  |  | 110% |  |  |

In additional result, it is implemented based on the VTM-7.0 reference software [2] and the anchor is CE 3-1.3 [1]. Both anchor and test is applied with the test condition and evaluation criteria used in CE3 [1].

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-1.3 | CE3-1.4 |  | CE3-1.3 | CE3-1.4 |  | CE3-1.3 | CE3-1.4 |  |
| Class A1 | 2.4 | 2.4 | -0.20% | 2.4 | 2.4 | -0.16% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.01% | 1.9 | 1.9 | -0.04% |  |  |  |
| Class B | 2.3 | 2.3 | -0.34% | 2.4 | 2.4 | -0.12% | 2.4 | 2.4 | -0.11% |
| Class C | 2.0 | 2.1 | -0.69% | 2.6 | 2.6 | -0.23% | 2.6 | 2.6 | -0.20% |
| Class D | 2.0 | 2.0 | -0.73% | 2.9 | 2.9 | -0.15% | 2.9 | 2.9 | -0.09% |
| Class E | 2.9 | 2.9 | -0.80% |  |  |  | 3.3 | 3.3 | -0.07% |
| Class F | 5.3 | 5.4 | -0.55% | 33.8 | 33.8 | -0.11% | 50.8 | 50.8 | -0.05% |
| TGM | 11.8 | 11.8 | 0.00% | 107.1 | 107.1 | 0.00% | 124.9 | 124.9 | 0.00% |
| Overall | 2.2 | 2.3 | -0.42% | 2.3 | 2.4 | -0.14% | 2.7 | 2.7 | -0.13% |
| Enc Time[%] | 105% |  |  | 100% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  | 104% |  |  |

# References

- T. Nguyen T. C. Ma A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding”, JVET-P2023, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).