| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0098-v3 |
| --- | --- |

| Title: | On QP Adjustment for Adaptive Color Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba K. Kondo Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Kenji.Kondo@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In VTM-7.1, after applying QP adjustment (-5, -5, -3) for ACT, the quantization parameter qP for TS can be less than QpPrimeTsMin, and the qP for non-TS can be less than 0. To keep the qP in the valid range, this contribution proposes to a) apply the QP adjustment for ACT before clipping the qP for TS to QpPrimeTsMin, and b) clip the qP for non-TS to 0 after the QP adjustment for ACT.

For DualITreeOff at low QP condition (QP=2, 7, 12, 17), it is reported that BDrate-differences of (G, B, R) are (-0.17%, -0.38%, -0.33%) for AI, (-0.09%, -0.25%, -0.27%) for RA and (-0.11%, -0.34%, -0.31%) for LB; BDrate-differences of (Y, U, V) are (0.00%, 0.00%, 0.00%) for AI, (0.00%, 0.00%, -0.01%) for RA and (-0.02%, 0.01%, -0.01%) for LB

For DualITreeOn at low QP condition, it is reported that BDrate-differences of (G, B, R) are (0.00%, 0.00%, 0.00%) for AI, (-0.06%, -0.25%, -0.29%) for RA and (-0.09%, -0.29%, -0.27%) for LB; BDrate-differences of (Y, U, V) are (0.00%, 0.00%, 0.00%) for AI, (0.00%, -0.05%, -0.07%) for RA and (-0.05%, -0.19%, -0.13%) for LB.

In v3, presentation deck was attchead.

Introduction

In the last meeting, Adaptive Colour Transform (ACT) was adopt [1], where the QP adjustment of (-5, -5, -3) are applied to the transform residuals to compensate the dynamic range change of residual signals before and after color transform.

In VTM-7.1 [3], a quantization parameter qP is modified as below:

// for TS

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5 : 3): 0 )

// for non-TS

qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5 : 3): 0 )

In other words, after the QP adjustment for ACT, the quantization parameter qP for TS can be less than QpPrimeTsMin, and the qP for non-TS can be less then 0.

Proposal

Regarding the qP for TS, clipping the qP for TS to QpPrimeTsMin was introduced by JVET-O0919 [4] to prevent cases in which the effective quantization step size for TS becomes smaller than 1. Thus, it is proposed to apply the QP adjustment for ACT before clipping the qP for TS to QpPrimeTsMin as below:

qP = Max( QpPrimeTsMin, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5 : 3): 0 ) )

Regarding the qP for non-TS, the valid range of qP for non-TS is [0, 63+QpBdOffset]. Thus, it is proposed to clip the qP for non-TS to 0 after QP adjustment for ACTas below:

qP = Max( 0, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5 : 3): 0 ) )

Simulation Results

The proposed method has been implemented on top of VTM-7.1 and following simulations were conducted at low QP condition using RGB/YUV test sequences described in [1]. In the simulation, IBC, PLT, BDPCM, HashME and ColorTransform are enabled for both anchor and test (--IBC=1 --HashME=1 --BDPCM=2 --PLT=1 --ColorTransform=1).

- Test1: VTM-7.1 with DualITreeOff vs Proposal with DualITreeOff at low QP condition with first 100 frames (QP=2, 7, 12, 17)

- Test2: VTM-7.1 with DualITreeOn vs Proposal with DualITreeOn at low QP condition with first 100 frames (QP=2, 7, 12, 17)

Note that for RGB sequence, following options are also used:

--InputColourSpaceConvert=RGBtoGBR --SNRInternalColourSpace=1

Table 3-1: Results of Test1 (DualITreeOff, low QP condition)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.35% | -0.36% | -0.34% |
| RGB, mixed content, 1440p & 1080p | -0.32% | -0.53% | -0.46% |
| RGB, Animation, 720p | 0.00% | -0.38% | -0.30% |
| RGB, camera captured, 1080p | 0.00% | -0.23% | -0.25% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.00% | 0.00% | 0.00% |
| YUV, Animation, 720p | 0.00% | 0.00% | 0.00% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.13% | -0.19% | -0.09% |
| RGB, mixed content, 1440p & 1080p | -0.20% | -0.23% | -0.31% |
| RGB, Animation, 720p | 0.01% | -0.40% | -0.34% |
| RGB, camera captured, 1080p | -0.01% | -0.18% | -0.32% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.00% | 0.00% | 0.00% |
| YUV, Animation, 720p | 0.01% | -0.02% | -0.04% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.22% | -0.40% | -0.35% |
| RGB, mixed content, 1440p & 1080p | -0.21% | -0.39% | -0.45% |
| RGB, Animation, 720p | 0.00% | -0.35% | -0.24% |
| RGB, camera captured, 1080p | 0.00% | -0.21% | -0.20% |
| YUV, text & graphics with motion, 1080p & 720p | -0.03% | -0.01% | -0.03% |
| YUV, mixed content, 1440p & 1080p | -0.03% | 0.02% | 0.02% |
| YUV, Animation, 720p | -0.02% | 0.02% | -0.02% |
| YUV, camera captured, 1080p | 0.01% | 0.00% | 0.01% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |

Table 3-2 Results of Test2 (DualITreeOn, low QP condition)

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | 0.00% |
| RGB, mixed content, 1440p & 1080p | 0.00% | 0.00% | 0.00% |
| RGB, Animation, 720p | 0.00% | 0.00% | 0.00% |
| RGB, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.00% | 0.00% | 0.00% |
| YUV, Animation, 720p | 0.00% | 0.00% | 0.00% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.12% | -0.15% | -0.20% |
| RGB, mixed content, 1440p & 1080p | -0.09% | -0.25% | -0.29% |
| RGB, Animation, 720p | -0.06% | -0.40% | -0.34% |
| RGB, camera captured, 1080p | 0.00% | -0.20% | -0.30% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.00% | -0.01% |
| YUV, mixed content, 1440p & 1080p | 0.01% | 0.01% | 0.01% |
| YUV, Animation, 720p | -0.01% | 0.00% | 0.01% |
| YUV, camera captured, 1080p | 0.00% | -0.20% | -0.30% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.20% | -0.40% | -0.34% |
| RGB, mixed content, 1440p & 1080p | -0.17% | -0.31% | -0.34% |
| RGB, Animation, 720p | 0.05% | -0.27% | -0.17% |
| RGB, camera captured, 1080p | -0.05% | -0.18% | -0.22% |
| YUV, text & graphics with motion, 1080p & 720p | -0.20% | -0.40% | -0.34% |
| YUV, mixed content, 1440p & 1080p | -0.03% | -0.06% | 0.01% |
| YUV, Animation, 720p | 0.05% | -0.27% | -0.17% |
| YUV, camera captured, 1080p | -0.01% | 0.00% | -0.01% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 100% |  |  |

Conclusion

It is recommended to adopt the proposed method into the next VTM/VVC WD.

Reference

[1] X. Xiu, Y.-W Chen, T.-C Ma, H.-H J, X. Wang, “Support of adaptive color transform for 444 video coding in VVC,” JVET-P0517, Oct. 2019

[2] Y.-K Wang, B. Bross, V. Drugeon, J. Chen, “AHG2: Editorial input on VVC draft text, ” JVET-Q0041, Dec. 2019

[3] VTM-7.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.1

[4] M. Karczewicz, Y.-H. Chao, H. Wang, M. Coban, “Non-CE8: Minimum QP for Transform Skip Mode,” JVET-O0919, July 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Spec changes

Changes relative to JVET-Q0041v2 is highled in yellow.

- 8.7.3 Scaling process for transform coefficients

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable predMode specifying the prediction mode of the coding unit,

- a variable cIdx specifying the colour component of the current block.

Output of this process is the (nTbW)x(nTbH) array d of scaled transform coefficients with elements d[ x ][ y ].

The quantization parameter qP is derived as follows:

- If cIdx is equal to 0, the following applies:

qP = Qp′Y		(1129)

- Otherwise, if TuCResMode[ xTbY ][ yTbY ] is equal to 2, the following applies:

qP = Qp′CbCr		(1130)

- Otherwise, if cIdx is equal to 1, the following applies:

qP = Qp′Cb		(1131)

- Otherwise (cIdx is equal to 2), the following applies:

qP = Qp′Cr		(1132)

The quantization parameter qP is modified and the variables rectNonTsFlag and bdShift are derived as follows:

- If transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

qP = qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 )	(1133)

qP = Max(0, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5:3)  : 0 ) )	(1133)

rectNonTsFlag = ( ( ( Log2( nTbW ) + Log2( nTbH ) ) & 1 )  = =  1 ) ? 1 : 0	(1134)

bdShift = BitDepth + rectNonTsFlag +		(1135)
		( ( Log2( nTbW ) + Log2( nTbH ) ) / 2 ) − 5 + pic_dep_quant_enabled_flag

- Otherwise (transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

qP = Max( QpPrimeTsMin, qP ) − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? 5 : 0 )	(1136)

qP = Max( QpPrimeTsMin, qP − ( cu_act_enabled_flag[ xTbY ][ yTbY ] ? (cIdx<2 ? 5:3)  : 0 ) )	(1336)

rectNonTsFlag = 0		(1137)

bdShift = 10			(1138)

The variable bdOffset is derived as follows:

bdOffset = ( 1  <<  bdShift )  >>  1		(1139)

The list levelScale[ ][ ] is specified as levelScale[ j ][ k ] = { { 40, 45, 51, 57, 64, 72 }, { 57, 64, 72, 80, 90, 102 } } with j = 0..1, k = 0..5.

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- pic_scaling_list_present_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and lfnst_idx[ xTbY ][ yTbY ] is not equal to 0.

- Otherwise, the following applies:

- The variable id is derived based on predMode, cIdx, nTbW, and nTbH as specified in Table 36 and the variable log2MatrixSize is derived as follows:

log2MatrixSize = ( id < 2 ) ? 1 : ( id < 8 ) ? 2 : 3	(1140)

- The scaling factor m[ x ][ y ] is derived as follows:

m[ x ][ y ] = ScalingMatrixRec[ id ][ i ][ j ]
	with i = ( x << log2MatrixSize ) >> Log2( nTbW ),
		 j = ( y << log2MatrixSize ) >> Log2( nTbH )	(1141)

- If id is greater than 13 and both x and y are equal to 0, m[ 0 ][ 0 ] is further modified as follows:

m[ 0 ][ 0 ] = ScalingMatrixDCRec[ id − 14 ]	(1142)

NOTE – A quantization matrix element m[ x ][ y ] can be zeroed out when any of the following conditions is true

- x is greater than 32

- y is greater than 32

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and x is greater than 16

- The decoded tu is not coded by default transform mode (i.e. transform type is not equal to 0 ) and y is greater than 16

- The scaling factor ls[ x ][ y ] is derived as follows:

–	If pic_dep_quant_enabled_flag is equal to 1 and transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 0, the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ (qP + 1) % 6 ] ) << ( (qP + 1) / 6 )	(1143)

–	Otherwise (pic_dep_quant_enabled_flag is equal to 0 or transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1), the following applies:

ls[ x ][ y ] = ( m[ x ][ y ] * levelScale[ rectNonTsFlag ][ qP % 6 ] ) << ( qP / 6 )	(1144)

- When BdpcmFlag[ xTbY ][ yYbY ][ cIdx ] is equal to 1, dz[ x ][ y ] is modified as follows:

–	If BdpcmDir[ xTbY ][ yYbY ][ cIdx ] is equal to 0 and x is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x − 1 ][ y ] + dz[ x ][ y ])	(1145)

–	Otherwise, if BdpcmDir[ xTbY ][ yTbY ][ cIdx ] is equal to 1 and y is greater than 0, the following applies:

dz[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dz[ x ][ y − 1 ] + dz[ x ][ y ])	(1146)

- The value dnc[ x ][ y ] is derived as follows:

dnc[ x ][ y ] = ( dz[ x ][ y ] * ls[ x ][ y ] +bdOffset )  >>  bdShift	(1147)

- The scaled transform coefficient d[ x ][ y ] is derived as follows:

d[ x ][ y ] = Clip3( CoeffMin, CoeffMax, dnc[ x ][ y ] )	(1148)

Table 7-1 – Specification of the scaling matrix identifier variable id according to predMode, cIdx, nTbW, and nTbH

| max( nTbW, nTbH ) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| predMode = MODE_INTRA | cIdx = 0 (Y) |  | 2 | 8 | 14 | 20 | 26 |
|  | cIdx = 1 (Cb) |  | 3 | 9 | 15 | 21 | 21 |
|  | cIdx = 2 (Cr) |  | 4 | 10 | 16 | 22 | 22 |
| predMode = MODE_INTER (INTER, IBC) | cIdx = 0 (Y) |  | 5 | 11 | 17 | 23 | 27 |
|  | cIdx = 1 (Cb) | 0 | 6 | 12 | 18 | 24 | 24 |
|  | cIdx = 2 (Cr) | 1 | 7 | 13 | 19 | 25 | 25 |