| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0106-v1 |
| --- | --- |

| Title: | AHG16: Combination of LFNST with transform skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Tim Hellman Minhua Zhou Wade Wan | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The signalling of LFNST index currently includes a condition preventing the use of LFNST when luma transform skip is enabled, but no corresponding condition was added for chroma transform skip. It is asserted that this is a design inconsistency. It may have been an unintentional oversight when chroma transform skip was added in JVET-P0058.

Given that LFNST is signalled at the very end of the CU, it is asserted that allowing mixed LFNST / transform-skip CUs introduces a significant increase in buffering requirements. Therefore, this contribution recommends adding the corresponding conditions to prevent the use of LFNST with either luma or chroma transform skip. Experimental results reveal that the suggested changes do not impact coding efficiency.

# Introduction

The presence of the lfnst_idx syntax element is currently conditioned on the use of luma transform skip. Specifically, LFNST is not possible when luma transform skip is enabled, as indicated below (the highlighted “0” index represents luma).

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

With the adoption of JVET-P0058, transform skip is now possible in chroma TUs. However, note that no similar condition was put into place to prevent LFNST with chroma transform skip.

Allowing LFNST in combination with chroma transform skip, but not luma transform skip is an inconsistent decision. It may have been an unintentional oversight when adding chroma transform skip in JVET-P0058.

This contribution recommends resolving this inconsistency. Specifically, it recommends preventing LFNST with either luma or chroma transform skip, since the alternative (allowing combinations of LFNST and transform skip TUs within a CU) has significant buffering requirements, as discussed below.

# Buffering Requirements for Combined LFNST / TS

Given that the LFNST index is signalled at the very end of the CU, allowing the use of mixed LFNST / transform-skip TUs within a CU has significant buffering implications.

With the adoption of JVET-P0365 (adding a flag to disable scaling lists for LFNST), the dequantization process now depends on whether LFNST is enabled or not. Therefore, the very first process that needs to be performed on any decoded coefficients (i.e. dequantization) depends on one of the last syntax elements in the CU (lfnst_idx). All coefficients in the CU must be buffered until the LFNST index is determined.

As shown in the syntax table from Section 1, LFNST signalling also includes a condition preventing any coefficients outside the first 16 lowest-frequency coefficients (LfnstZeroOutSigCoeffFlag). Therefore, with non-transform skip TUs, at most 16 coefficients per component (48 coefficients) need to be buffered before the LFNST index can be determined.

Allowing mixed LFNST / transform-skip TUs would significantly increase this buffering requirement. Transform skipped TUs within an LFNST coded CU are not included in the “ZeroOutSigCoeff” condition mentioned above. Therefore, up to 1,024 coefficients per component from these transform-skip TUs would need to be buffered before the LFNST index could be determined.

For example, as shown in the figure below, if a non-transform skip luma TU has 16 coefficients, followed by two 32x32 chroma transform-skip TUs, this would require buffering 2,064 coefficients before the LFNST index could be determined.

To prevent this increase in buffering, this contribution recommends resolving the inconsistent handling of luma / chroma transform-skip by adding the corresponding condition preventing the use of LFNST with chroma transform-skip.

# Proposal

This contribution proposes to to modify the LFNST signalling conditions as highlighted in the following syntax table. Specifically, adding the requirement that LFNST is not allowed with either luma or chroma transform skip.

In addition, the modification below also makes a suggested fix to only check the luma transform_skip_flag when coding luma TUs (e.g. not for DUAL_TREE_CHROMA) and only check the chroma transform_skip_flag when coding chroma TUs (e.g. not for DUAL_TREE_LUMA).

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && (treeType == DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0) && (treeType == DUAL_TREE_LUMA ¦ ¦ (transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0)) && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

Table 1. Proposed changes

# Experimental Results

Tests were carried out by using the VTM7.0 software, and the common test conditions (CTC) defined in [4] was followed. The summary results are provided in Table 2.

Note that the run time may not be accurate due to the heterogeneous nature of the LSF farm.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.05% | 97% | 94% |
| Class A2 | -0.01% | -0.01% | 0.01% | 105% | 106% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 109% |
| Class C | 0.00% | -0.02% | 0.05% | 93% | 99% |
| Class E | 0.00% | 0.00% | -0.03% | 101% | 103% |
| Overall | 0.00% | -0.01% | 0.02% | 98% | 103% |
| Class D | 0.00% | 0.03% | -0.01% | 95% | 107% |
| Class F (optional) | -0.01% | 0.01% | 0.00% | 99% | 91% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 103% | 98% |
| Class A2 | -0.01% | 0.11% | 0.08% | 97% | 96% |
| Class B | 0.02% | 0.06% | 0.04% | 94% | 89% |
| Class C | 0.02% | 0.07% | 0.08% | 97% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.05% | 97% | 93% |
| Class D | -0.05% | -0.22% | -0.06% | 96% | 92% |
| Class F (optional) | 0.01% | 0.00% | -0.04% | 98% | 99% |

Table 2. Summary Results

# Conclusion

It should be decided whether combinations of LFNST with transform skip will be allowed or not.  Allowing it for chroma transform skip and preventing it for luma transform skip, as currently defined, seems like an unusual choice. Given that the LFNST index is sent at the end of the CU, allowing mixed LFNST / transform-skip CUs introduces a significant increase in buffering. Therefore, the current contribution recommends preventing the use of LFNST with either luma or chroma transform skip.

# Spec Text

See changes in Table 1 (highlighted in yellow).

# References

[1] Y.-K. Wang, B. Bross, V. Drugeon, “AHG2: Editorial input on VVC draft text”, document JVET-Q0041, JVET, Brussels, Belgium, January 2020.

[2] T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, “CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, document JVET-P0058, JVET, Geneva, Switzerland, October 2019.

[3] A. Ramasubramonian, B. Ray, G. Van der Auwera, V. Seregin, H. Egilmez, M. Karczewicz, “AHG15: Scaling matrices for LFNST-coded blocks”, document JVET-P0365, JVET, Geneva, Switzerland, October 2019.

[4] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).