| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0108-v1 |
| --- | --- |

| Title: | CE3-related: On signaling overhead for low-level lossless flags in CE3-2.5 for lossy case |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Alican Nalci, Hongtao Wang, Hilmi E. Egilmez, Muhammed Coban 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In CE3-2.5 low-level (CU or TU) based flags are discussed for lossless coding. These flags allow switching between the two residual coding methods: (1) transform-skip residual coding and (2) regular residual coding for lossless. The signaling of these low-level flags for lossy case are disabled if the high-level (PPS) option is disabled as  “TransformSkipResidualCoding=0”. However, it is important to see the effect of these low-level flags in CE3-2.5 on lossy case if PPS-level flag is not disabled in config (e.g. when “TransformSkipResidualCoding=1”). This document summarizes the signaling overhead of the lossless related flags in CE3-2.5 (sub tests 1, 2, 3). This is achieved by signaling the low-level flags in CE3-2.5 for the lossy case. This document also proposes a modification to sub test 2 of CE3-2.5 (TU-based signaling) to remove the signaling overhead of the TU-based flag for the lossy case.

# Introduction

In VTM-7.0, lossless coding is possible via already existing syntax with TS enabled both for luma and chroma [1]. This form of lossless coding can only use transform skip residual coding (TSRC) and is unable to switch between TSRC and regular residual coding (RRC). In CE 3-2.5, low-level signaling alternatives (e.g. TU or CU based) are proposed to switch between the two residual coding methods for improved bit-rate savings. This low-level functionality can be disabled at the high-level using a PPS flag which disables any possible impact for lossy coding. However, if the high-level syntax in CE 3-2.5 is enabled for lossy case the signaling of TU or CU based low-level flags would impose additional signaling overhead and effect for the lossy case.

In this document, the signaling overhead for sub-tests 1, 2 and 3 are investigated. Additionally, a fix to TU-based signaling in CE3-2.5 is proposed which removes it’s signaling overhead for the lossy case and improves the BD rate of CE3-2.5 sub test 2 for the lossy case.

# Signaling Overhead of CE3-2.5 for Lossy Case

In the results presented below, VTM-7.0 is used as the anchor on top of the CTC configurations. In all CE3-2.5 lossy tests below “TransformSkipResidualCoding=1” option is additionally added to the config. CE3-2.5 code is also modified such that low-level flags (TU and CU) are still signaled but do not change the residual coding method used for lossy coding. Results are obtained for AI, RA and LDB sequences.

Lossy signaling overhead of CE3-2.5 (sub test 1)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.11% | 0.18% | 99% | 99% |
| Class A2 | 0.05% | 0.12% | 0.05% | 98% | 98% |
| Class B | 0.07% | 0.07% | 0.02% | 99% | 97% |
| Class C | 0.09% | 0.12% | 0.15% | 99% | 96% |
| Class E | 0.12% | 0.09% | 0.23% | 99% | 95% |
| Overall | 0.07% | 0.06% | 0.12% | 99% | 97% |
| Class D | 0.15% | 0.14% | 0.24% | 100% | 95% |
| Class F | 0.10% | 0.20% | 0.13% | 99% | 96% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.07% | 0.09% | 98% | 99% |
| Class A2 | 0.14% | 0.23% | 0.15% | 97% | 99% |
| Class B | 0.22% | 0.29% | 0.23% | 99% | 98% |
| Class C | 0.37% | 0.28% | 0.34% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.23% | 0.23% | 0.22% | 98% | 97% |
| Class D | 0.76% | 0.32% | 0.73% | 99% | 98% |
| Class F | 0.40% | 0.39% | 0.30% | 98% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.41% | 0.31% | 0.27% | 98% | 94% |
| Class E | 1.42% | 1.78% | 1.54% | 98% | 96% |
| Overall |  |  |  |  |  |
| Class D | 0.80% | 0.87% | 0.91% | 99% | 101% |
| Class F | 0.70% | 0.38% | 1.01% | 98% | 98% |

Table 1. Signaling overhead of CE3-2.5 sub-test 1 (CU level signaling) for lossy case if “TransformSkipResidualCoding=1” (Pending Full LDB Results).

Lossy signaling overhead of CE3-2.5 (sub test 2)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.09% | 0.07% | 99% | 99% |
| Class A2 | 0.00% | 0.04% | -0.01% | 97% | 98% |
| Class B | 0.01% | 0.01% | -0.06% | 98% | 95% |
| Class C | 0.03% | 0.05% | 0.14% | 99% | 95% |
| Class E | 0.04% | -0.03% | 0.06% | 98% | 97% |
| Overall | 0.02% | 0.00% | 0.03% | 98% | 97% |
| Class D | 0.05% | 0.08% | 0.12% | 100% | 98% |
| Class F | 0.05% | 0.03% | 0.08% | 99% | 97% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.07% | 0.01% | 98% | 99% |
| Class A2 | -0.02% | -0.08% | 0.07% | 96% | 98% |
| Class B | 0.00% | 0.17% | -0.09% | 99% | 98% |
| Class C | 0.07% | -0.10% | -0.05% | 97% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.03% | 98% | 97% |
| Class D | 0.07% | -0.02% | 0.03% | 99% | 98% |
| Class F | 0.02% | 0.04% | 0.02% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.09% | -0.21% | 0.08% | 97% | 93% |
| Class E | 0.04% | 0.17% | -0.31% | 97% | 93% |
| Overall |  |  |  |  |  |
| Class D | 0.08% | 0.30% | -0.06% | 98% | 99% |
| Class F | 0.03% | 0.32% | 0.42% | 98% | 99% |

Table 2. Signaling overhead of CE3-2.5 sub-test 2 (TU level signaling) for lossy case if “TransformSkipResidualCoding=1”. (Pending Full LDB Results).

Lossy signaling overhead of CE3-2.5 (sub test 3)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.10% | 0.17% | 98% | 98% |
| Class A2 | 0.05% | 0.09% | 0.06% | 98% | 98% |
| Class B | 0.07% | 0.08% | 0.02% | 99% | 95% |
| Class C | 0.09% | 0.13% | 0.15% | 99% | 95% |
| Class E | 0.11% | 0.12% | 0.21% | 99% | 95% |
| Overall | 0.07% | 0.07% | 0.11% | 99% | 96% |
| Class D | 0.14% | 0.15% | 0.23% | 100% | 96% |
| Class F | 0.12% | 0.07% | 0.13% | 99% | 97% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.00% | 0.09% | 98% | 99% |
| Class A2 | 0.05% | 0.16% | 0.10% | 97% | 98% |
| Class B | 0.16% | 0.09% | 0.11% | 98% | 98% |
| Class C | 0.22% | 0.00% | 0.08% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.15% | 0.06% | 0.09% | 98% | 97% |
| Class D | 0.41% | 0.03% | 0.07% | 98% | 97% |
| Class F | 0.20% | 0.16% | 0.03% | 98% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.34% | 0.03% | 0.02% | 98% | 95% |
| Class E | 0.45% | 0.77% | 0.85% | 97% | 93% |
| Overall |  |  |  |  |  |
| Class D | 0.50% | 0.68% | 0.14% | 99% | 99% |
| Class F | 0.28% | -0.10% | 0.45% | 97% | 97% |

Table 3. Signaling overhead of CE3-2.5 sub-test 3 (CU level signaling) for lossy case if “TransformSkipResidualCoding=1” (Pending Full LDB Results).

# Proposed Method

In this proposal, signalling of the TU-level residual coding flag from CE3-2.5 is further constrained based on the QP value. This is achieved by enforcing a QP level check on top of CE3-2.5 sub-test 2 before signaling the TU-level lossless residual switch flag. This effectively removes the signaling overhead of this flag for the lossy case and minimizes the effect of CE3-2.5 on lossy case. Following spec changes on top of CE3-2.5 show the proposed approach:

#### 7.3.9.10 Transform unit syntax (modified from CE3-2.5)

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( pps_tsrc_flag && transform_skip_flag[ x0 ][ y0 ][ 0 ] && ( qP <= 4 ) ) |  |
| tu_tsrc_flag [ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦¦ !tu_tsrc_flag [ x0 ][ y0 ][ 0 ]) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( pps_tsrc_flag && transform_skip_flag[ x0 ][ y0 ][ 1 ] && ( qP <= 4 ) ) |  |
| tu_tsrc_flag [ x0 ][ y0 ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦¦ !tu_tsrc_flag [ x0 ][ y0 ][ 1 ]) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( pps_tsrc_flag && transform_skip_flag[ x0 ][ y0 ][ 2 ] && ( qP <= 4 ) ) |  |
| tu_tsrc_flag [ x0 ][ y0 ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦¦ !tu_tsrc_flag[ x0 ][ y0 ][ 2 ]) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

# Results

In the results presented below, VTM-7.0 is used as the anchor on top of the CTC configurations. The proposed change is implemented on top of CE3-2.5 sub test 2. In the test below “TransformSkipResidualCoding=1” option is additionally added to the config. Results are obtained for AI, RA and LDB sequences.

The following table shows the lossy signalling overhead of the TU level flag in CE3-2.5 after the proposed QP-based constraint.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 93% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 97% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 4. Signaling overhead of CE3-2.5 sub-test 2 (TU level signaling) with the proposed QP-based restriction for lossy case when “TransformSkipResidualCoding=1” (Pending LDB Results).

# Recommendation

It is recommended to add a QP-based constraint on in signaling the lossless related residual coding switch flag.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).