| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0110-v4 |
| --- | --- |

| Title: | On Intra Prediction Mode and Chroma BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Luong Pham Van, Hilmi E. Egilmez, Muhammed Coban, Geert Van Der Auwera, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes two aspects and fixes for chroma BDPCM. The first aspect (bug-fix) provides an alignment of the intra prediction mode for chroma blocks when chroma BDPCM (e.g. horizontal or vertical mode) is used in 444 coding. This aspect simplifies the MPM list derivations process and provides coding gains. The same alignment was already adopted for luma blocks in JVET-O0315.

The second aspect provides an additional context derivation fix for BDPCM on top of aspect #1. Currently, BDPCM is applied to luma and chroma separately. However, BDPCM contexts are shared between luma and chroma blocks. In the spec text BDPCM has separate contexts for luma and chroma unlike the VVC software. Considering that TS has separate contexts luma and chroma, it is proposed to separate BDPCM contexts for luma and chroma and align code with spec. This provides additional coding gains.

Aspect 1 provides -0.24%/-0.11% overall BD-Rate changes under AI/RA configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is off.

Aspect 1 provides -0.13%/-0.01% overall BD-Rate changes under AI/RA configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is on.

Aspect 2 provides -0.36%/-0.14% overall BD-Rate changes under AI/RA configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is off.

Aspect 2 provides -0.23%/-0.08% overall BD-Rate changes under AI/RA configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is on.

Additional Results

Aspect 1 provides -0.26%/-0.12% overall BD-Rate changes under AI/RA/LDB configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is off and palette is off.

Aspect 2 provides -0.43%/-0.21% overall BD-Rate changes under AI/RA/LDB configurations versus the VTM-7.0 anchor on 4:4:4 sequences when dual-tree is off and palette is off.

# Introduction

In VVC design, block-based quantized residual domain DPCM (BDPCM) is used for chroma blocks for 444 sampling. In JVET-O0315, luma intra prediction mode was aligned with the BDPCM mode (vertical and horizontal). In this case intra mode for luma could be either INTRA_ANGULAR50 or INTRA_ANGULAR18 depending on the BDPCM mode (e.g. vertical or horizontal). In JVET-P0059, chroma BDPCM was adopted for 444 in which chroma intra prediction mode does not follow the BDPCM mode used. That is the internal intra prediction mode for chroma blocks can be different than INTRA_ANGULAR50 or INTRA_ANGULAR18. The original JVET-P0059 for chroma BDPCM does not have such an alignment for chroma either in code or spec text.

# Proposed Method

In aspect #1, a bug-fix/alignment is provided for chroma intra prediction mode derivation. Intra prediction after the proposed fix follows the respective BDPCM mode for chroma. This approach is currently in VVC for luma. This is the same as JVET-O0315 but for chroma blocks instead.

Second, BDPCM is only used with transform skip (TS). TS has separate contexts for luma and chroma blocks. On the other hand, BDPCM contexts for are currently shared between luma and chroma blocks in the VVC software even though it is applied separately for luma and chroma. In the spec text, BDPCM contexts are separate for luma and chroma. In aspect #2, BDPCM contexts are separated for luma and chroma in software and aligned with spec text. The resulting change aligns with the context derivation of TS and results in coding gains for 444.

# Results

The proposed scheme is implemented on top of VTM-7 and the anchor is VTM-7.0 with CTC configurations [2]. The proposed chroma BDPCM changes are only tested on 444 sequences since chroma BDPCM is only enabled for 444. Two sets of results were obtained for both aspects with dual tree on and dual tree off settings, these results are as follows:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.07% | 0.21% | 0.16% | 100% | 102% |
| TGM 720p | -0.26% | 0.19% | 0.21% | 100% | 101% |
| Animation | -0.13% | 0.18% | 0.12% | 100% | 99% |
| Mixed content | -0.60% | 0.23% | 0.13% | 100% | 104% |
| Camera-Captured content | -0.15% | 0.12% | 0.03% | 100% | 101% |
| Overall | -0.24% | 0.19% | 0.15% | 100% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | 0.08% | 0.04% | 100% | 101% |
| TGM 720p | -0.09% | 0.32% | 0.29% | 100% | 100% |
| Animation | 0.00% | 0.09% | 0.10% | 100% | 100% |
| Mixed content | -0.39% | 0.33% | 0.28% | 100% | 100% |
| Camera-Captured content | -0.04% | 0.01% | -0.07% | 100% | 99% |
| Overall | -0.11% | 0.18% | 0.15% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.05% | 0.04% | 0.06% | 99% | 98% |
| TGM 720p | -0.26% | -0.30% | 0.04% | 101% | 101% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.19% | 0.80% | 1.19% | 101% | 99% |
| Camera-Captured content | -0.04% | -0.02% | -0.02% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 1. (Aspect #1) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is OFF.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.04% | 0.32% | 0.28% | 100% | 102% |
| TGM 720p | -0.11% | 0.00% | 0.07% | 100% | 105% |
| Animation | -0.45% | -0.61% | -0.64% | 100% | 104% |
| Mixed content | -0.13% | 0.25% | 0.15% | 100% | 102% |
| Camera-Captured content | -0.02% | -0.35% | -0.53% | 101% | 98% |
| Overall | -0.13% | -0.03% | -0.07% | 100% | 103% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.07% | 0.14% | 0.18% | 100% | 101% |
| TGM 720p | -0.07% | 0.08% | 0.03% | 100% | 100% |
| Animation | -0.07% | -0.13% | -0.12% | 101% | 100% |
| Mixed content | 0.01% | 0.35% | 0.29% | 100% | 100% |
| Camera-Captured content | -0.03% | -0.16% | -0.30% | 101% | 100% |
| Overall | -0.01% | 0.08% | 0.05% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.04% | 0.00% | 0.01% | 100% | 99% |
| TGM 720p | -0.12% | 0.30% | 0.36% | 101% | 102% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.06% | 0.19% | 0.43% | 100% | 99% |
| Camera-Captured content | 0.06% | -0.11% | -0.21% | 100% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 2. (Aspect #1) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is ON.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.26% | 0.20% | 0.15% | 99% | 104% |
| TGM 720p | -0.38% | 0.24% | 0.47% | 99% | 101% |
| Animation | -0.20% | 0.10% | 0.06% | 100% | 98% |
| Mixed content | -0.76% | 0.18% | 0.07% | 99% | 103% |
| Camera-Captured content | -0.14% | 0.14% | 0.04% | 100% | 101% |
| Overall | -0.36% | 0.18% | 0.19% | 99% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | 0.10% | 0.09% | 100% | 101% |
| TGM 720p | -0.13% | 0.15% | 0.37% | 100% | 100% |
| Animation | -0.06% | 0.07% | 0.05% | 100% | 100% |
| Mixed content | -0.38% | 0.44% | 0.52% | 99% | 100% |
| Camera-Captured content | -0.04% | 0.13% | 0.00% | 100% | 100% |
| Overall | -0.14% | 0.17% | 0.22% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | 0.11% | 0.11% | 99% | 100% |
| TGM 720p | -0.14% | 0.32% | 0.02% | 101% | 102% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.19% | 0.60% | 1.01% | 99% | 98% |
| Camera-Captured content | -0.05% | 0.03% | 0.04% | 99% | 96% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 3. (Aspect #2) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is OFF + Separate contexts for Luma and Chroma BDPCM.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.16% | 0.11% | 0.09% | 100% | 100% |
| TGM 720p | -0.17% | -0.12% | -0.10% | 100% | 106% |
| Animation | -0.48% | -0.62% | -0.67% | 100% | 103% |
| Mixed content | -0.29% | 0.12% | 0.02% | 100% | 101% |
| Camera-Captured content | -0.03% | -0.35% | -0.50% | 101% | 99% |
| Overall | -0.23% | -0.14% | -0.19% | 100% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | 0.08% | 0.08% | 100% | 101% |
| TGM 720p | -0.11% | 0.00% | 0.06% | 100% | 101% |
| Animation | -0.13% | -0.21% | -0.21% | 100% | 100% |
| Mixed content | -0.01% | 0.26% | 0.27% | 100% | 100% |
| Camera-Captured content | -0.06% | -0.23% | -0.30% | 101% | 100% |
| Overall | -0.08% | 0.00% | 0.01% | 100% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | -0.03% | -0.07% | 100% | 100% |
| TGM 720p | -0.24% | 0.17% | 0.23% | 102% | 103% |
| Animation | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Mixed content | -0.17% | 0.12% | 0.17% | 100% | 101% |
| Camera-Captured content | 0.00% | -0.23% | -0.16% | 100% | 98% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4. (Aspect #2) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is ON + Separate contexts for Luma and Chroma BDPCM.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.17% | 0.29% | 0.27% | 101% | 98% |
| TGM 720p | -0.26% | 0.28% | 0.24% | 102% | 108% |
| Animation | -0.16% | 0.06% | 0.07% | 100% | 104% |
| Mixed content | -0.57% | 0.22% | 0.18% | 100% | 101% |
| Camera-Captured content | -0.13% | 0.10% | 0.01% | 100% | 105% |
| Overall | -0.26% | 0.21% | 0.17% | 101% | 103% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.09% | 0.10% | 0.12% | 101% | 101% |
| TGM 720p | -0.07% | 0.25% | 0.27% | 99% | 99% |
| Animation | -0.05% | 0.01% | 0.02% | 100% | 100% |
| Mixed content | -0.36% | 0.28% | 0.22% | 100% | 100% |
| Camera-Captured content | -0.06% | 0.19% | 0.03% | 100% | 99% |
| Overall | -0.12% | 0.17% | 0.14% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.01% | 0.18% | 0.17% | 100% | 100% |
| TGM 720p | -0.08% | 0.25% | -0.14% | 102% | 104% |
| Animation | -0.21% | 0.32% | -0.04% | 102% | 103% |
| Mixed content | -0.12% | 0.56% | 0.13% | 101% | 102% |
| Camera-Captured content | -0.01% | 0.06% | 0.09% | 99% | 95% |
| Overall | -0.09% | 0.28% | 0.04% | 101% | 101% |

Table 5. (Aspect #1) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is OFF and Palette is Off.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.42% | 0.06% | 0.07% | 99% | 102% |
| TGM 720p | -0.52% | 0.05% | 0.09% | 99% | 107% |
| Animation | -0.21% | 0.07% | 0.08% | 97% | 95% |
| Mixed content | -0.73% | 0.13% | 0.10% | 98% | 100% |
| Camera-Captured content | -0.12% | 0.08% | 0.00% | 100% | 102% |
| Overall | -0.43% | 0.07% | 0.08% | 98% | 102% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.17% | 0.01% | 0.05% | 100% | 99% |
| TGM 720p | -0.23% | 0.19% | 0.10% | 100% | 100% |
| Animation | -0.12% | -0.07% | 0.02% | 100% | 100% |
| Mixed content | -0.43% | 0.25% | 0.25% | 99% | 101% |
| Camera-Captured content | -0.04% | 0.20% | -0.16% | 101% | 101% |
| Overall | -0.21% | 0.11% | 0.07% | 100% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.14% | 0.08% | 0.02% | 100% | 100% |
| TGM 720p | -0.35% | -0.12% | 0.23% | 103% | 105% |
| Animation | -0.24% | 0.17% | -0.05% | 102% | 102% |
| Mixed content | -0.24% | 0.36% | 0.34% | 101% | 101% |
| Camera-Captured content | -0.02% | 0.11% | -0.01% | 98% | 97% |
| Overall | -0.22% | 0.10% | 0.11% | 101% | 101% |

Table 6. (Aspect #2) Fix for Chroma Intra Prediction Mode for Chroma BDPCM when Dual-tree is OFF, Palette is Off + Separate contexts for Luma and Chroma BDPCM.

# Recommendation

It is recommended to adopt both aspects into VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).