| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0112-v1 |
| --- | --- |

| Title: | On level definitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Bytedance, San Diego, CA, USA Yong He InterDigital, San Diego, CA, USA Ping Wu ZTE, Slough, Berkshire, UK | Tel: Email: | yekui.wang@bytedance.com yong.he@interdigital.com pwuzte@gmail.com |
| Source: | Bytedance Inc., InterDigital Communications, Inc., ZTE UK Ltd. |  |  |

_____________________________

# Abstract

This contribution proposes a few changes to the level definitions text in the latest VVC draft text, summarized as follows:

- The equation for the derivation of the variable MaxDpbSize is updated to use the maximum picture size among pictures within a CLVS instead of using the variable PicSizeInSamplesY, and a value of MaxDpbSize is derived for each layer.

- For each layer, a constraint is specified to require the value of a specific instance of the max_dec_pic_buffering_minus1[ ] syntax element plus one shall be less than or equal to the value of MaxDpbSize of the layer. Furthermore, a constraint on the overall DPB size for an OLS containing multiple layers is specified.

- The definitions of the constraints on picture size, picture width, and picture height use the maximum picture size, picture width, and picture height instead of the variable PicSizeInSamplesY and the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples. Furthermore, the constraints are specified separately for each of the SPSs referenced by the layers.

- The definitions of several constraints for an AU on the CPB removal time, the nominal CPB removal time, the DPB output time, and the sum of the NumBytesInNalUnit variables use the sum of the picture size of all pictures in an AU instead of the variable PicSizeInSamplesY.

- Instead of specifying a limit on the maximum number of slices per picture for each level and using the limit in specifying some constraints for an AU on CPB removal times, a limit on the maximum number of slices per AU is specified and used in specifying those constraints.

The exact proposed text changes are included in Section 3 of this contribution.

# Introduction

The current level definitions in the latest VVC text have the following technical issues:

- The equation for the derivation of the variable MaxDpbSize, i.e., Equation (A.1), uses the variable PicSizeInSamplesY, which is the picture size of a particular picture. However, in VVC, the picture size can change from picture to picture within a CLVS. Therefore, the maximum picture size among pictures within a CLVS should be used for the derivation of MaxDpbSize instead. Furthermore, because there can be multiple layers, and different layers in the bitstream of an OLS can have different values of the maximum picture size among pictures within a CLVS, there needs to be a value of MaxDpbSize derived for each layer.

- The current constraint related to the DPB size, specifying that the value of max_dec_pic_buffering_minus1[ ] + 1 shall be less than or equal to MaxDpbSize, applies only to single-layer bitstreams. For an OLS that contains multiple layers in its bitstream, because there can be multiple, difference instances of the max_dec_pic_buffering_minus1[ ] syntax element that apply to the different layers, the existing constraint does not apply. Instead, there needs to be a constraint for each layer using the appropriate instance of the max_dec_pic_buffering_minus1[ ] syntax element. Furthermore, there needs to be a constraint on the overall DPB size for an OLS containing multiple layers in a manner such that the same overall DPB size limit would work regardless of the number of layers.

- The definitions of the constraints on picture size, picture width, and picture height, i.e., items a, b, and c of the general tier and level limits, use the variable PicSizeInSamplesY and the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples. Again, due to that in VVC the picture size can change from picture to picture within a CLVS, the values of the maximum picture size, picture width, and picture height should be used instead. Furthermore, because there can be multiple layers, and different layers in the bitstream of an OLS have different values of the maximum picture size, picture width, and picture height among pictures within a CLVS, the constraints need to be specified separately for each layer or each of the SPSs referenced by the layers.

- The definitions of several constraints for an AU on the CPB removal time, the nominal CPB removal time, the DPB output time, and the sum of the NumBytesInNalUnit variables, i.e., items a, b, c, d, g, and i of the profile-specific level limits, use the variable PicSizeInSamplesY, which is the picture size of a particular picture. However, since an AU may contain multiple pictures, the sum of the picture size of all pictures in an AU should be used instead.

- Finally, instead of specifying a limit on the maximum number of slices per picture for each level and using the limit in specifying some constraints for an AU on CPB removal times, i.e., items c and d of the profile-specific level limits, a limit on the maximum number of slices per AU should be specified and used in specifying those constraints.

# Proposal

The exact proposed text changes are included in Section 3.

The changes are summarized as follows (there are also some editorial improvements that have been included in the text changes, but those are not summarized below):

- The equation for the derivation of the variable MaxDpbSize is updated to use the maximum picture size among pictures within a CLVS instead of using the variable PicSizeInSamplesY, and a value of MaxDpbSize is derived for each layer.

- For each layer, a constraint is specified to require the value of a specific instance of the max_dec_pic_buffering_minus1[ ] syntax element plus one shall be less than or equal to the value of MaxDpbSize of the layer. Furthermore, a constraint on the overall DPB size for an OLS containing multiple layers is specified.

- The definitions of the constraints on picture size, picture width, and picture height use the maximum picture size, picture width, and picture height instead of the variable PicSizeInSamplesY and the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples. Furthermore, the constraints are specified separately for each of the SPSs referenced by the layers.

- The definitions of several constraints for an AU on the CPB removal time, the nominal CPB removal time, the DPB output time, and the sum of the NumBytesInNalUnit variables use the sum of the picture size of all pictures in an AU instead of the variable PicSizeInSamplesY.

- Instead of specifying a limit on the maximum number of slices per picture for each level and using the limit in specifying some constraints for an AU on CPB removal times, a limit on the maximum number of slices per AU is specified and used in specifying those constraints.

# Proposed spec text changes

The proposed changes on top of the latest VVC draft text are provided below, where most relevant parts that have been added or modified are highlighted in yellow.

  - Overview of profiles, tiers and levels

Profiles, tiers and levels specify restrictions on bitstreams and hence limits on the capabilities needed to decode the bitstreams. Profiles, tiers and levels may also be used to indicate interoperability points between individual decoder implementations.

Each profile specifies a subset of algorithmic features and limits that shall be supported by all decoders conforming to that profile.

NOTE – Encoders are not required to make use of any particular subset of features supported in a profile.

Each level of a tier specifies a set of limits on the values that may be taken by the syntax elements of this Specification. The same set of tier and level definitions is usally used with all profiles, but individual implementations may support a different tier and within a tier a different level for each supported profile. For any given profile, a level of a tier generally corresponds to a particular decoder processing load and memory capability.

In this annex, phrases like "the bitstream" should be intepreted as "the bitstream of the operation point", and phrases like "AU n" and "a layer" should be interpreted as "AU n in the bitstream of the operation point" and "a layer in the bitstream of the operation point", respectively, where "the operation point" is the operation point with which the profile, tier, or level is associated with.

  - Tiers and levels

    - General tier and level limits

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of tier capabilities, the tier with general_tier_flag equal to 0 is considered to be a lower tier than the tier with general_tier_flag equal to 1.

For purposes of comparison of level capabilities, a particular level of a specific tier is considered to be a lower level than some other level of the same tier when the value of the general_level_idc or sublayer_level_idc[ i ] of the particular level is less than that of the other level.

The following is specified for expressing the constraints in this annex:

- Let AU n be the n-th AU in decoding order, with the first AU being AU 0 (i.e., the 0-th AU).

- Let PicSizeMaxInSamplesY be the value of pic_width_max_in_luma_samples * pic_height_max_in_luma_samples for a particular referenced SPS.

When the specified level is not level 8.5, for each layer in the OLS, the value MaxDpbSize is derived as follows [Ed. (YK): Note that, as an editorial improvement, this equation was firstly moved herein from clause A.4.1, as the DPB size limit is rather a general level limit than a profile-specific limit, then it was modified as highlighted]:

if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A.1)
else if( PicSizeMaxInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A.1, and maxDpbPicBuf is equal to 8.

When the specified level is not level 8.5, bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- PicSizeMaxInSamplesY for each referenced SPS shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

- The value of pic_width_max_in_luma_samples for each referenced SPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of pic_height_max_in_luma_samples for each referenced SPS shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- For each referenced PPS the value of NumTileColumns shall be less than or equal to MaxTileCols and the value of NumTileRows shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

- For the VCL HRD parameters, CpbSize[Htid][ i ] shall be less than or equal to CpbVclFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, CpbSize[Htid][ i ] shall be less than or equal to CpbNalFactor * MaxCPB for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where CpbSize[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1, CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table A.1  in units of CpbNalFactor bits.

- For each layer in the OLS, the value of max_dec_pic_buffering_minus1[ maxTid ] + 1 shall be less than or equal to MaxDpbSize for the layer, where max_dec_pic_buffering_minus1[ maxTid ] is the value of max_dec_pic_buffering_minus1[ i ] for the highest value of i in the dpb_parameters( ) syntax structure that is determined as the follows: if the layer is an output layer of the OLS, the dpb_parameters( ) syntax structure is the one that applies to the layer when the layer is an output layer in an OLS; otherwise, the dpb_parameters( ) syntax structure is the one that applies to the layer when the layer is not an output layer in an OLS.

- Immediately after the decoding of each AU, let numDecPics be the number of decoded pictures in the DPB, and picSizeInSamplesY[ i ] for i in the range of 0 to numDecPics − 1, inclusive, be the value of PicSizeInSamplesY of the i-th decoded picture in the DPB, the value of i = 0numDecPics - 1picSizeInSamplesY[ i ] shall be less than or equal to maxDpbPicBuf* MaxLumaPs, where maxDpbPicBuf is equal to 8 and MaxLumaPs is specified in Table A.1.

Table A.1 specifies the limits for each level of each tier for levels other than level 8.5.

A tier and level to which a bitstream conforms are indicated by the syntax elements general_tier_flag and general_level_idc, and a level to which a sublayer representation conforms are indicated by the syntax element sublayer_level_idc[ i ], as follows:

–	If the specified level is not level 8.5, general_tier_flag equal to 0 indicates conformance to the Main tier, general_tier_flag equal to 1 indicates conformance to the High tier, according to the tier constraints specified in Table A.1 and general_tier_flag shall be equal to 0 for levels below level 4 (corresponding to the entries in Table A.1 marked with "-"). Otherwise (the specified level is level 8.5), it is a requirement of bitstream conformance that general_tier_flag shall be equal to 1 and the value 0 for general_tier_flag is reserved for future use by ITU-T | ISO/IEC and decoders shall ignore the value of general_tier_flag.

–	general_level_idc and sublayer_level_idc[ i ] shall be set equal to a value of 30 times the level number specified in Table A.1.

Table A.1 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max # of slices per AU MaxSlicesPerAu | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |

    - Profile-specific level limits

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10 profile or the Main 4:4:4 10 profile, HbrFactor is set equal to 1.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

Let numDecPics be the number of pictures in AU n, and for i in the range of 0 to numDecPics − 1, inclusive, picSizeInSamplesY[ i ] be the value of PicSizeInSamplesY for the i-th picture in AU n, and picSizeMaxInSamplesY[ i ] be the greatest value among the list of PicSizeMaxInSamplesY values for the i-th picture in AU n and all the pictures in AU n that belong to the direct or indirect reference layers of the layer containing the i-th picture in AU n. The variables AuSizeInSamplesY[ n ] and AuSizeMaxInSamplesY[ n ] are set equal to i = 0numDecPics - 1picSizeInSamplesY[ i ] and i = 0numDecPics - 1picSizeMaxInSamplesY[ i ], respectively.

Bitstreams conforming to the Main 10 or Main 4:4:4 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- The nominal removal time of AU n (with n greater than 0) from the CPB, as specified in clause C.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n − 1 ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 that applies to AU n − 1.

- The difference between consecutive output times of pictures of different AUs from the DPB, as specified in clause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output.

- The CPB removal time of AU 0 shall satisfy the constraint that the number of slices in AU 0 is less than or equal to Min( Max( 1, MaxSlicesPerAu * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSlicesPerAu * AuSizeInSamplesY[ 0 ] / MaxLumaPs ), MaxSlicesPerAu ), where MaxSlicesPerAu, MaxLumaPs, and MaxLumaSr are the values specified in Table A.1 and Table A.2, respectively, that apply to AU 0.

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slices in AU n is less than or equal to Min( (Max( 1, MaxSlicesPerAu * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSlicesPerAu ), where MaxSlicesPerAu, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2 that apply to AU n.

- For the VCL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for AU 0 shall be less than or equal to FormatCapabilityFactor * ( Max( AuSizeInSamplesY[ 0 ], fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0.

- The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n.

- The removal time of AU 0 shall satisfy the constraint that the number of tiles in each picture in AU 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * AuSizeInSamplesY[ 0 ] / MaxLumaPs ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU 0.

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in each picture in AU n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU n.

...

# Patent rights declaration(s)

Bytedance Inc., InterDigital Communications, Inc., and ZTE UK Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).