| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0114-v1 |
| --- | --- |

| Title: | AHG9: A few more general constraints flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following regarding general constraint flags:

- Remove frame_only_constraint_flag, which is redundant to general_frame_only_constraint_flag.

- Add a general constraint on no tile partitioning (i.e., one tile only for each picture).

- Add a general constraint on no slice partitioning (i.e., one slice only for each picture).

- Add a general constraint on no subpicture partitioning (i.e., one subpicture only for each picture).

- Add a general constraint on no reference picture resampling.

- Add general_non_projected_constraint_flag, the value equal to 1 specifies that there is no omnidirectional projection SEI messages, and change the semantics of general_non_packed_constraint_flag to cover only frame packing, as frame packing and no omnidirectional projection can be independently applied.

# Proposal

The proposed text changes on top of the latest VVC draft text are provided below. The most relevant parts that have been added or modified are highlighted in yellow.

- 7.3.3.2 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| frame_only_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| ... |  |
| } |  |

- 7.4.4.2 General constraint information semantics

general_progressive_source_flag and general_interlaced_source_flag are interpreted as follows:

–	If general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in OlsInScope should be interpreted as progressive only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the source scan type of the pictures in OlsInScope should be interpreted as interlaced only.

–	Otherwise, if general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the source scan type of the pictures in OlsInScope should be interpreted as unknown or unspecified.

–	Otherwise (general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 1), the source scan type of each picture in OlsInScope should be interpreted as unknown or unspecified.

NOTE 1 – Decoders may ignore the values of general_progressive_source_flag and general_interlaced_source_flag. Moreover, the actual source scan type of the pictures is outside the scope of this Specification and the method by which the encoder selects the values of general_progressive_source_flag and general_interlaced_source_flag is unspecified.

general_non_packed_constraint_flag equal to 1 specifies that there are no frame packing arrangement SEI messages present in the bitstream of the OlsInScope. general_non_packed_constraint_flag equal to 0 specifies that there may or may not be one or more frame packing arrangement SEI messages present in the bitstream of the OlsInScope.

NOTE 2 – Decoders may ignore the value of general_non_packed_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of frame packing arrangement SEI messages.

general_non_packed_constraint_flag equal to 1 indicates that the cropped output pictures of the decoded bitstream are suitable to be displayed. general_non_packed_constraint_flag equal to 0 indicates that the cropped output pictures of the decoded bitstream may require further processing to be suitable for displaying. [Ed. Consider a better naming of this constraint flag, e.g., display_suitability_flag or something like that.]

NOTE 2 – Examples for content that requires additional processing before displaying are bitstreams containing one or more frame packing arrangement SEI messages, equirectangular projection SEI messages, or cubemap projection SEI messages. [Ed. This NOTE should be updated at a later phase of VVC standardization based on which SEI messages are adopted into the VVC design.]

general_frame_only_constraint_flag equal to 1 specifies that OlsInScope conveys pictures that represent frames. general_frame_only_constraint_flag equal to 0 specifies that OlsInScope conveys pictures that may or may not represent frames.

NOTE 3 – Decoders may ignore the value of general_frame_only_constraint_flag, as there are no decoding process requirements associated with it.

general_non_projected_constraint_flag equal to 1 specifies that there are no SEI messages indicating an omnidirectional projection to be applied to the cropped output pictures before displaying present in the bitstream of the OlsInScope. general_non_projected_constraint_flag equal to 0 specifies that there may or may not be one or more such SEI messages present in the bitstream of the OlsInScope.

NOTE 4 – Decoders may ignore the value of general_non_projected_constraint_flag, as there are no decoding process requirements associated with the presence or interpretation of such SEI messages, which e.g., can be equirectangular projection SEI messages or generalized cubemap projection SEI messages.

intra_only_constraint_flag equal to 1 specifies that slice_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose such a constraint.

max_bitdepth_constraint_idc specifies that bit_depth_minus8 shall be in the range of 0 to max_bitdepth_constraint_idc, inclusive.

max_chroma_format_constraint_idc specifies that chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc, inclusive.

no_ref_pic_resampling_constraint_flag equal to 1 specifies that sps_ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint.

one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile. one_tile_per_pic_constraint_flag equal to 0 does not impose such a constraint.

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint.

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

frame_only_constraint_flag equal to 1 specifies that the CVS conveys pictures that represent frames. frame_only_constraint_flag equal to 0 does not impose such a constraint.

no_qtbtt_dual_tree_intra_constraint_flag equal to 1 specifies that qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra_constraint_flag equal to 0 does not impose such a constraint.

...

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the author).