| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0123-v1 |
| --- | --- |

| Title: | CE4-related: Modifications of GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes several changes to GEO to address the issue regarding hardware validation checks for encoder.

(1) disable 64x8 and 8x64 sized GEO block;

(2) support scalable number of modes;

(3) set number of modes to 32;

(4) fix number of modes per block to 14 by only allowing (inverse) diagonal angle.

Coding performance of aspect (1)-(4) is as below:

(1) disable 64x8 and 8x64 sized GEO block:				RA:	   0.**%, 0.**%, 0.**%

(CE4 common base as anchor)						LDB:     0.03%, -0.04%, -0.03%

(2) scalable number of modes (16 modes):				RA:	   -0.09%, -0.09%, -0.08%

(VTM7.0 as anchor)								LDB:     -0.28%, -0.20%, -0.20%

scalable number of modes (64 modes):				RA:	   -0.28%, -0.36%, -0.30%

(VTM7.0 as anchor)								LDB:     -0.70%, -0.62%, -0.62%

(3) set number of modes to 32 modes:					RA:	   -0.19%, -0.25%, -0.19%

(VTM7.0 as anchor)								LDB	   -0.53%, -0.47%, -0.35%

(4)fix number of modes per block to 14					RA:	   -0.11%, -0.10%, -0.03%

(VTM7.0 as anchor)								LDB:     -0.33%, -0.40%, -0.20%

# Introduction

Geometrical partitioning (GEO) was firstly proposed in JVET-J1030/JVET-K0146 which applied to both AMVP and merge mode. In JVET-O0489, GEO was proposed for merge mode with 140 supported partitioning modes and 19 supported block size (8x8 to 128x128). In JVET-P0884/0885, efforts were made to remove multiplication of weight mask calculation and number of modes was reduced to 82. In JVET-Q0059, number of modes is further reduced to 64. It can be observed that continuous efforts are mode to reduce complexity as well total number of cases to reduce hardware validation check efforts. Summary is given in Table 1.

Table 1: number of test cases between TPM, wedge mode and GEO

|  | Total # of cases | # modes per block size | # of supported block sizes |
| --- | --- | --- | --- |
| TPM | 50 | 2 | 25 |
| Wedge mode (in other standard) | 144 | 16 | 8 |
| GEO (JVET-O0489) | 2,660 | 140 | 19 |
| GEO (JVET-P0884/JVET-Q0079, CE4 common base) | 1,558 | 82 | 19 |
| GEO (JVET-Q0059, CE4-1) | 1,216 | 64 | 19 |

# Proposal

## Remove 64x8 and 8x64 sized GEO block

It is proposed not to allow 64x8 and 8x64 sized coding block for GEO. This aspect is implemented on CE4 common base (JVET-Q0079). It is noted that this aspect can also be combined with CE4-2.1 so that the complexity can be reduced more efficiently.

## Scalable geo by signaling number of valid modes in SPS

The scalable idea for GEO was first proposed in JVET-P0174. This aspect focuses on making number of modes ‘scalable’ which is to allow encoder to choose to use either 16 valid geo modes or 64 valid geo modes.

In this way it is more feasible for encoders to choose a suitable complexity level according to its own actual application need and budget of hardware validation checks.

A flag is signaled in SPS level to switch between 16 and 64 modes.

- If 16 mode is selected, only the first 16 modes of GEO mode table are valid.

- If 64 mode is selected, all 64 modes of GEO mode table are valid.

This aspect is implemented over CE4-1(total number of valid modes is 64). Note that GEO mode table is reordered according to statistics. and signaling total valid number of geo modes affects coding of merge_geo_parititon_idx with cmax set to 16 and 64 respectively.

Table 2: Reordered GEO table

|  | # of cases | # modes per block size | # of supported block sizes |
| --- | --- | --- | --- |
| TPM | 50 | 2 | 25 |
| GEO (JVET-Q0059, CE4-1) | 1,216 | 64 | 19 |
| Proposal (64 mode) | Same as above | Same as above | Same as above |
| Proposal (16 mode) | 304 | 16 | 19 |

Syntax change is illustrated as below:

Table 3: proposed SPS syntax change to support ‘scalable’ GEO

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| sps_geo_enabled_flag | u(1) |
| if(sps_geo_enabled_flag) |  |
| sps_geo_mode_set_id | u(1) |
| … |  |

sps_geo_mode_set_id specifies whether there are 16 or 64 valid modes for geo inter prediction. sps_geo_mode_set_id equals to 0 specifies that there are 16 valid modes and sps_geo_mode_set_id equals to 1 specifies that there are 64 valid modes. When sps_geo_mode_set_id is not present, it is inferred to be equal to 1.

| merge_geo_partition_idx[ ][ ] | TB | cMax = sps_geo_mode_set_id == 0 ? 16 : 64 |
| --- | --- | --- |

As additional information, Coding gain for using 8, 16, 32, 48, 64 are also tested and results are reported as below. (Anchor is VTM7.0)

Valid number of modes is 8: 							RA:	   0.06%, 0.18%, 0.16%

LDB:     0.06%, 0.14%, 0.26%

Valid number of modes is 16: 							RA:	   -0.09%, -0.09%, -0.08%

LDB:     -0.28%, -0.20%, -0.20%

Valid number of modes is 32: 							RA:	   -0.19%, -0.25%, -0.19%

LDB	   -0.53%, -0.47%, -0.35%

Valid number of modes is 48: 							RA:	   -0.25%, -0.29%, -0.27%

LDB:     0.00%, 0.00%, 0,00%

Valid number of modes is 64: 							RA:	   -0.28%, -0.36%, -0.30%

LDB:     -0.70%, -0.62%, -0.62%

Table 4: Reordered GEO table

Mode number - Coding gain curve is given in Figure below to illustrate the trade off. (Anchor is VTM7.0).

Figure 1: Coding gain of 8, 16, 32, 48, 64 modes

Additional tests are conducted that using only first 16 modes with cmax of merge_geo_parititon_idx set to 64 and 16 respectively (software is implemented on CE4-1 and geo mode table order is not changed). From the simulation results, it can be observed that both signaling max value of cmax of merge_geo_partition_idx and geo mode table reordering contribute to coding efficiency.

Table 5: results of using only first 16 modes with cmax of geo_parititoning_idx set to 64 (VTM7.0 as anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.20% | 0.24% | 101% | 100% |
| Class A2 | 0.19% | 0.35% | 0.41% | 101% | 100% |
| Class B | 0.18% | 0.31% | 0.22% | 101% | 100% |
| Class C | 0.23% | 0.33% | 0.36% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.19% | 0.30% | 0.30% | 101% | 100% |
| Class D | 0.15% | 0.02% | 0.21% | 101% | 100% |
| Class F | 0.13% | 0.20% | 0.26% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.40% | 0.37% | 0.69% | 101% | 100% |
| Class C | 0.55% | 0.52% | 0.79% | 100% | 100% |
| Class E | 0.35% | 1.18% | 0.56% | 99% | 100% |
| Overall | 0.44% | 0.62% | 0.69% | 100% | 100% |
| Class D | 0.48% | 0.96% | 0.37% | 100% | 100% |
| Class F | 0.29% | 0.55% | 0.76% | 100% | 100% |

Table 6: results of using only first 16 modes with cmax of geo_parititoning_idx set to 16 (VTM7.0 as anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.05% | 0.07% | 101% | 101% |
| Class A2 | 0.05% | 0.18% | 0.22% | 100% | 100% |
| Class B | 0.06% | 0.09% | 0.22% | 101% | 101% |
| Class C | -0.01% | 0.02% | 0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.06% | 0.14% | 101% | 101% |
| Class D | -0.07% | 0.13% | 0.08% | 101% | 100% |
| Class F | 0.01% | 0.04% | 0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.14% | 0.20% | 0.46% | 101% | 101% |
| Class C | 0.18% | 0.04% | 0.23% | 100% | 100% |
| Class E | -0.19% | 0.88% | 0.78% | 99% | 100% |
| Overall | 0.07% | 0.31% | 0.47% | 100% | 100% |
| Class D | 0.06% | 0.25% | 0.24% | 100% | 100% |
| Class F | 0.21% | -0.01% | 0.49% | 100% | 100% |

## Fix number of modes to 32

From aspect 1.2, having number of modes equal to 32 gives a good trade off between number of cases to verify and coding efficiency. It is proposed to fix number of mode to 32.

## Reduce number of modes per block to 14

In this aspect, it is proposed to reduce number of modes per block to 14. For each block, only diagonal and invert diagonal directions are valid. GEO weight mask calculation process is reused. And 64x8 and 8x64 blocks are disallowed.

Figure 2: 14 valid modes per block

Table 7: Number of test cases of aspect 1.5

|  | # of cases | # modes per block size | # of supported block sizes |
| --- | --- | --- | --- |
| TPM | 50 | 2 | 25 |
| GEO (JVET-Q0059, CE4-1) | 1,216 | 64 | 19 |
| Aspect 1.4 | 238 | 14 | 17 |

# Experimental results

Coding performance is reported as below. The CTC (common test conditions) defined in [1] was followed.

Note that enc/dec time is not reliable.

Table 8: (aspect 2.1) Remove 64x8 and 8x64 sized GEO block (CE4 common base as anchor implemented over CE4 common base)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.01% | -0.05% | -0.06% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.09% | -0.15% | 104% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.03% | 0.04% | 105% | 101% |
| Class C | 0.03% | -0.04% | -0.16% | 105% | 99% |
| Class E | 0.11% | -0.14% | 0.02% | 102% | 99% |
| Overall | 0.03% | -0.04% | -0.03% | 104% | 100% |
| Class D | -0.10% | -0.29% | -0.63% | 106% | 102% |
| Class F | 0.00% | -0.25% | -0.07% | 102% | 99% |

Table 9: (aspect 2.2) scalable number of modes (16 modes, implemented over CE4-1, VTM7.0 as anchor):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.09% | 0.00% | 102% | 101% |
| Class A2 | -0.06% | 0.04% | 0.05% | 101% | 100% |
| Class B | -0.06% | 0.03% | -0.04% | 101% | 101% |
| Class C | -0.21% | -0.34% | -0.28% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | -0.09% | -0.08% | 101% | 100% |
| Class D | -0.16% | -0.32% | -0.25% | 101% | 100% |
| Class F | -0.22% | -0.21% | -0.19% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | -0.28% | -0.17% | 101% | 100% |
| Class C | -0.25% | -0.42% | -0.09% | 101% | 99% |
| Class E | -0.52% | 0.22% | -0.38% | 99% | 99% |
| Overall | -0.28% | -0.20% | -0.20% | 100% | 99% |
| Class D | -0.33% | 0.12% | -0.53% | 100% | 100% |
| Class F | -0.34% | -0.49% | 0.01% | 100% | 100% |

Table 10: (aspect 2.2) scalable number of modes (64 modes, implemented over CE4-1, VTM7.0 as anchor):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.20% | -0.18% | 103% | 101% |
| Class A2 | -0.19% | -0.14% | -0.02% | 103% | 100% |
| Class B | -0.14% | -0.11% | -0.09% | 102% | 100% |
| Class C | -0.61% | -0.97% | -0.86% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.36% | -0.30% | 102% | 100% |
| Class D | -0.41% | -0.64% | -0.52% | 102% | 100% |
| Class F | -0.33% | -0.34% | -0.35% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | -0.44% | -0.26% | 103% | 100% |
| Class C | -0.81% | -1.15% | -0.96% | 103% | 99% |
| Class E | -1.18% | -0.21% | -0.77% | 101% | 99% |
| Overall | -0.70% | -0.62% | -0.62% | 102% | 99% |
| Class D | -0.68% | -0.56% | -0.76% | 102% | 99% |
| Class F | -0.53% | -0.75% | 0.05% | 102% | 99% |

Table 11: (aspect 2.2/2.3) scalable number of modes (32 modes, implemented over CE4-1, VTM7.0 as anchor):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.19% | -0.07% | 102% | 101% |
| Class A2 | -0.13% | -0.12% | 0.02% | 101% | 100% |
| Class B | -0.12% | -0.08% | -0.11% | 101% | 100% |
| Class C | -0.41% | -0.60% | -0.53% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.25% | -0.19% | 101% | 100% |
| Class D | -0.31% | -0.46% | -0.52% | 101% | 100% |
| Class F | -0.29% | -0.32% | -0.27% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.29% | -0.34% | -0.17% | 101% | 100% |
| Class C | -0.59% | -0.80% | -0.48% | 101% | 99% |
| Class E | -0.84% | -0.24% | -0.49% | 99% | 99% |
| Overall | -0.53% | -0.47% | -0.35% | 100% | 99% |
| Class D | -0.54% | -0.46% | -0.64% | 100% | 99% |
| Class F | -0.41% | -1.17% | -0.22% | 100% | 99% |

Table 12: (aspect 2.2) scalable number of modes (8 modes, implemented over CE4-1, VTM7.0 as anchor):

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.18% | 0.21% | 102% | 101% |
| Class A2 | 0.03% | 0.17% | 0.23% | 101% | 100% |
| Class B | 0.06% | 0.21% | 0.13% | 101% | 100% |
| Class C | 0.06% | 0.15% | 0.13% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.18% | 0.16% | 101% | 100% |
| Class D | -0.03% | -0.01% | 0.00% | 101% | 100% |
| Class F | -0.11% | -0.03% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.09% | 0.26% | 101% | 100% |
| Class C | 0.18% | 0.09% | 0.40% | 101% | 99% |
| Class E | -0.14% | 0.57% | 0.08% | 99% | 99% |
| Overall | 0.06% | 0.14% | 0.26% | 100% | 99% |
| Class D | 0.00% | 0.32% | 0.48% | 100% | 99% |
| Class F | -0.03% | -0.47% | 0.43% | 100% | 99% |

Table 13: (aspect 2.2) scalable number of modes (48 modes, implemented over CE4-1, VTM7.0 as anchor):

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.15% | -0.18% | 103% | 101% |
| Class A2 | -0.17% | -0.07% | 0.02% | 102% | 100% |
| Class B | -0.15% | -0.11% | -0.10% | 102% | 100% |
| Class C | -0.55% | -0.79% | -0.74% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.29% | -0.27% | 102% | 100% |
| Class D | -0.41% | -0.53% | -0.60% | 102% | 100% |
| Class F | -0.33% | -0.36% | -0.29% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 102% | 100% |
| Class C | -0.76% | -1.14% | -0.81% | 102% | 99% |
| Class E | -0.93% | -0.30% | -1.11% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 102% | 99% |
| Class D | -0.64% | -0.42% | -0.44% | 102% | 99% |
| Class F | -0.51% | -1.04% | -0.09% | 102% | 99% |

Table 14: (aspect 2.4) fix geo number of mode per block to 14 (implemented over CE4-1, VTM7.0 as anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.08% | -0.07% | 100% | 101% |
| Class A2 | -0.05% | 0.02% | 0.10% | 99% | 100% |
| Class B | -0.06% | 0.04% | 0.08% | 100% | 100% |
| Class C | -0.25% | -0.37% | -0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.03% | 100% | 100% |
| Class D | -0.18% | -0.24% | -0.18% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.04% | 100% | 100% |
| TGM | -0.17% | -0.22% | -0.17% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.36% | -0.03% | 100% | 100% |
| Class C | -0.31% | -0.57% | -0.36% | 100% | 100% |
| Class E | -0.64% | -0.24% | -0.26% | 98% | 99% |
| Overall | -0.33% | -0.40% | -0.20% | 99% | 100% |
| Class D | -0.30% | -0.26% | -0.51% | 100% | 99% |
| Class F | -0.17% | -0.50% | 0.06% | 99% | 100% |

# Discussions

This contribution proposes several changes to GEO to solve concern regarding hardware validation efforts:

(1) disable 64x8 and 8x64 sized GEO block with coding impact of 0.**% (RA) and 0.03% (LDB) on Y component;

(2) support scalable number of modes with coding impact of -0.09% (RA) and -0.28% (LDB) on Y component for 16 modes and -0.28% (RA) and -0.70% (LDB) on Y component for 64 modes

(3) set number of modes to 32 with coding impact of -0.19% (RA) and -0.53% (LDB) on Y component

(4) fix number of modes per block to 14 with coding impact of -0.11% (RA) and -0.33% (LDB) on Y component.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).