| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0123-v3 |
| --- | --- |

| Title: | CE4-related: Modifications of GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution proposes several changes to GEO to address the issue regarding hardware validation checks for encoder.

(1) disable 64x8 and 8x64 sized GEO block;

(2) fix number of modes per block to 14 by only allowing (inverse) diagonal angle.

Coding performance is as below:

(1) disable 64x8 and 8x64 sized GEO block:				RA:	   0.00%, -0.02%, -0.02%

(CE4 common base as anchor)						LDB:     0.03%, -0.04%, -0.03%

(2)fix number of modes per block to 14					RA:	   -0.11%, -0.10%, -0.03%

(VTM7.0 as anchor)								LDB:     -0.33%, -0.40%, -0.20%

# Introduction

Geometrical partitioning (GEO) was firstly proposed in JVET-J1030/JVET-K0146 which applied to both AMVP and merge mode. In JVET-O0489, GEO was proposed for merge mode with 140 supported partitioning modes and 19 supported block size (8x8 to 128x128). In JVET-P0884/0885, efforts were made to remove multiplication of weight mask calculation and number of modes was reduced to 82. In JVET-Q0059, number of modes is further reduced to 64.

Continuous efforts are made to reduce complexity as well total number of cases to address hardware validation check issues.

Table 1: number of test cases between TPM, wedge mode and GEO

|  | Total # of cases | # modes per block size | # of supported block sizes |
| --- | --- | --- | --- |
| TPM | 50 | 2 | 25 |
| Wedge mode (in other standard) | 144 | 16 | 8 |
| GEO (JVET-O0489) | 2,660 | 140 | 19 |
| GEO (JVET-P0884/JVET-Q0079, CE4 common base) | 1,558 | 82 | 19 |
| GEO (JVET-Q0059, CE4-1) | 1,216 | 64 | 19 |

# Proposal

## Remove 64x8 and 8x64 sized GEO block

It is proposed to disable 64x8 and 8x64 sized coding block for GEO. This aspect is implemented on CE4 common base (JVET-Q0079). It is noted that this aspect can also be combined with CE4-2.1 so that the complexity can be reduced more efficiently. Same encoder change of CE4-2.1 (increase maximum tested modes) is applied.

## Reduce number of modes per block to 14

In this aspect, it is proposed to reduce number of modes per block to 14. For each block, only diagonal and invert diagonal directions are valid. Same GEO weight mask calculation process as CE4 common anchor is reused. And 64x8 and 8x64 blocks are disallowed for GEO mode.

Figure 2: 14 valid modes per block

Table 7: Number of test cases of aspect 1.5

|  | # of cases | # modes per block size | # of supported block sizes |
| --- | --- | --- | --- |
| TPM | 50 | 2 | 25 |
| GEO (JVET-Q0059, CE4-1) | 1,216 | 64 | 19 |
| Aspect 1.4 | 238 | 14 | 17 |

# Experimental results

Coding performance is reported as below. The CTC (common test conditions) defined in [1] was followed.

Note that enc/dec time is not reliable.

Table 8: (aspect 2.1) Remove 64x8 and 8x64 sized GEO block (CE4 common base as anchor implemented over CE4 common base)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.02% | 101% | 98% |
| Class A2 | 0.02% | 0.06% | -0.05% | 103% | 99% |
| Class B | -0.01% | 0.01% | 0.00% | 103% | 100% |
| Class C | -0.01% | -0.05% | -0.06% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.02% | 103% | 99% |
| Class D | -0.01% | -0.09% | -0.15% | 104% | 101% |
| Class F | 0.00% | 0.01% | -0.02% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.03% | 0.04% | 105% | 101% |
| Class C | 0.03% | -0.04% | -0.16% | 105% | 99% |
| Class E | 0.11% | -0.14% | 0.02% | 102% | 99% |
| Overall | 0.03% | -0.04% | -0.03% | 104% | 100% |
| Class D | -0.10% | -0.29% | -0.63% | 106% | 102% |
| Class F | 0.00% | -0.25% | -0.07% | 102% | 99% |

Table 9: (aspect 2.2) fix geo number of mode per block to 14 (implemented over CE4-1, VTM7.0 as anchor)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.08% | -0.07% | 100% | 101% |
| Class A2 | -0.05% | 0.02% | 0.10% | 99% | 100% |
| Class B | -0.06% | 0.04% | 0.08% | 100% | 100% |
| Class C | -0.25% | -0.37% | -0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.03% | 100% | 100% |
| Class D | -0.18% | -0.24% | -0.18% | 100% | 100% |
| Class F | -0.10% | -0.07% | -0.04% | 100% | 100% |
| TGM | -0.17% | -0.22% | -0.17% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | -0.36% | -0.03% | 100% | 100% |
| Class C | -0.31% | -0.57% | -0.36% | 100% | 100% |
| Class E | -0.64% | -0.24% | -0.26% | 98% | 99% |
| Overall | -0.33% | -0.40% | -0.20% | 99% | 100% |
| Class D | -0.30% | -0.26% | -0.51% | 100% | 99% |
| Class F | -0.17% | -0.50% | 0.06% | 99% | 100% |

# Discussions

This contribution proposes several changes to GEO to solve concern regarding hardware validation efforts:

(1) disable 64x8 and 8x64 sized GEO block with coding impact of 0.00% (RA) and 0.03% (LDB) on Y component;

(2) fix number of modes per block to 14 with coding impact of -0.11% (RA) and -0.33% (LDB) on Y component.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

[2]	H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, R.-L. Liao, J. Chen, Y. Ye, J. Luo, L. Xu, F. Chen, L. Wang, Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, M Bläser, J. Sauer, “CE4 Common Base: Geometric inter prediction”, JVET-Q0079, January 2020, Brussels, BE.

[3]	H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, R.-L. Liao, J. Chen, Y. Ye, J. Luo, L. Xu, F. Chen, L. Wang, Z. Deng, L. Zhang, H. Liu, K. Zhang, Y. Wang, M Bläser, J. Sauer, “CE4-1: Geometric Inter Prediction with 64 Modes”, JVET-Q0059, January 2020, Brussels, BE.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).