| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0144 |
| --- | --- |

| Title: | Non-CE3: Contexts in Chroma transform skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to introduce individual contexts for luma and chroma in all syntaxes of  transform skip residual (TSRC) to improve coding gains of Lossless coding. Derivation process for ctxInc for chroma TSRC contexts is completely the same as those for luma TSRC contexts. Simulation results of Lossless coding, CTC, and LowQP are reportedly shown as follows:

Lossless:

Overall:     -0.23%AI,  -0.65%RA,  -0.93%LB

CTC:

ClassF:      0.00%AI,  -0.02%RA,   0.00%LB

ClassTGM:  -0.11%AI,  0.00%RA,   -0.02%LB

LowQP

ClassF:     -0.09%AI,  -0.05%RA,   -0.05%LB

ClassTGM:  -0.28%AI,  0.06%RA,   -0.20%LB

Introduction

Chroma transform skip (TS) was adopted [2] in the last meeting. In current TS, the same contexts are used from coded_sub_block_flag to abs_level_gt9_flag between Luma and Chroma, whereas transform residual (RRC) uses different contexts between Luma and Chroma. (Table1)

Table 1: Number of contexts in RRC and TSRC

|  | RRC |  | TSRC |
| --- | --- | --- | --- |
|  | Luma | Chroma | Luma/Chroma |  |
| coded_sub_block_flag | 2 | 2 | 3 |  |
| sig_coeff_flag | 36 | 24 | 3 |  |
| coeff_sign_flag |  |  | 6 |  |
| abs_level_gt1_flag | 21 | 11 | 4 |  |
| parity_flag | 21 | 11 | 1 |  |
| abs_level_gt3_flag | 21 | 11 | 1 |  |
| abs_level_gt5_flag |  |  | 1 |  |
| abs_level_gt7_flag |  |  | 1 |  |
| abs_level_gt9_flag |  |  | 1 |  |

Proposal

It is proposed to introduce chroma contexts in TSRC to improve coding gains in Lossless mode. Derivation process for ctxInc for chroma TS contexts is completely the same as those for luma TS contexts.

Table 2: Number of contexts in RRC and proposed TSRC

|  | RRC |  | TSRC |  |
| --- | --- | --- | --- | --- |
|  | Luma | Chroma | Luma | Chroma |
| coded_sub_block_flag | 2 | 2 | 3 | 3 |
| sig_coeff_flag | 36 | 24 | 3 | 3 |
| coeff_sign_flag |  |  | 6 | 6 |
| abs_level_gt1_flag | 21 | 11 | 4 | 4 |
| parity_flag | 21 | 11 | 1 | 1 |
| abs_level_gt3_flag | 21 | 11 | 1 | 1 |
| abs_level_gt5_flag |  |  | 1 | 1 |
| abs_level_gt7_flag |  |  | 1 | 1 |
| abs_level_gt9_flag |  |  | 1 | 1 |

Simulation results

Table 3, 4 and 5 show the lossless, CTC and LowQP results of proposed chroma contexts, respectively. In this test, QP value in configuration file is set to be 4. Runtime of LowQP is not accurate because of different clusters.

Table 3: Simulation results of proposal in lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | proposal |  | VTM7.0 | proposal |  | VTM7.0 | proposal |  |
| Class A1 | 2.2 | 2.2 | -0.28% | 2.2 | 2.3 | -0.79% |  |  |  |
| Class A2 | 1.6 | 1.6 | -0.05% | 1.7 | 1.7 | -0.24% |  |  |  |
| Class B | 2.2 | 2.2 | -0.19% | 2.3 | 2.4 | -0.59% | 2.3 | 2.4 | -0.60% |
| Class C | 1.9 | 1.9 | -0.22% | 2.4 | 2.5 | -0.91% | 2.4 | 2.4 | -0.96% |
| Class D | 1.9 | 1.9 | -0.23% | 2.8 | 2.8 | -0.85% | 2.7 | 2.8 | -0.92% |
| Class E | 2.8 | 2.8 | -0.41% |  |  |  | 3.1 | 3.2 | -1.44% |
| Class F | 5.3 | 5.3 | -0.17% | 33.7 | 33.9 | -0.43% | 50.7 | 50.9 | -0.45% |
| TGM | 11.8 | 11.9 | -0.36% | 107.1 | 107.6 | -0.55% | 124.9 | 125.4 | -0.52% |
| Overall | 2.1 | 2.1 | -0.23% | 2.2 | 2.2 | -0.65% | 2.6 | 2.6 | -0.93% |
| Enc Time[%] | 100% |  |  | 96% |  |  | 91% |  |  |
| Dec Time[%] | 108% |  |  | 102% |  |  | 99% |  |  |

Table 4: Simulation results of proposal in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.03% | 100% | 99% |
| Class A2 | -0.01% | 0.01% | -0.04% | 100% | 99% |
| Class B | 0.00% | -0.03% | -0.02% | 100% | 100% |
| Class C | 0.01% | 0.00% | 0.05% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.07% | 0.03% | 100% | 99% |
| Class F | 0.00% | 0.02% | -0.01% | 100% | 100% |
| TGM | -0.11% | -0.35% | -0.35% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.06% | 0.08% | 101% | 99% |
| Class A2 | -0.03% | 0.03% | 0.06% | 99% | 98% |
| Class B | -0.01% | 0.03% | -0.10% | 101% | 99% |
| Class C | 0.02% | 0.08% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 99% |
| Class D | -0.03% | -0.22% | -0.16% | 100% | 100% |
| Class F | 0.01% | 0.02% | -0.06% | 101% | 100% |
| TGM | -0.08% | -0.32% | -0.33% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.20% | -0.16% | 99% | 98% |
| Class C | 0.00% | -0.21% | -0.04% | 104% | 98% |
| Class E | 0.04% | 0.09% | -0.11% | 100% | 96% |
| Overall | 0.00% | -0.13% | -0.11% | 101% | 97% |
| Class D | -0.03% | -0.15% | -0.51% | 104% | 101% |
| Class F | 0.00% | -0.13% | 0.77% | 100% | 98% |
| TGM | -0.02% | -0.37% | -0.40% | 101% | 99% |

Table 5: Simulation results of proposal in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.03% | 102% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 102% | 100% |
| Class B | -0.01% | 0.01% | 0.00% | 100% | 101% |
| Class C | -0.01% | -0.02% | -0.02% | 110% | 112% |
| Class E | -0.07% | 0.01% | 0.05% | 106% | 105% |
| Overall | -0.02% | 0.00% | 0.00% | 104% | 104% |
| Class D | -0.02% | 0.02% | -0.02% | 92% | 90% |
| Class F | -0.09% | -0.14% | -0.12% | 105% | 106% |
| TGM | -0.28% | -0.45% | -0.44% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.02% | -0.05% | 97% | 95% |
| Class C | 0.00% | -0.09% | -0.11% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.01% | -0.16% | 98% | 97% |
| Class F | -0.05% | -0.07% | 0.02% | 98% | 97% |
| TGM | 0.06% | -0.30% | -0.32% | 103% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.03% | -0.03% | 98% | 100% |
| Class C | 0.00% | -0.03% | -0.05% | 94% | 90% |
| Class E | -0.03% | 0.03% | 0.05% | 96% | 94% |
| Overall | -0.01% | -0.02% | -0.02% | 96% | 95% |
| Class D | 0.00% | 0.01% | -0.06% | 111% | 108% |
| Class F | -0.05% | -0.10% | -0.18% | 94% | 95% |
| TGM | -0.20% | -0.43% | -0.41% | 112% | 108% |

Conclusion

This proposal introduces chroma contexts in transform skip residual to improve coding gains in Lossless mode. Simulation results show the good coding gains in RA and LB of lossless mode. We suggest to adopt this proposal into the next test model.

References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[2] Takeshi Tsukuba , Masaru Ikeda, Yoichi Yagasaki, Teruhiko Suzuki “ CE8-2.1: Transform Skip for Chroma with limiting maximum number of context-coded bin in TS residual coding”, JVET-P0058

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).