| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0145 |
| --- | --- |

| Title: | CE3-related: Unified rice parameter derivation of transform skip residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to derive Rice parameter in transform skip residual coding (TSRC) in an unified way with transform regular residual coding (RRC). This method is based on adopted rice parameter derivation of CE7-.1.3b-alt [2] in the last meeting. The same Rice parameter table as transform residual coding (RRC) is used for TSRC, and sum value used in deriving Rice parameter is normalized based on the number of reference neighbor coefficients.

Simulation results of Lossless coding, CTC, and LowQP reportedly are shown as follows:

Lossless:

Overall (ClassA-E):	-4.87%AI,  -3.39%RA,  -2.29%LB

ClassF:     			-3.81%AI,  -3.00%RA,  -2.74%LB

ClassTGM:  		-3.80%AI,  -6.00%RA,  -5.98%LB

CTC:

Overall:	       -0.01%AI,  -0.00%RA,  -0.02%LB

ClassF:     			-0.14%AI,  -0.15%RA,  -0.15%LB

ClassTGM:  		-0.36%AI,  -0.18%RA,  -0.25%LB

LowQP:

Overall:	       -0.07%AI,  -0.XX%RA,  -0.02%LB

ClassF:     			-1.30%AI,  -1.63%RA,  -0.58%LB

ClassTGM:  		-1.17%AI,  -1.83%RA,  -1.02%LB

Introduction

In the current VTM7.0, Rice parameter which is used in bypass coding in TSRC is fixed to be equal to 1. This leads to performance degradation of TSRC as compared to RRC in lossless coding.

Proposal

In our proposal, Rice parameter is calculated by using the sum of two reference coefficients in the similar way as RRC. The same Rice parameter Table as that of RRC is used. To compensate for the difference in the number of reference coefficients between RRC and TSRC, component value of TSRC is normalized by doubling. This form is the normalized version of the Rice parameter derivation method proposed in CE7-1.3b-alt [2].

Table 1 shows the comparison of Rice parameter derivation methods between RRC and TSRC of our proposal. Rice parameter derivation method of CE3-1.2 is also shown for comparison.

Table 1: Comparison of rice parameter derivation method: RRC, TSRC(proposal), and CE 3-1.2

|  | RRC (VTM7.0) | TSRC (proposal) | CE3-1.2 |
| --- | --- | --- | --- |
| Number of reference coefficients used | 5 | 2 | 2 |
| Component assigned to Rice parameter Table | Sum – 5*baseLevel | (Sum – 2*baseLevel)<<1 | Sum + offset |
| Rice parameter Table | A | A | B |

Here,

- Rice parameter Table A is { 0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3}

- Rice parameter Table B is { 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3 }

- offset is equal to ( baseLevel == 10 ? -8 : (baseLevel == 0 ? 0 : -4) ).

Rice parameter Table can be saved for TSRC in our proposal.

Simulation results

Table 2, 3 and 4 show the lossless, CTC and LowQP results of the proposed Rice parameter derivation, respectively. In this test, QP value in configuration file is set to be 4. Runtime of LowQP is not accurate because of different clusters.

Table 2: Simulation results: Lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | Qxxxx |  | VTM7.0 | Qxxxx |  | VTM7.0 | Qxxxx |  |
| Class A1 | 2.2 | 2.3 | -4.38% | 2.2 | 2.3 | -4.44% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.29% | 1.7 | 1.8 | -6.77% |  |  |  |
| Class B | 2.2 | 2.2 | -3.36% | 2.3 | 2.4 | -1.49% | 2.3 | 2.4 | -1.50% |
| Class C | 1.9 | 2.0 | -4.76% | 2.4 | 2.5 | -2.45% | 2.4 | 2.5 | -2.30% |
| Class D | 1.9 | 2.0 | -5.26% | 2.8 | 2.8 | -2.61% | 2.7 | 2.8 | -2.44% |
| Class E | 2.8 | 2.9 | -4.59% |  |  |  | 3.1 | 3.2 | -3.60% |
| Class F | 5.3 | 5.5 | -3.81% | 33.7 | 34.8 | -3.00% | 50.7 | 51.9 | -2.74% |
| TGM | 11.8 | 12.3 | -3.80% | 107.1 | 115.8 | -6.00% | 124.9 | 135.7 | -5.98% |
| Overall | 2.1 | 2.2 | -4.87% | 2.2 | 2.3 | -3.39% | 2.6 | 2.6 | -2.29% |
| Enc Time[%] | 100% |  |  | 102% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  | 104% |  |  |

Table 3: Simulation results: CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.03% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | -0.01% | 99% | 98% |
| Class B | 0.00% | 0.03% | -0.10% | 101% | 101% |
| Class C | -0.03% | 0.03% | 0.01% | 100% | 100% |
| Class E | 0.00% | 0.01% | -0.07% | 100% | 102% |
| Overall | -0.01% | 0.01% | -0.03% | 100% | 100% |
| Class D | -0.06% | 0.07% | -0.05% | 100% | 100% |
| Class F | -0.14% | -0.15% | 0.04% | 100% | 99% |
| TGM | -0.36% | -0.29% | -0.34% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class A2 | -0.04% | -0.06% | -0.01% | 100% | 99% |
| Class B | 0.01% | -0.04% | 0.06% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.02% | 100% | 100% |
| Class D | -0.05% | -0.17% | -0.03% | 100% | 99% |
| Class F | -0.15% | 0.01% | -0.05% | 100% | 99% |
| TGM | -0.18% | -0.20% | -0.19% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.32% | -0.20% | 100% | 98% |
| Class C | 0.00% | -0.29% | -0.15% | 100% | 99% |
| Class E | -0.07% | 0.44% | -0.36% | 100% | 99% |
| Overall | -0.02% | -0.12% | -0.22% | 100% | 99% |
| Class D | -0.01% | -0.09% | -0.48% | 100% | 99% |
| Class F | -0.15% | -0.47% | 0.01% | 100% | 100% |
| TGM | -0.25% | -0.36% | -0.35% | 100% | 99% |

Table 4: Simulation results: LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.03% | -0.04% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.02% | 100% | 99% |
| Class B | -0.03% | -0.06% | -0.09% | 99% | 99% |
| Class C | -0.13% | -0.14% | -0.13% | 99% | 99% |
| Class E | -0.18% | -0.30% | -0.39% | 100% | 100% |
| Overall | -0.07% | -0.11% | -0.13% | 100% | 99% |
| Class D | -0.15% | -0.18% | -0.18% | 100% | 100% |
| Class F | -1.30% | -0.74% | -0.72% | 99% | 99% |
| TGM | -1.17% | -1.04% | -1.03% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.02% | -0.04% | 96% | 94% |
| Class C | -0.05% | -0.05% | -0.06% | 103% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.09% | -0.01% | -0.17% | 97% | 95% |
| Class F | -1.63% | -0.59% | -0.68% | 99% | 97% |
| TGM | -1.83% | -1.06% | -1.03% | 106% | 105% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.01% | 100% | 100% |
| Class C | -0.03% | -0.01% | 0.01% | 100% | 100% |
| Class E | -0.05% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.02% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.05% | -0.01% | -0.20% | 100% | 100% |
| Class F | -0.58% | -0.50% | -0.57% | 100% | 99% |
| TGM | -1.02% | -0.69% | -0.69% | 101% | 100% |

Table 5, 6 and 7 show the lossless, CTC and LowQP results of proposal over CE3-1.2.

Table 5: Simulation results over CE3-1.2: Lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | CE3-1.2 | Qxxxx |  | CE3-1.2 | Qxxxx |  | CE3-1.2 | Qxxxx |  |
| Class A1 | 2.3 | 2.3 | -0.86% | 2.3 | 2.3 | -0.86% |  |  |  |
| Class A2 | 1.7 | 1.7 | -1.52% | 1.8 | 1.8 | -1.45% |  |  |  |
| Class B | 2.2 | 2.2 | -0.27% | 2.4 | 2.4 | -0.07% | 2.4 | 2.4 | -0.08% |
| Class C | 2.0 | 2.0 | -0.44% | 2.5 | 2.5 | 0.01% | 2.5 | 2.5 | 0.04% |
| Class D | 2.0 | 2.0 | -0.53% | 2.8 | 2.8 | -0.02% | 2.8 | 2.8 | 0.03% |
| Class E | 2.9 | 2.9 | -0.16% |  |  |  | 3.2 | 3.2 | 0.06% |
| Class F | 5.5 | 5.5 | -0.15% | 34.8 | 34.8 | -0.03% | 52.1 | 51.9 | 0.10% |
| TGM | 12.3 | 12.3 | -0.01% | 115.7 | 115.8 | -0.05% | 136.0 | 135.7 | 0.20% |
| Overall | 2.2 | 2.2 | -0.60% | 2.3 | 2.3 | -0.48% | 2.6 | 2.6 | -0.01% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  | #NUM! |  |  |

Table 6: Simulation results over CE3-1.2: CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-1.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | #NUM! | #NUM! |
| Class A2 | 0.00% | 0.00% | 0.01% | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | -0.05% | #NUM! | #NUM! |
| Class C | 0.00% | 0.02% | -0.05% | #NUM! | #NUM! |
| Class E | 0.00% | 0.04% | 0.00% | #NUM! | #NUM! |
| Overall | 0.00% | 0.01% | -0.02% | #NUM! | #NUM! |
| Class D | 0.01% | 0.06% | 0.06% | #NUM! | #NUM! |
| Class F | 0.03% | 0.06% | 0.10% | #NUM! | #NUM! |
| TGM | 0.05% | 0.05% | 0.03% | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE3-1.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | #NUM! | #NUM! |
| Class A2 | 0.01% | -0.01% | -0.04% | #NUM! | #NUM! |
| Class B | -0.01% | 0.01% | 0.00% | #NUM! | #NUM! |
| Class C | 0.02% | 0.10% | 0.11% | #NUM! | #NUM! |
| Class E |  |  |  | #NUM! | #NUM! |
| Overall | 0.00% | 0.03% | 0.02% | #NUM! | #NUM! |
| Class D | -0.03% | -0.10% | 0.02% | #NUM! | #NUM! |
| Class F | 0.03% | 0.02% | 0.07% | #NUM! | #NUM! |
| TGM | 0.09% | 0.02% | -0.01% | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE3-1.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.02% | -0.10% | #NUM! | #NUM! |
| Class C | -0.01% | -0.11% | 0.08% | #NUM! | #NUM! |
| Class E | -0.01% | 0.05% | 0.11% | #NUM! | #NUM! |
| Overall | 0.00% | -0.03% | 0.01% | #NUM! | #NUM! |
| Class D | 0.01% | -0.87% | 0.27% | #NUM! | #NUM! |
| Class F | -0.01% | -0.18% | -0.43% | #NUM! | #NUM! |
| TGM | 0.01% | -0.09% | -0.17% | #NUM! | #NUM! |

Table 7: simulation results over CE3-1.2: LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE3-1.2(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.00% | #NUM! | #NUM! |
| Class A2 | 0.00% | -0.01% | 0.00% | #NUM! | #NUM! |
| Class B | 0.00% | -0.01% | -0.01% | #NUM! | #NUM! |
| Class C | -0.03% | -0.01% | -0.02% | #NUM! | #NUM! |
| Class E | -0.01% | 0.00% | -0.01% | #NUM! | #NUM! |
| Overall | 0.00% | -0.01% | -0.01% | #NUM! | #NUM! |
| Class D | -0.05% | -0.04% | -0.05% | #NUM! | #NUM! |
| Class F | 0.03% | 0.07% | 0.04% | #NUM! | #NUM! |
| TGM | 0.18% | 0.18% | 0.19% | #NUM! | #NUM! |
|  | Random Access Main 10 |  |  |  |  |
|  | Over CE3-1.2(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.02% | 0.02% | #NUM! | #NUM! |
| Class C | -0.01% | 0.01% | 0.00% | #NUM! | #NUM! |
| Class E |  |  |  | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.06% | -0.07% | #NUM! | #NUM! |
| Class F | 0.13% | 0.18% | 0.10% | #NUM! | #NUM! |
| TGM | 0.12% | 0.07% | 0.12% | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE3-1.2(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.00% | #NUM! | #NUM! |
| Class C | 0.01% | -0.01% | 0.02% | #NUM! | #NUM! |
| Class E | 0.02% | -0.02% | 0.05% | #NUM! | #NUM! |
| Overall | 0.01% | -0.01% | 0.02% | #NUM! | #NUM! |
| Class D | 0.00% | 0.04% | -0.02% | #NUM! | #NUM! |
| Class F | -0.02% | 0.06% | 0.11% | #NUM! | #NUM! |
| TGM | 0.16% | 0.19% | 0.18% | #NUM! | #NUM! |

Conclusion

It is proposed to derive Rice parameter for TSRC by using the sum of two reference coefficients in the similar way as RRC. Simulation results show the interesting gain especially in lossless and LowQP modes. We suggest to adopt this proposal into the next test model.

References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[2] Mohammed Golam Sarwer, Ruling Liao, Daniel Luo, Yan Ye, “CE7-1.3: Simplification of transform-skip residual coding”, JVET-P0072

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).