| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0151 |
| --- | --- |

| Title: | AHG12: On entry point offset signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Yao-Jen Chang Vadim Seregin Adarsh K. Ramasubramonian Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution signalling of subset of entry point offsets necessary for enabling raster scan processing of CTBs at picture level when picture partitioning and/or WPP is used is proposed.

# Introduction

In VVC (Draft 7) [1] when tiles and/or WPP are used, entry_point_offsets can be optionally signalled at slice header level. When tiles only case is used, entry point offsets correspond to tile starting points (except for the first one in the slice).   When WPP is used, entry point offsets correspond to start of is CTB row in each tile (except of the first one in the slice). Certain decoders may be designed to process CTBs in raster scan order at picture level. To support raster scan processing of CTBs in such decoders, signalling of entry point offsets for all tiles except for the ones at the left picture boundary (when subpicture merging/extraction is not used) is necessary when tiles and/or WPP is used. WPP entry point offsets are not needed in such processing case.

# Proposal

In order to support raster processing of CTBs at picture level, it is proposed to mandate signalling subset of entry point offsets that is required to enable such decoding scheme. The subset of entry point offsets corresponds to tile entry point offsets only. Optionally WPP entry point offsets can be signalled.

Proposed spec. changes:

=========================VVC draft 7 changes BEGIN==============================

#### 7.3.2.4 Picture parameter set RBSP syntax

| … |  |
| --- | --- |
| entropy_coding_sync_enabled_flag | u(1) |
| if( !no_pic_partition_flag ¦ ¦ entropy_coding_sync_enabled_flag ) |  |
| entry_point_offsets_present_flag | u(1) |
| … | u(1) |

#### 7.3.7.1 General slice header syntax

| …. |  |
| --- | --- |
| if( entry_point_offsets_present_flag && ¦ ¦ NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

#### 7.4.8.1 General slice header semantics

…

When entry_point_offsets_present_flag is equal to 1 or no_pic_partition_flag is equal to 1, the variable NumEntryPoints, which specifies the number of entry points in the current slice, is derived as follows:

NumEntryPoints = 0
for( i = 1; i < NumCtuInCurrSlice; i++ ) {
	CtbAddrInRs = CtbAddrInCurrSlice[ i ]
	CtbAddrX = ( CtbAddrInRs % PicWidthInCtbsY )
	CtbAddrY = ( CtbAddrInRs / PicWidthInCtbsY )	(147)
	if( CtbAddrX  = =  CtbToTileColBd[ CtbAddrX ]  &&
		( CtbAddrY  = =  CtbToTileRowBd[ CtbAddrY ]  | |  (entropy_coding_sync_enabled_flag && entry_point_offsets_present_flag) )
		NumEntryPoints++
}

offset_len_minus1 plus 1 specifies the length, in bits, of the entry_point_offset_minus1[ i ] syntax elements. The value of offset_len_minus1 shall be in the range of 0 to 31, inclusive.

entry_point_offset_minus1[ i ] plus 1 specifies the i-th entry point offset in bytes, and is represented by offset_len_minus1 plus 1 bits. The slice data that follow the slice header consists of NumEntryPoints + 1 subsets, with subset index values ranging from 0 to NumEntryPoints, inclusive. The first byte of the slice data is considered byte 0. When present, emulation prevention bytes that appear in the slice data portion of the coded slice NAL unit are counted as part of the slice data for purposes of subset identification. Subset 0 consists of bytes 0 to entry_point_offset_minus1[ 0 ], inclusive, of the coded slice data, subset k, with k in the range of 1 to NumEntryPoints − 1, inclusive, consists of bytes firstByte[ k ] to lastByte[ k ], inclusive, of the coded slice data with firstByte[ k ] and lastByte[ k ] defined as:

firstByte[ k ] =n=1k( entry_point_offset_minus1[ n-1 ]+1 )	(148)

lastByte[ k ] = firstByte[ k ] + entry_point_offset_minus1[ k ]	(149)

The last subset (with subset index equal to NumEntryPoints) consists of the remaining bytes of the coded slice data.

When entropy_coding_sync_enabled_flag is equal to 0 and the slice contains one or more complete tiles, each subset shall consist of all coded bits of all CTUs in the slice that are within the same tile, and the number of subsets (i.e., the value of NumEntryPoints + 1) shall be equal to the number of tiles in the slice.

When entropy_coding_sync_enabled_flag is equal to 0 and the slice contains a subset of CTU rows from a single tile, the NumEntryPoints shall be 0, and the number of subsets shall be 1. The subset shall consist of all coded bits of all CTUs in the slice.

When entropy_coding_sync_enabled_flag is equal to 1 and entry_point_offsets_present_flag is equal to 1, each subset k with k in the range of 0 to NumEntryPoints, inclusive, shall consist of all coded bits of all CTUs in a CTU row within a tile, and the number of subsets ( i.e., the value of NumEntryPoints + 1 ) shall be equal to the total number of tile-specific CTU rows in the slice.

slice_header_extension_length specifies the length of the slice header extension data in bytes, not including

=========================VVC draft 7 changes END==============================

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).