| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0159 |
| --- | --- |

| Title: | AHG8: Indication for output picture size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Adarsh K. Ramasubramonian Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

Reference picture resampling allows to have different decoded picture sizes and such different size pictures can be output. In such cases, the application may need to rescale the output picture before displaying to have a uniform size of the pictures. VVC decoder has resampling filters and the decoder may perform output picture scaling process, however the target picture size is not currently present in the bitstream.

In this contribution, it is proposed to provide alternative picture output information, which can be used to rescale an output picture. The output picture size is proposed to be signalled in the VUI.

# Introduction

Similar concept was proposed in JVET-P0591, where the output picture size and cropping window were signalled in SPS and PPS, respectively, with the resampling performed in the picture output process. Some experts commented that such indication does not affect the decoding process and may be outside of the codec scope.

With RPR, the decoded output pictures may have various picture sizes and currently such cropped different resolution pictures are output by decoder. After cropping, the output picture may be just directly scaled by decoder, as it already supports resampling filters, to the desired picture output size to have a uniform resolution output; however, the target picture size is not available at the decoder.

Currently, only max picture size as a multiple of 8 samples is available in the SPS, and a picture size as a multiple of 8 samples with a cropping window information is available in PPS.

# Proposal

In this contribution, it is proposed to provide alternative picture output information, where the rescaling can be performed to bring the picture to the desired picture size. The desired output picture size is signaled in the VUI in this proposal.

Using this information, if the output picture size is different from the picture size signaled in VUI a decoder may perform picture rescaling. Since these processes may be outside of the codec scope, only indication is proposed and no associated decoding processes (e.g., different decoders may choose different rescaling filters).

The scaling ratio for such a rescaling is derived by using the picture size after cropping (using conformance cropping window) and the one signaled in VUI, since both do not have a multiple of 8 samples picture size constraint.

| vui_parameters( ) { | Descriptor |
| --- | --- |
| … |  |
| alternative_output_picture_flag | u(1) |
| if( alternative_output_picture_flag ) { |  |
| alternative_output_pic_width_in_luma_samples | ue(v) |
| alternative_output_pic_height_in_luma_samples | ue(v) |
| } |  |
| … |  |

alternative_output_picture_flag equal to 1 specifies that an alternative output picture size is present in VUI, which may be used to resample the output picture. alternative_output_picture_flag equal to 0 specifies that the alternative output picture size is not present.

alternative_output_pic_width_in_luma_samples specifies the alternative width of each output picture referring to the SPS in units of luma samples. alternative_output_pic_width_in_luma_samples shall not be equal to 0 and shall be less than or equal to pic_width_max_in_luma_samples.

alternative_output_pic_height_in_luma_samples specifies the alternative height of each output picture referring to the SPS in units of luma samples. alternative_output_pic_height_in_luma_samples shall not be equal to 0 and shall be less than or equal to pic_height_max_in_luma_samples.

The alternative_output_pic_width_in_luma_samples and alternative_output_pic_height_in_luma_samples, when present, may be used to determine an alternative output picture size for the pictures output from the DPB after the cropping with the conformance cropping window. The output cropped pictures may be scaled by using scaling ratio values horScale and verScale derived as follows:

PicCroppedWidthL = pic_width_in_luma_samples − 	
			 SubWidthC * ( conf_win_right_offset + conf_win_left_offset )

PicCroppedHeightL = pic_height_in_luma_samples −			SubHeightC * ( conf_win_bottom_offset + conf_win_top_offset )

horScale = ( ( PicCroppedWidthL  <<  14 ) + ( alternative_output_pic_width_in_luma_samples >>  1 ) ) / alternative_output_pic_width_in_luma_samples

verScale = ( ( PicCroppedHeightL  <<  14 ) + ( alternative_output_pic_height_in_luma_samples >>  1 ) ) / alternative_output_pic_height_in_luma_samples

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).