| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0163 |
| --- | --- |

| Title: | AHG9: On mixed NAL unit types in a video picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ping Wu | Email: | pwuzte@gmail.com |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In JVET-P2001 (VVC Draft 7), there is a design on indicating mixed NAL unit types for a video picture, which depends on a syntax element “mixed_nalu_types_in_pic_flag” in Picture Parameter Set (PPS). In this contribution, it proposes to add another syntax element as “mixed_irap_nalu_types_in_pic_flag” to indicate mixed irap nalu types and only irap nalu types in a video picture. The added indication will support wider range of application, particularly involving subpictures partition applications. The added syntax element can be used to serve a flexible random access point indications with rather independently coded subpictures in a video picture.

# Introduction and Problem Statement

1.1 The relevant existing design for NAL unit types in VVC specification JVET-P2001:

The information below is extracted from JVET-P2001 [1], The key information is highlighted in “blue” colour:

In VVC Spec’s Table 5 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure |  | NAL unit type class |
| --- | --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) |  | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) |  | VCL |
| 2 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) |  | VCL |
| 3 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) |  | VCL |
| 4..6 | RSV_VCL_4.. RSV_VCL_6 | Reserved non-IRAP VCL NAL unit types |  | VCL |
| 7 8 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) |  | VCL |
| 9 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) |  | VCL |
| 10 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) |  | VCL |
| 11 12 | RSV_IRAP_11 RSV_IRAP_12 | Reserved IRAP VCL NAL unit types |  | VCL |
| 13 | DPS_NUT |  | Decoding parameter set decoding_parameter_set_rbsp( ) | non-VCL |
| 14 | VPS_NUT |  | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 15 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) |  | non-VCL |
| 16 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) |  | non-VCL |
| 17 18 | PREFIX_APS_NUT SUFFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) |  | non-VCL |
| 19 | PH_NUT | Picture header picture_header_rbsp( ) |  | non-VCL |
| 20 | AUD_NUT | AU delimiter access_unit_delimiter_rbsp( ) |  | non-VCL |
| 21 | EOS_NUT | End of sequence end_of_seq_rbsp( ) |  | non-VCL |
| 22 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) |  | non-VCL |
| 23 24 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) |  | non-VCL |
| 25 | FD_NUT | Filler data filler_data_rbsp( ) |  | non-VCL |
| 26 27 | RSV_NVCL_26 RSV_NVCL_27 | Reserved non-VCL NAL unit types |  | non-VCL |
| 28..31 | UNSPEC_28.. UNSPEC_31 | Unspecified non-VCL NAL unit types |  | non-VCL |

Relating to random access conditions, some key terms are defined in VVC specification. In JVET-P2001 spec, under 3 “Definitions”:

  - clean random access (CRA) picture: An IRAP picture for which each VCL NAL unit has nal_unit_type equal to CRA_NUT.

NOTE – A CRA picture does not refer to any pictures other than itself for inter prediction in its decoding process, and may be the first picture in the bitstream in decoding order, or may appear later in the bitstream. A CRA picture may have associated RADL or RASL pictures. When a CRA picture has NoIncorrectPicOutputFlag equal to 1, the associated RASL pictures are not output by the decoder, because they may not be decodable, as they may contain references to pictures that are not present in the bitstream.

3.54 	gradual decoding refresh (GDR) picture: A picture for which each VCL NAL unit has nal_unit_type equal to GDR_NUT.

  - instantaneous decoding refresh (IDR) picture: An IRAP picture for which each VCL NAL unit has nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

NOTE – An IDR picture does not refer to any pictures other than itself for inter prediction in its decoding process, and may be the first picture in the bitstream in decoding order, or may appear later in the bitstream. Each IDR picture is the first picture of a CVS in decoding order. When an IDR picture for which each VCL NAL unit has nal_unit_type equal to IDR_W_RADL, it may have associated RADL pictures. When an IDR picture for which each VCL NAL unit has nal_unit_type equal to IDR_N_LP, it does not have any associated leading pictures. An IDR picture does not have associated RASL pictures.

  - intra random access point (IRAP) picture: A coded picture for which all VCL NAL units have the same value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive.

NOTE 1 – An IRAP picture does not refer to any pictures other than itself for inter prediction in its decoding process, and may be a CRA picture or an IDR picture. The first picture in the bitstream in decoding order must be an IRAP or GDR picture. Provided the necessary parameter sets are available when they need to be referenced, the IRAP picture and all subsequent non-RASL pictures in the CVS in decoding order can be correctly decoded without performing the decoding process of any pictures that precede the IRAP picture in decoding order.

NOTE 2 – The value of mixed_nalu_types_in_pic_flag for an IRAP picture is equal to 0. When mixed_nalu_types_in_pic_flag is equal to 0 for a picture, and any slice of the picture has nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, all other slices of the picture have the same value of nal_unit_type, and the picture is known to be an IRAP picture.

For an indication of mixed type of NAL unit types for a picture, the flag (syntax element) “mixed_nalu_types_in_pic_flag” has been located in PPS (Picture Parameter Set) as:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| ……………………………………… |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| …………………………………….. |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In VVC spec, under 7.4.3.4 Semantics:

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	For all the following PUs in the CLVS in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

The flag “no_mixed_nalu_types_in_pic_constraint_flag” has been located inside general_constraint_info( ) as the follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| ………………………………………………. |  |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| …………………………………………………. |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| num_reserved_constraint_bytes | u(8) |
| for( i = 0; i < num_reserved_constraint_bytes; i++ ) |  |
| gci_reserved_constraint_byte[ i ] | u(8) |
| } |  |

The relevant semantics:

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint.

In VVC spec  under 7.4.2.2 “NAL unit header semantics”, it states:

For VCL NAL units of any particular picture, the following applies:

–	If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

–	Otherwise (mixed_nalu_types_in_pic_flag equal to 1), one or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GRA_NUT.

Note that there is a typo in above paragraph. The word “GRA_NUT” is not correct; for instance. In above Table 5, there is no entry for “GRA_NUT”. A fix has been provided in JVET-Q004 for current VVC specification: replacing GRA_NUT with GDR_NUT. The typo is highlighted in “green” colour.

1.2 Handling subpictures with VVC specification JVET-P2001:

4

8

12

13

14

17

18

21

22

24

23

20

19

16

15

3

7

11

10

6

5

9

2

1

In VVC Spec Figure 7: There are 18 tiles, 24 subpictures and 24 slices (20x12=240 CTUs).

(Note: this Figure is borrowed from JVET_Q0041).

In one potential use case, in which we coded every picture using the partitions in above “Figure 7”, we only choose to display subpictures 5, 6, 9, 10 (a rectangular region) to start. At one random access point along the time line, we want to make a scene change (cut) to display subpictures 6, 7, 10, 11 instead of original 5, 6, 9, 10. If we follow the exact constraints in current VVC spec (JVET-P2001), at that random access point, we will have all the subpictures marked in above Figure from 1 to 24 to have the exact same IRAP NAL unit type (for instance, IDR_N_LP for all 24 subpictures, or CRA_NUT for all 24 subpictures).  If we want to have a mixed NAL unit types case, then we can not gurantee we only have IRAP subpictures/pictures.

# Proposed solution

It is proposed to add one more flag “mixed_irap_nalu_types_in_pic_flag” in PPS as the follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| ……………………………………… |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| if(mixed_nalu_types_in_pic_flag) |  |
| mixed_irap_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| …………………………………….. |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The unchanged semantics as:

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

The added new semantics as:

mixed_irap_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is with mixed nal_unit_types in the range of IDR_W_RADL to CRA_NUT, inclusive. mixed_irap_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS do not have the same value of nal_unit_type and that the picture is not an IRAP picture.

# Conclusion

With added “mixed_irap_nalu_types_in_pic_flag”, the VVC spec can provide an improved support to mixed random access NAL unit types indication.

For application standard, like DVB standard, the added flag will provide a more flexible indication mechanism for random access indication support.

# Reference

[1] Versatile Video Coding (Draft 7), JVET-P2001, download address:  http://phenix.int-evry.fr/jvet/

Select  “All meetings” and then select  “Geneva” (with number “16”).

Please download version 14 - date 2019-11-14 14:34:15; or JVET-P2001-vE.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).