| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0164 |
| --- | --- |

| Title: | AHG9: On indication of rectangular slice height in video subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ping Wu | Email: | pwuzte@gmail.com |
| Source: | ZTE Corporation |  |  |

_____________________________

# Abstract

In current VVC Draft 7 specification, a picture partitions can be done as multiple tiles, slices or subpictures. There is a problem for rectangular slices indication: when only with rigid and simple tile partitions for a complete video picture, it doesn’t allow flexibly indicating the rectangular slice height in CTU heights within a subpicture. In this contribution, a solution is proposed to allow slice height in CTUs to be indicated either inside a tile or inside a subpicture.

# Introduction and Problem Statement

## Handling subpictures in VVC Draft 7:

In current VVC Draft 7 specification, the structure of subpictures is indicated in Sequence Parameter Set (SPS) as:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | u(4) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| ref_pic_resampling_enabled_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| ……………………………… |  |
| sps_extension_flag | u(1) |
| if( sps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The highlighted texts in “blue” colour in above SPS marked the way that subpictures was defined: in general, all the partitions are based on CTU (Coding Tree Unit -- basic unit), by specifying top left X, Y locations plus subpictures width and height information, subpictures can be defined efficiently.

Figure 1: One example of a video picture partition (10 subpictures in total).

With current subpicture design, the partition of subpictures in Figure 1 (each coloured block is associated with a subpicture) can be supported. The processing order of all subpictures in Figure 1 can be arranged differently (more than one possible way). The key constraint is that when a subpicture is encoded, this subpicture’s entire left boundary and top boundary should be available. For instance: (1) the boundary pixels for the current subpicture are belonging to other subpictures that have already been processed (decoded), or (2) the relevant boundaries for the current subpicture are picture boundaries. The possible two processing orders for these ten subpictures are given in Figure 2 (a) and (b).

10

9

8

7

6

5

4

3

2

1

(a)

10

9

8

2

7

6

5

4

3

1

(b)

Figure 2: the possible subpictures encoding orders illustration.

## Handling tiles and slices in VVC Draft 7:

As illustrated in VVC spec JVET-P2001 (original Figure 6-4):

Figure 3 (original Figure 6-4 in spec) – A picture with 18 by 12 luma CTUs that is partitioned into 12 tiles and 3 raster-scan slices (informative)

Figure 4 (original Figure 6-5 in spec) – A picture with 18 by 12 luma CTUs that is partitioned into 24 tiles and 9 rectangular slices (informative)

Figure 5 (original Figure 6-6 in spec) – A picture that is partitioned into 4 tiles and 4 rectangular slices (informative)

4

8

12

13

14

17

18

21

22

24

23

20

19

16

15

3

7

11

10

6

5

9

2

1

Figure 6 (original Figure 6-7 in spec) There are 18 tiles, 24 subpictures and 24 slices (20x12=240 CTUs). (Note: this Figure is borrowed from JVET_Q0041).

In all these figures, it is clear that the tiles structure in VVC cannot flexibly support the video picture partition as showed in Figure 1.

The tiles structure was described in PPS (Picture Parameter Set) as the following (relevant texts highlighted in “blue” colour):

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| ………………………….. |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| ………………………………………………… |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The highlighted texts in “blue” colour showed that the tiles structure can be seen as a specific grid type.

If we really want to use tiles concepts in a partition arrangement and at the same time we would want to support Figure 1 illustrated subpictures structure arrangement. In following Figure 7, it show a possibility: the picture will need to be partitioned as 5x5= 25 tiles as displayed. In Figure 7, each coloured box is still a subpicture partitioned in this video picture. Note that according to VVC current specification, in Annex A “Profiles, tiers, levels”, the maximum number of tile columns and rows are quite limited; for instance, for level 4, it is 5x5; for level 6 it is 22x20.

Figure 7: The partition can be arranged as 5x5=25 tiles, but the tiles coding order will be restricted and the coding order through each CTU in scanning order is not entirely flexible.

# Proposed solution

Supporting multiple rectangular slices within subpictures in JVET-P2001:

According to current spec, the only way to support multiple slices is to use PPS part. In Figure 1’s example, we initially define 10 subpictures structure (top left X and Y location, plus subpicture width and height information). For simplification, let us assume that we define the whole picture as one single tile. Now let us examine a typical use case: for Figure 1 illustrated subpictures partition, we would like to have two slices inside each subpicture, therefore, in total we should have 20 slices (for 10 subpictures). Because subpictures structure is used, plus that the “sps_num_subpics_minus1” is known in SPS and the “num_slices_in_pic_minus1” is known in PPS (note that the subpictures processing order, and the slices processing order are also known with Idx leading to every single of them), when the heights (in ctu size measures) for all slices inside this video picture are specified in PPS and made available, then the partition map based on ctu grid with the marking of subpictures, tiles and slices are completely clear. For each slice, its starting ctb address is known, the width of the slice will follow the width of its associated subpicture, and its height (in ctu size measure) is indicated in the PPS bitstream. Also the number of slices inside each subpicture is also clear.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| ………………………….. |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| rect_slice_height_in_subpic_only_flag | u(1) |
| if (rect_slice_height_in_subpic_only_flag) { |  |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) |  |
| slice_height_in_ctu_in_subpic_minus1[ i++ ] | ue(v) |
| } else { |  |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| ………………………………………………… |  |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# The necessary changes in JVET-P2001 specification

- Under 6.5.1 “CTB raster scanning, tile scanning, and subpicture scanning processes”:

The existing texts may need some further clarifications.

The list CtbToSubPicIdx[ ctbAddrRs ] for ctbAddrRs ranging from 0 to PicSizeInCtbsY − 1, inclusive, specifying the conversion from a CTB address in picture raster scan to a subpicture index, is derived as follows:

for( ctbAddrRs = 0; ctbAddrRs < PicSizeInCtbsY; ctbAddrRs++ ) {
	posX = ctbAddrRs % PicWidthInCtbsY * CtbSizeY 
	posY = ctbAddrRs / PicWidthInCtbsY * CtbSizeY 
	CtbToSubPicIdx[ ctbAddrRs ] = −1
	for( i = 0; CtbToSubPicIdx[ ctbAddrRs ] < 0  &&  i  <=  sps_num_subpics_minus1; i++ ) {	(31)
		if( ( posX  >=  subpic_ctu_top_left_x[ i ] * CtbSizeY )  &&
				( posX < ( subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] + 1 ) * CtbSizeY )  &&
				( posY  >=  subpic_ctu_top_left_y[ i ] * CtbSizeY )  &&
				( posY < ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] + 1 ) * CtbSizeY ) )
			CtbToSubPicIdx[ ctbAddrRs ] = i
	}
}

The list NumSlicesInSubpic[ i ] and SliceSubpicToPicIdx[ i ][ k ], specifying the number of rectangular slices in the i-th subpicture and picture-level slice index of the k-th slice in the i-th subpicture, are derived is derived as follows:

for( j = 0; j  <=  sps_num_subpics_minus1; j++ )
	NumSlicesInSubpic[ j ] = 0
for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
	posX = CtbAddrInSlice[ i ][ 0 ] % PicWidthInCtbsY * CtbSizeY 
	posY = CtbAddrInSlice[ i ][ 0 ] / PicWidthInCtbsY * CtbSizeY 
	for( j = 0; j  <=  sps_num_subpics_minus1; j++ ) {
		if( ( posX  >=  subpic_ctu_top_left_x[ j ] * CtbSizeY )  &&	(32)
				( posX < ( subpic_ctu_top_left_x[ j ] + subpic_width_minus1[ j ] + 1 ) * CtbSizeY )  &&
				( posY  >=  subpic_ctu_top_left_y[ j ] * CtbSizeY )  &&
				( posY < ( subpic_ctu_top_left_y[ j ] + subpic_height_minus1[ j ] + 1 ) * CtbSizeY ) ) {
			SliceSubpicToPicIdx[ j ][ NumSlicesInSubpic[ j ] ] = i
			NumSlicesInSubpic[ j ]++
		}
	}
}

The “Green” colour highlighted texts are modified according to “JVET VVC Trac” information #742.

- Under 7.4.3.4 “Picture parameter set RBSP semantics”:

The proposed addition to VVC spec semantics texts are only the following and are marked in “red” colour:

rect_slice_height_in_subpic_only_flag equal to 1 specifies that slices height is specified with subpictures partition only. rect_slice_height_in_subpic_only_flag equal to 0 specifies that slices height is specified with tiles partition.

slice_height_in_ctu_in_subpic_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of CTU rows from a single subpicture. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1, inclusive.

# Conclusion

The proposed solution in this contribution provides an alternative way (to current Versatile Video Coding  - VVC Draft 7 -- JVET-P2001) to specify the rectangular slices information inside subpictures structure and the design is rather independent from specifying rectangular slices with tiles structure.

# Reference

[1] Versatile Video Coding (Draft 7), JVET-P2001, download address:  http://phenix.int-evry.fr/jvet/

Select  “All meetings” and then select  “Geneva” (number 16); the JVET-P2001 can be located.

# Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).